3GPP TSG-SA WG6 Meeting #67	S6-252482

Fukuoka, Japan 19th – 23th May 2025	(revision of S6-252240, S6-2244)

Source:	China Mobile

Title:	New solution on Transmission enhancement based on traffic characteristics

Spec:	3GPP TR 23.700-51 v0.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

Add new solution for Analytics and provisioning of XR traffic characteristics.

2. Reason for Change

This paper is going to address the KI#1 and KI#2.

This solution analyzes the traffic characteristics, and enhance the transmission based on those traffic characteristics.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v0.1.0.

* * * First Change * * * *

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol # X | X | X |  |  |

## 5.x	Solution #x: Transmission enhancement based on traffic characteristics

This solution provides a solution to enhance the transmission for XR traffic based on their traffic characteristics.

### 5.x.1	Architecture Impacts

None

### 5.x.2	Solution description

XR service traffic of different types may exhibit different traffic characteristics,

For Video traffic, taking 2K/60FPS, GOP:30, H.264 encoded DL video traffic as an example, the traffic characteristics including:

- Bitrate:  (2K=2560×1440(Pixels/Frame)) × 1.5 bytes (YUV420) × 60 FPS × 8 ÷ 100 ≈ 26.5 Mbps)

- Traffic Periodicity‌: the frame inter arrival time is 16.67 ms, then the Traffic Periodicity‌ for I frame is 0.5s.

- Packet Size: the size of I frame is generally 2-3 times larger than P frame.

For Audio traffic, characteristics including:

- Bitrate: can be calculated as the sampling rate × bit depth × number of channels;

For Haptic traffic, characteristics including:

- transmission interval: For some tactile sensor, it will have sampling rate (e.g., 1 kHz), so that the data output will be sent at fixed intervals.

In summary, the traffic characteristics can be calculated by SEALDD server are as follows:

For video, the Bitrate can be calculated based on the following equation:

    - Bitrate(kbps)= (Resolution×Raw Frame Size(Resolution×bpp)×FPS×Compression Ratio)/ GOP​×8×10−3

    - Frame inter arrival time= 1/FPS

For audio, the Bitrate can be calculated based on the following equation:

    - Bitrate(kbps)=sampling rate × bit depth × number of channels

Based on the characteristics, the SEALDD layer could enhance the transmission for that traffic based on the 5GC services and SEALDD layer services.

5.x.2.1	SEALDD enabled data transmission quality guarantee based on traffic characteristics

Figure 5.x.2.1-1: SEALDD enabled data transmission quality guarantee based on traffic characteristics

1.	VAL server sends request for SEALDD enabled data transmission quality guarantee based on traffic characteristics. The request may include the media configuration information (i.e., Video: Resolution, encoded method (e.g., Compression Ratio, Color Encoding Model (bits per pixel, bpp)), Frames Per Second (FPS), group of pictures (GOP); Audio: sampling rate, bit depth, number of channels; Haptic: sampling rate, etc.).

2a.	SEALDD server calculates the traffic characteristics based on received media configuration information.

2b.	If the received media information is not enough to calculate the traffic characteristics, the SEALDD server request the traffic characteristic from ADAES server.

Editor’s note: the traffic characteristic analysis service in ADAES server is FFS.

3a.	Based on the traffic characteristics from step2, SEALDD server could do the transmission enhancement by request the rate control based on the bitrate.

Editor’s note: the enhancement for rate control service in SEALDD is FFS.

3b.	SEALDD server request AF Session with required QoS as defined in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3] can be reused to provision the protocol description and the calculated Traffic Periodicity, bitrate.

Editor’s note: the enhancement for NRM service in SEAL is FFS.

### 5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>