3GPP TSG-SA WG6 Meeting #67	S6-252486

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252052)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0009 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for Download digital asset media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In Table 8.4.3.3-1: Download digital asset media request, there is Digital asset identifier. But in Table 8.4.3.4-1: Download digital asset media response, there is List of digital assets. It is confused. - In clause 8.2 Digital asset profile management & 8.3 Digital asset discovery, for the procedures and information flows, the operations just for one Digital asset. - So it is suggested that keep one Digital asset for Download digital asset media operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 8.4.2.2 Download digital asset media, delete the last sentence of step 2. - In clause 8.4.3.4 Download digital asset media request, no need parameter List of digital assets. S6-251376 has already deleted parameter Digital asset identifier and changed parameter Digital asset profile identifier to Digital asset identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with Digital asset profile management & Digital asset discovery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.4.2.2	Download digital asset media

Figure 8.4.2.2-1: Download digital asset media

1.	A requestor (e.g. DA client) sends a download digital asset media request to the DA server. The request includes information listed in Table 8.4.3.3-1.

2.	The DA server authenticates and authorizes the request and verifies the digital asset identifier. If authorized, the DA server determines the digital asset media as per the information provided in step 1.

3.	The DA server sends a download digital asset media response to the requestor. The response includes information listed in Table 8.4.3.4-1.

# * * * * Second change * * * *

8.4.3.3	Download digital asset media request

Table 8.4.3.3-1 describes the information elements for the download digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.3-1: Download digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | An identifier for the digital asset. |

8.4.3.4	Download digital asset media response

Table 8.4.3.4-1 describes the information elements for the download digital asset media response sent from a DA server to a requestor (e.g. DA client).

Table 8.4.3.4-1: Download digital asset media response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure cause is provided. |
| Digital asset identifier | O | If digital asset is a collection, then this identifies the digital asset collection. |
| Digital asset profile identifier | M | An identifier for the digital asset. |
| List of digital asset media | M | List of digital asset media to be downloaded |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

# * * * * End of changes * * * *