3GPP TSG-SA WG6 Meeting #67	S6-252499

Fukuoka, Japan, 19th – 23rd May 2025	(Revision of S6-252143, 252415)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0045 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting localization of users/UEs by multiple spatial maps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the new WID on application enablement for mobile metaverse services-Phase 2 (S6-250534), there is the following objective: 2. Enhancements to Spatial map service: a. Enhancement to exposure of object(s) tracking like including velocity b. To support relation between spatial map layers to link layers of spatial maps and linking of multiple spatial maps for better localization of users/UEs . For bullet b, it is assumed that spatial maps could be linked by spatial map coverage naturely. VAL server or SEAL SM client can obtain some spatial maps after triggering Discover spatial map request if the area of interest is covered by multiple spatial maps. Then, when VAL server or SEAL SM client requsts spatial localizaiton service, it can provide those spatial maps information to localize users/UEs. However, the current procedure does not support multiple spacitial maps in spatial localization service request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add an IE “Spatial Map list” to indicate multiple spatial maps used for localization, in Spatial localization service request message - Accordingly modify the related procedure description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Localizaiton of users/UEs by multiple spatial maps is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1, 9.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.4.1	General

This spatial localization services enables consumer to request SEAL SM Server for localization of a VAL user or VAL UE.

### 9.4.2	Procedure

Figure 9.4.2-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to invoke spatial localization services. The service is provided by SEAL SM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 9.4.2-1: Spatial localization services

1)	The VAL server (or SEAL SM client) sends request to perform localization service in the spatial map(s) to the SEAL SM server. The request includes identity of the map(s) on which the action is to be performed. The request may include list of users (or UEs) to localize.

2)	The SEAL SM server authorizes the requestor (e.g. VAL server or SEAL SM client). If the requestor is not authorized then the SEAL SM server sends failure response. If the request is authorized: the SEAL SM server, identifies all users/UEs along with their three dimensional location in the spatial map. If area of interest IE is present then the SEAL SM server identifies the target users/UEs within the area of interest only, otherwise, the SEAL SM server identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL SM server sends response to the VAL server (or SEAL SM client). The response includes the object localization and identity.

NOTE:	In this release, the localization of UE is considered. Localization of all objects will be considered in future release.

* * * Next Change * * * *

### 9.4.3	Information flows

#### 9.4.3.1	Spatial localization service request

Table 9.4.3.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.4.3.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL User or VAL UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map list |  | List of spatial maps to be used for spatial localization |
| > Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target identities | O | List of identity of the users or objects to localize |