3GPP TSG-SA WG6 Meeting#67	S6-252544

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252364 )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0307 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revocation of authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | API invoker authorization revocation may occur e.g. because of change of business agreements, RO authorization revocation, etc. E.g., RO authorization revocation allows a RO to revoke their authorization for an API invoker to access their resources. When the authorization is revoked, the CCF Authorization function should avoid attempts to use active access tokens issued before authorization revocation and still not expired. The best way to mitigate the use of invalid active access tokens is to issue access tokens with relatively short expiration times. However, even with short expiration times, it might occur that the authorization revocation takes place while a preivously provided access token is still active and in use. The CCF triggered Authorization revocation mechanism should ensure that when the authorization is revoked the application cannot use previously issued access token(s) which are still active according to their expiration time. In addition, the CCF triggered authorization revocation mechanism should prevent exposed APIs from accepting or receiving access tokens whose claims have been revoked. Note that authorization revocation is a broader measure than access token revocation, since it also impacts the ability to obtain new access tokens. The current authorization revocation procedure works for access token revocation but lacks granularity information: when an API invoker receives an authorization revocation notification for accessing a service API, the API invoker should stop using the service API, and all the active access tokens should become invalid. With the support in Rel19 of finer authorization granularity (based e.g. in resource and operation) and RO authorization revocation, the currently specified authorization revocation mechanism becomes deficient, since many access tokens, though being valid, will become invalidated, with the corresponding increase of signaling and potential impact in user experience |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to include the requirements to support Authorization revocation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no requirements that allow that an API invoker can e.g. identify which access tokens have become invalid because of authorization revocation. The exposed API function cannot identify which access tokens have become invalid because of authorization revocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2, 6.3.4, 6.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.3.2	Requirements

[AR-4.3.2-a] The CAPIF shall provide mechanisms to hide the topology of the PLMN trust domain from the API invokers accessing the service APIs from outside the PLMN trust domain.

[AR-4.3.2-b] The CAPIF shall provide mechanisms to authenticate API invokers prior to accessing the service APIs.

[AR-4.3.2-c] The CAPIF shall provide mechanisms to authenticate API invokers upon the service API invocation.

[AR-4.3.2-d] The CAPIF shall provide mechanisms to authorize API invokers to access the service APIs.

[AR-4.3.2-e] The CAPIF shall provide mechanisms to validate authorization of the API invokers upon the service API invocation.

[AR-4.3.2-f] The CAPIF shall provide mechanisms for mutual authentication between the CAPIF and the API invoker.

[AR-4.3.2-g] The CAPIF shall provide mechanisms to control the service API access for every API invocation.

[AR-4.3.2-h] The communication between the CAPIF and the API invoker shall be confidentiality protected.

[AR-4.3.2-i] The communication between the CAPIF and the API invoker shall be integrity protected.

[AR-4.3.2-j] The CAPIF shall provide mechanisms to authenticate the service API publishers to publish and manage the service API information.

[AR-4.3.2-k] The CAPIF shall provide mechanisms to authorize the service API publishers to publish and manage service API information.

[AR-4.3.2-l] The CAPIF shall provide mechanisms to validate authorization of the service API publishers to publish and manage service API information.

[AR-4.3.2-m] The CAPIF shall provide mechanisms to enable that upon authorization revocation, the API invoker and the API Exposing Function are notified about the revocation of previously granted authorization.

* * * Next Change * * * *

### 6.3.4	API exposing function

The API exposing function is the provider of the service APIs and is also the service communication entry point of the service API to the API invokers. The API exposing function consists of the following capabilities:

-	Authenticating the API invoker based on the identity and other information required for authentication of the API invoker provided by the CAPIF core function;

-	Validating the authorization provided by the CAPIF core function;

-	Supporting authorization revocation;

-	Logging the service API invocations at the CAPIF core function; and

-	Hiding the topology of the PLMN trust domain from API invokers, depending on configured policy.

* * * Next Change * * * *

### 6.4.6	Reference point CAPIF-3 (between the API exposing function and the CAPIF core function)

The CAPIF-3 reference point, which exists between the API exposing function and the CAPIF core function, is used for exercising access and policy related control for service API communications initiated by the API invoker.

The CAPIF-3 reference point supports:

-	Authenticating the API invoker based on the identity and credentials of the API invoker;

-	Providing authorization for the API invoker prior to accessing the service API;

-	Providing authorization revocation information;

-	Authorization verification for the API invoker upon accessing the service API;

-	Authorization verification for the API invoker based on RNAA;

-	Controlling the service API access based on PLMN operator configured policies;

-	Logging the service API invocations; and

-	Charging the service API invocations.

NOTE:	The security aspects of CAPIF-3 are specified in subclause 6.6 of 3GPP TS 33.122 [12].

* * * END of Change * * * *