3GPP TSG-SA WG6 Meeting #68	S6-253013

Gothenburg, Sweden 25th – 29th August 2025

Source:	Motorola Solutions

Title:	Supporting Non session-based Recording support for MCPTT , MCVIDEO messages.

Spec:	3GPP TR 23.700-39 V0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Adinarayana Setty (Adinarayana.setty@motorolasolutions.com)

Harish N (harish.negalaguli@motorolasolutions.com)

1. Introduction

This contribution proposes Secnarios, Key issues and solution for supporting Non session based recording support for MCPTT, MCVIDEO messages.

2. Reason for Change

To define procedures for supporting recoding of Non session based MCPTT and MCVIDEO messages.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.2.0.

* * * First Change * * * *

# 2	References

[10]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control.”

[11]	TS 24.281: “Mission Critical Video (MCVideo) signalling control.”

* * * Next Change * * * *

### 4.1.x Recording support for Non session based MCPTT messages.

Figure 4.1.x-1: Functional entities for supporting recording for Non-Session based MCPTT messages.

This scenario describes the recording support for Non session based MCPTT Service messages between MC user A, MC user B.

MC user A, MC user B (MCPTT ID A, MCPTT ID B) has/have been set as the target for the recording by a Recording admin user. The procedure described in 3GPP TS 23.280 [2], clause 10.18.2.2 will be used to update target user’s MCPTT profile in the CMS to support recording.

NOTE:	Parameter required parameters for supporting recording in MCPTT user profile is FFS.

Target MC users can be part of Non-session-based procedures for MCPTT Services. Following are the procedures where MCPTT user could be part of Non session based MCPTT Services.

Procedure to support User authentication and authorization for MCPTT services, Affiliation and de-affiliation to or from MCPTT group(s), MCPTT group selection, Location information (on-network), Subscription and notification for functional alias are described in 3GPP TS 23.379 [3] clause 10.2, 3GPP TS 23.379 [3] clause 10.3, 3GPP TS 23.379 [3] clause 10.4, 3GPP TS 23.379 [3] clause 10.13, 3GPP TS 23.379 [3] clause 10.18 respectively.

### 4.1.y Recording support for Non session based MCVIDEO messages.

Figure 4.1.y-1: Functional entities for supporting recording for Non-Session MCVIDEO messages.

This scenario describes the recording support for Non session based MCVIDEO Service messages between MC user A, MC user B.

MC user A, MC user B (MCVIDEO ID A, MCVIDEO ID B) has/have been set as the target for recording by a Recording admin user. The procedure described in 3GPP TS 23.280 [2], clause 10.18.2.2 will be used to update target user’s MCVIDEO profile in the CMS to support recording.

NOTE:	Parameters required for supporting recording in MCVIDEO user profile is FFS.

Target MC users can be part of Non-session-based procedures for MCVIDEO Services. Following are the procedures where MCVIDEO user could be part of Non session based MCVIDEO Services.

Procedure to support User authentication and authorization for MCVIDEO services, Affiliation and de-affiliation to/from MCVideo group(s), Functional alias management, User auhentication and authorization for MCVideo service, Location information (on-network) are described in 3GPP TS 23.281[4] clause 7.9, 3GPP TS 23.281[4] clause 7.9a, 3GPP TS 23.281[4] clause 7.12, 3GPP TS 23.281[4] clause 7.14.

* * * Next Change * * * *

## 5.x	Key Issue x – Recording support for Non session based MCPTT.

Non session based MCPTT covers the recording support for Non-session-based procedures or messages required to support private, adhoc, group communications for MCPTT service.

As described in the scenario 4.1.x, the required MCPTT procedures, information flows already exist in the Rel-19 MCX specifications. To fully support Non-session based MCPTT recording procedures, existing reference point i.e. REC-4 defined in 3GPP TS 23.280 [2], clause 7.5.2.34 needs to be extended to support Non session based MCPTT recording. Procedures need to be defined in 3GPP TS 23.280 [2] to support Non session based MCPTT recording.

## 5.y	Key Issue y – Recording support for Non session based MCVIDEO.

Non session based MCVIDEO covers the recording support for Non-session-based procedures or messages required to support private, adhoc, group communications for MCVIDEO service.

As described in the scenario 4.1.y, the required MCVIDEO procedures, information flows already exist in the Rel-19 MCX specifications. To fully support Non-session based MCVIDEO recording procedures, existing reference point i.e. REC-4 defined in 3GPP TS 23.280 [2], clause 7.5.2.34. needs to be extended to support Non session based MCVIDEO recording. Procedures need to be defined in 3GPP TS 23.280 [2] to support Non session based MCVIDEO recording.

* * * Next Change * * * *

## 6.x	Solution x: Non session based MCPTT recording support

### 6.x.1	Description

The 3GPP standard technical specifications TS 23.280 [2], TS 23.379 [3], TS 23.281[4], and TS 23.282[5] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCDATA), Mission Critical Video (MCVIDEO), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure 6.x.1.3 lists non session based recording procedures for MCPTT Service.

#### 6.x.1.1	Functional model and reference points

As described in Key issue 5.x and scenario 4.1.x, existing REC-4 interface as defined in 3GPP TS 23.280 [2], clause 7.5.2.34 shall be extended to support Non session based MCPTT recording.

#### 6.x.1.2	Configurations

None.

#### 6.x.1.3	Procedures

The 3GPP specifications TS 23.379 [3] and TS 24.379 [10] outline the architecture, flows, and procedures for supporting MCPTT communication.

TS 23.379 [3] clause 10.6, TS 23.379 [3] clause 10.7, TS 23.379 [3] clause 10.14, TS 23.379 [3] clause 10.19 outlines procedures for supporting MCPTT Group call, MCPTT Private call, Ambient listening call, MCPTT Ad hoc group call respectively.

The following procedure outlined in diagram 6.x.1.3-1 proposes non session based MCPTT recording flow.

Pre-conditions:

1.	MCPTT users involved in communication are already registered for receiving MCPTT service.

2.	All MCPTT clients involved in the communication are in the same MCPTT system.

3.	All MCPTT Groups are hosted in the same MCPTT system.

Figure 6.x.1.3-1: MCPTT recording flow.

1.	 Non session based MCPTT service procedures are invoked between set of MCPTT users.

2.	Upon the completion of Non session based MCPTT service procedures between group of MCPTT users, MCPTT server will initiate the transfer of MCPTT Non session based messaging information to the recording server. This transfer SHALL include Non session based MCPTT Messaging content, Metadata etc.

NOTE: Procedure used for transferring recording content from MCPTT server to Recording server will be FFS.

3.	On the receipt of Non session based MCPTT recording information, recording server shall push this information to Recording mass storage server (for later retrieval).

4.	As per the procedures defined in TS 23.280 [2] clause 10.18.4.2, MC Replay UE shall fetch and replay MCDATA recordings from recording server.

### 6.x.2	Impacts on existing functional entities and reference points

None

### 6.x.3	Solution evaluation

TBD.

## 6.y	Solution y: Non session based MCVIDEO recording support

### 6.y.1	Description

The 3GPP standard technical specifications TS 23.280 [2], TS 23.379 [3], TS 23.281[4], and TS 23.282[5] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCDATA), Mission Critical Video (MCVIDEO), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure 6.y.1.3 lists non session based recording procedures for MCVIDEO Service.

#### 6.y.1.1	Functional model and reference points

As described in Key issue 5.y and scenario 4.1.y, existing REC-4 interface as defined in 3GPP TS 23.280 [2], clause 7.5.2.34 shall be extended to support Non session based MCVIDEO recording.

#### 6.y.1.2	Configurations

None.

#### 6.y.1.3	Procedures

The 3GPP specifications TS 23.281 [4] and TS 24.281 [11] outline the architecture, flows, and procedures for supporting MCVIDEO communication.

TS 23.281 [4] clause 7.1, TS 23.281 [4] clause 7.2, TS 23.281 [4] clause 7.3, TS 23.281 [4] clause 7.4, TS 23.281 [4] clause 7.19 outlines procedures for supporting MCVIDEO Group call, MCVIDEO Private call, MCVIDEO Pull, MCVIDEO Push, MCPTT Ad hoc group call respectively.

The following procedure outlined in diagram 6.y.1.3-1 proposes non session based MCVIDEO recording flow.

Pre-conditions:

1.	MCVIDEO users involved in communication are already registered for receiving MCVIDEO service.

2.	All MCVIDEO clients involved in the communication are in the same MCVIDEO system.

3.	All MCVIDEO Groups are hosted in the same MCVIDEO system.

Figure 6.x.1.3-1: MCVIDEO recording flow.

1.	 Non session based MCVIDEO service procedures are invoked between set of MCVIDEO users.

2.	Upon the completion of Non session based MCVIDEO service procedures between group of MCVIDEO users, MCVIDEO server will initiate the transfer of MCVIDEO Non session based messaging information to the recording server. This transfer SHALL include Non session based MCVIDEO Messaging content, Metadata etc.

NOTE: Procedure used for transferring recording content from MCVIDEO server to Recording server will be FFS.

3.	On the receipt of Non session based MCVIDEO recording information, recording server shall push this information to Recording mass storage server (for later retrieval).

4.	As per the procedures defined in TS 23.280 [2] clause 10.18.4.2, MC Replay UE shall fetch and replay MCVIDEO recordings from recording server.

### 6.x.2	Impacts on existing functional entities and reference points

None

### 6.x.3	Solution evaluation

TBD.

* * * End of Changes * * * *