3GPP TSG-SA WG6 Meeting #68	S6-253021

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on Scenario for Discreet monitoring of MCPTT and MCVideo private calls

Spec:	3GPP TR 23700-37-020

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

A scenario detailing MCPTT and MCVideo private calls is added to clarify the usecase and identify techical impact. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

Scenario for MCPTT and MCVideo private calls is missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.2.0.

* * * First Change * * * *

## 4.2	Scenario 1:	Discreet monitoring of MCPTT and MCVideo private calls

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user A and MC user B are served by the same MC system, MC system A and MC service user B is within the authority of authorized MC user A. The scenario is illustrated in figure 4.2-1 below.

Figure 4.2-1: Discreet monitoring of MCPTT or MCVideo private call in single MC system

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC user B places or receives an MCPTT or MCVideo private call to or from MC user C.

-	MC system A provides MC service client A with information concerning the initiation of the call, including the identity of MC user C.

-	MC system A provides the media exchanged between the MC user B and MC user C in the private call.

-	When the private call completes, the call completion is conveyed to MC service client A.

NOTE:	The scenario also applies to emergency private calls.

* * * End of Change * * * *