3GPP TSG-SA WG6 Meeting #68	S6-253029

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on Scenario for Discreet monitoring of one to server MCVideo push

Spec:	3GPP TR 23700-37-020

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

A scenario detailing one to server MCVideo push is added to clarify the usecase and identify techical impact. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

Scenario for one to server MCVideo push is missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.2.0.

* * * First Change * * * *

## 4.10	Scenario 9:	Discreet monitoring of one to server MCVideo push

This scenario describes the case where authorized MC user A requests discreet monitoring for MC user B where MC user B is within the authority of authorized MC user A, and where authorized MC user A receives MCVideo pushes to the MCVideo server that have been initiated by MC user B. In figure 4.10-1 below, MC user A and MC user B have the same primary MC system, MC system A.

Figure 4.10-1: Discreet monitoring of one to server MCVideo push

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC user B initiates an MCVideo push to the MCVideo server.

-	MC system A provides MC service client A with information concerning the initiation of the MCVideo push, and provides the MCVideo media sent from MC user B to the MCVideo server.

* * * End of Changes * * * *