3GPP TSG-SA WG6 Meeting #68	S6-253072

Gothenburg, Sweden 25th – 29th August 2025

Source:	China Telecom

Title:	Key Issue on enhancing satellite access based SEALDD service

Spec:	3GPP TR 23.700-02 V0.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	Zhou Zhe (zhouz11@chinatelecom.cn)

1. Introduction

In satellite access scenarios, to optimize content delivery, it is possible to consider deploying the corresponding data transmission components on satellites, just as the UPF and base stations are deployed on satellites. When satellite-assisted data transmission is used, users in remote areas who wish to transmit data to the cloud/edge network/central server can use satellites as relays for data transmission.

2. Reason for Change

Enhancements in satellite access based SEALDD service, to improve satellite assisted data transmission.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-02 V0.0.0.

* * * First Change * * * *

4.X	Key Issue #X: Enhancing satellite access based SEALDD service

4.x.1	Description

SA1 defines the following requirements in TS 22.261 for efficient content delivery: A 5G system with satellite access shall be able to optimise the delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service, as well as broadcasting/multicasting on very large to global coverages.

In satellite access scenarios, to optimize content delivery, it is possible to consider deploying the corresponding data transmission components on satellites, just as the UPF and base stations are deployed on satellites. When satellite-assisted data transmission is used, users in remote areas who wish to transmit data to the cloud/edge network/central server can use satellites as relays for data transmission.

It is worth noting that in the SEALDD architecture, there is direct signaling interaction between the user and the VAL server, and the specific data transmission between the two can be regarded as being proxied by the SEALDD server and client to establish a dedicated transmission channel to improve transmission efficiency. Therefore, to meet the requirements of SA1, the existing SA6 data transmission enabling layer needs to be enhanced, such as deploying the SEALDD server/client on satellites, leveraging the connection between the satellite and the terrestrial network for data transmission.

4.x.2	Open Issues

This key issue will study:

1.	Whether and how to deploy SEALDD elements on satellite.

2.	How to discovery and select satellite based SEALDD server/client.

* * * End of Changes * * * *