3GPP TSG-SA WG6 Meeting #68	S6-253137

Gothenburg, Sweden 25th – 29th August 2025

Source:	China Mobile

Title:	New KI on Enhance AIoT service operation, optimization and information exposure.

Spec:	3GPP TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Tianji Jiang, tianjijiang@chinamobile.com

1. Introduction

Add new KI upon providing value-added services to, optimizing the processing of service requests for, and exposing the information of AIoT devices to the consumer.

2. Reason for Change

SA2 has defined two AIoT services in TS 23.369, i.e., inventory and command servcies. While the fundamental architecture and framework for the support of AIoT services have been studied in SA2, there exist some unique scenarios for which the SA6 application enablement layer can help enhance the service support.

Suppose there are multiple 3rd-party App servers (VAL servers), each of which may issue an AIoT service request (via its respective AF’s which might be co-located) that covers only a sub-area of a large targeted area. Depending on the local configuration or dynamic settings at an AIoT application (or AIoTApp) enabler and also depending on the AIoT filtering information (TS 23.369, clause 5.8) associated with a service request, the AIoTApp enabler may aggregate these multiple AIoT requests into an integrated service request by unmasking some settings of the filter information so as to generate a more coarse filtering set and sent to the network. On the way back (Uplink), the same AIoTApp enabler can process the AIoT device information received from the network by applying the original more granular filter information. The filtered AIoT device info is distributed accordingly to the suitable (App-AS) requestor (i.e., VAL server). This aggregation operation in application enablement layer helps improve the AIoT service efficiency.

In another scenario, one AF (with the authorized AF-ID) sends continuously a batch of 'inventory service’ requests covering different areas (with different area-IDs), though these areas belonging to a same (large) targeted area whose identity could be a geographical civic address. Then, an AIoTApp enabler can aggregate these requests and send only one combined 'inventory service’ request to the AIOTF in the 5G network (possibly via NEF). The waiting time window for the service request aggregation can be configured locally at the AIoTApp enabler or determined dynamically based on historical request patterns.

The SA2 document TS 23.369 clause 4.5.3 describes that an AIOTF can provide an interface to a trusted AF for AIoT services.  That is, an AIoTApp enabler, if being a trusted-AF, can directly communicate with the AIOTF in network bypassing the NEF. In this case, some AIoT related functions that are assumed by NEF (in TS 23.369) might be delegated to an AIoTApp enabler. With this offloading operation, the SA6 AIoTApp enabler can utilize directly the exposure capabilities and/or AIoT device information from the SA2 WG. For example, the SA2-referenced AIoT RAN reader ID is a network internal info that cannot be exposed to the 3rd-party AF (TS 23.369 clause 5.3.3). However, the SA6 AIoTApp enabler may directly engage with the AIOTF and request the AIOTF to get the mapping from an internal RAN reader ID to the corresponding public information (e.g., civic address).

This paper proposes to study the providing value-added services to, optimizing the processing of service requests for, and exposing the information of AIoT devices to the consumer:

-	Study how to enrich the processing (e.g., filtering, aggregation) of AIoT service requests and the collected Ambient IoT device information for multiple App servers.

-	Study how to optimize the processing of service requests for the same App server.

-	Study how to utilize the exposure capabilities from other working group (e.g., SA2) and provide the value-added AIoT device information to the consumer (e.g., VAL server).

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2”.

* * * Next Change (all new texts) * * * *

## 4.x	Key issue #x: Enhanced AIoT service operation, optimization and information exposure to customers

SA2 has defined two AIoT services in 3GPP TS 23.369[x1], i.e., inventory and command servcies. While the fundamental architecture and framework for the support of AIoT services have been studied in SA2, there exist some unique scenarios for which the SA6 application enablement layer can help enhance the service support.

Suppose there are multiple 3rd-party App servers (VAL servers), each of which may issue an AIoT service request (via its respective AF’s which might be co-located) that covers only a sub-area of a large targeted area. Depending on the local configuration or dynamic settings at an AIoT application (or AIoTApp) enabler and also depending on the AIoT filtering information (3GPP TS 23.369[x1], clause 5.8) associated with a service request, the AIoTApp enabler may aggregate these multiple AIoT requests into an integrated service request by unmasking some settings of the filter information so as to generate a more coarse filtering set and sent to the network. On the way back (Uplink), the same AIoTApp enabler can process the AIoT device information received from the network by applying the original more granular filter information. The filtered AIoT device info is distributed accordingly to the suitable (App-AS) requestor (i.e., VAL server). This aggregation operation in application enablement layer helps improve the AIoT service efficiency.

In another scenario, one AF (with the authorized AF-ID) sends continuously a batch of 'inventory service’ requests covering different areas (with different area-IDs), though these areas belonging to a same (large) targeted area whose identity could be a geographical civic address. Then, an AIoTApp enabler can aggregate these requests and send only one combined 'inventory service’ request to the AIOTF in the 5G network (possibly via NEF). The waiting time window for the service request aggregation can be configured locally at the AIoTApp enabler or determined dynamically based on historical request patterns.

The 3GPP Spec. TS 23.369[x1] clause 4.5.3 describes that an AIOTF can provide an interface to a trusted AF for AIoT services.  That is, an AIoTApp enabler, if being a trusted-AF, can directly communicate with the AIOTF in network bypassing the NEF. In this case, some AIoT related functions that are assumed by NEF might be delegated to an AIoTApp enabler. With this offloading operation, the SA6 AIoTApp enabler can utilize directly the exposure capabilities and/or AIoT device information from the SA2 WG. For example, the SA2-referenced AIoT RAN reader ID is a network internal info that cannot be exposed to the 3rd-party AF (see TS 23.369 clause 5.3.3). However, the SA6 AIoTApp enabler may directly engage with the AIOTF and request the AIOTF to get the mapping from an internal RAN reader ID to the corresponding public information (e.g., civic address).

This paper proposes to study how to provide value-added services to, optimize the processing of service requests for, and expose the information of AIoT devices to, consumers via the application enablement layer. It is proposed to study the open issues:

-	Study how to enrich the processing (e.g., filtering, aggregation, etc.) of AIoT service requests and the collected Ambient IoT device information for multiple App servers.

-	Study how to optimize the processing of service requests for the same App server.

-	Study how to utilize the exposure capabilities from other working group (e.g., SA2) and provide the value-added AIoT device information to the consumer (e.g., VAL server).

* * * End Change * * * *