3GPP TSG-SA6 Meeting #68	S6-253156

Stor-Göteborg, Sweden, 25th Aug 2025 – 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to AIMLE framework to introduce UE performance

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

The ML model performance monitor as described in clause 6.21 of TS 23.482 covers a capability of AIMLE server for monitoring detecting a degradation relation to an ML operation / analytics operation and translating to an ML model degradation (expected or predicted) and performing an action to alleviate this issue (new model training or re-training). However, this degradation in performance happens mainly due to client’s output. In case of synchronous training, the accuracy of client can hamper model’s accuracy. While, in case of asynchronous training, accuracy and delay in producing output of a training by AIMLE client can hamper ML model’s accuracy.

In scenarios where a rogue or low performing AIMLE client/UE participate in ML task, it may always degrade a model performance. Hence it becomes utter important to pinpoint such AIMLE clients and accordingly allow them to participate in a task.

Here in this paper, we have proposed mechanisms to check AIMLE client’s performance and accordingly choose them for participation or terminate their participation in on going ML training task. The performance metrics of clients need to be saved in MLR using a new service. These metrics can be used for adding or removing a client to client set.

2. Reason for Change

To enhance ML model performance by performance evaluation of participating clients and accordingly selecting or removing a client from AIML training of HFL training. The clause 8.22 includes sending configuration requests to clients who have degraded ML model performance. Hence, it admits that low performing client can cause retraining of a ML model. So, it becomes important to define a framework that can monitor UE/AIMLE client performance and prevent low performing AIMLE clients to participate or degrade ML model performance

3. Conclusions

This paper proposes solution to enhance ML model performance by proposing a client performance framework which can affect client selection/removal from AIML task, HFL training and data management operations.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.XX	Solution on AIMLE Client Performance monitoring

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

The clause 8.22 of TS 23.482 discusses ML model performance framework. It also includes sending configuration requests to clients that have degraded ML model performance. Such clients can be tracked for future selections in AIML tasks. The low performing clients can be marked as rogue. Client performance monitoring becomes an important aspect to achieve high accuracy model without performing retraining.

The client performance can be detected on multiple parameters like accuracy of model after updating parameters returned by client or how much time client is taking in generating training output or how much variation is there in output of client in comparison to other participating clients.

As described in clause 8.13 of 3GPP TS 23.482, the VAL server requests an AIMLE server to assist in client selection by making AIMLE client selection subscription request. This clause shall be modified to support client performance-based selection. The ML repository shall support API to retrieve client profiles as defined in clause 2 of 8.7.2.2, clause 2 of 8.8.2.1 and clause 2 of 8.9.2.1 etc. The clause 8.13 also requires monitoring support from other 5GC nodes. Hence, if ML repository gets update on client performance due to an ongoing task the it should also support subscription to client profile and notifications.

### 7.XX.1.2 	Procedures

#### 7.XX.1.2.1	General

The following are introduced for AIMLE client performance monitoring:

-	MLR client selection; and

- 	MLR client performance subscription, update, notification and unsubscribe

-	MLR client performance data request and response

Other APIs included in the call flow are:

- AIMLE client selection subscription and AIMLE client selection update notification

- ML model training notification

#### 7.XX.1.2.2	AIMLE client selection subscription on basis of AIMLE client performance.

Figure 7.XX.1.2.2-1: Procedure for AIML client selection subscription based on performance

- The VAL server requests AIMLE client selection subscription as defined in section 8.12. the request also contains performance criteria for an AIMLE client.

- The AIMLE server sends request to ML repository to provide a list of clients as per requested client selection criteria containing performance requirements in “MLR client selection request”.

- The ML repository returns a list of clients satisfying client selection criteria in “MLR client selection response”.

- The AIMLE server sends a list of clients received from ML repository to VAL server in “AIMLE client selection subscription response”.

- The AIMLE server subscribes to ML repository using “MLR client performance monitoring subscription” to get updates on performance of clients. This request shall contain list of clients whose performances needs to be monitored.

- The ML repository returns a success response in “MLR client performance monitoring subscription response” for positive scenario.

- Whenever, there is a change in client performance metrics updated by an AIMLE server for ongoing task, ML repository initiates “MLR performance monitoring notification” containing performance metrics.

- The AIMLE server sends “AIMLE client selection subscription” notification to VAL server containing updated performance metrics.

Figure 7.XX.1.2.2-2: Procedure for AIML client performance update in ML repository

- The AIMLE server has scheduled an AIMLE task on AIMLE client and the client is sending training output for training iterations in ML model training notification as defined in point 4 of clause 8.3.2 and point 8 of clause 8.12.2.1.

- The AIMLE server, on getting the parameters in output of ML training by UE, performs operation like checking the deviations in client output as compared to other clients or performance of model after AIMLE client’s update or delay that AIMLE client makes in generating the output etc. and computes performance metrics for UE.

- The AIMLE server updates client’s performance metrics in ML repository using “MLR client performance update request”.

- The ML repository stores the client’s performance data and returns a success response along with updated performance metrics containing historical performance data.

### 7.XX.2	Information flows

#### 7.XX.2.1	MLR client selection request

Table 87.XX.2.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection procedure.

Table 7.XX.2.1-1: MLR client selection request

| Information element | Status | Description |
| --- | --- | --- |
| AIMLE server ID | M | The identifier of the requestor. |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| List of AIMLE client IDs | O (NOTE 1) (NOTE 3) | A list of AIMLE client IDs that was previously discovered for inclusion into an AIMLE client set. |
| AIMLE client selection criteria | O (NOTE 2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| AIMLE client performance criteria | O | AIMLE client performance requirements |
| Number of required AIMLE clients | O (NOTE 2) (NOTE 3) | Indicates the requested number of AIMLE clients to be selected based on selection criteria. |
| Minimum number of required AIMLE clients | O | Indicates the minimum acceptable number of AIMLE clients to be selected. |
| NOTE 1: This information element is only present for VAL server selection. NOTE 2: These information elements are only present for AIMLE server selection. NOTE 3: At least one of the information elements are present. |  |  |

#### 7.XX.2.2	MLR client selection response

Table 7.XX.2.2-1 shows the response sent by the ML repository to the AIMLE server for the AIMLE client selection procedure.

Table 7.XX.2.2-1: AIMLE client selection response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client set identifier | O (NOTE 1) | An identifier to associate with the set of AIMLE clients that the VAL server has provided in the request. The AIMLE client set can be updated by using this identifier. |
| List of AIMLE client IDs | O (NOTE 1) (NOTE2) | A list of AIMLE client IDs that matches the AIML selection criteria. |
| NOTE 1: This is present if the response status is success. NOTE 2: This information element is present for dynamic selection. |  |  |

#### 7.XX.2.2	MLR client performance monitoring subscription request

Table 7.XX.2.2-1 shows the response sent by the ML repository to the AIMLE server for the AIMLE client selection procedure.

Table 7.XX.2.2-1: AIMLE client performance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client set identifier | O (NOTE 1) | An identifier to associate with the set of AIMLE clients that the VAL server has provided in the request. The AIMLE client set can be updated by using this identifier. |
| List of AIMLE client IDs | O (NOTE 1) (NOTE2) | A list of AIMLE client IDs that matches the AIML selection criteria. |
| Expiration Criteria | O | Time or if client profile contains rogue indication |
| NOTE 1: This is present if the response status is success. NOTE 2: This information element is present for dynamic selection. |  |  |

#### 7.XX.2.2	MLR client performance monitoring subscription response

Table 7.XX.2.2-1 shows the response sent by the ML repository to the AIMLE server for the AIMLE client selection procedure.

Table 7.XX.2.2-1: AIMLE client performance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the subscription was successful. |
| > Subscription ID | M | The identifier of the subscription. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription status, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated subscription never expires. |
| Failure response | O (NOTE) | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |
| NOTE: One of the IEs shall be present. |  |  |

#### 7.XX.3.3	MLR client performance monitoring notification

Table 8.22.3.2-1 shows the notification sent by the AIMLE server to the VAL server for the ML model performance degradation.

Table 8.22.3.2-1: ML model performance monitoring notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| AIMLE Client ID | M | Identity of the ML model. |
| AIML client Rogue indication | M | Identifies the degradation of the ML model. |
| AIMLE client performance metrics | O | The performance metrics which are expected to be degraded (F1-score, recall, precision, accuracy). |
| Cause | O | Cause in case of subscription termination by MLR |

#### 7.XX.3.4	MLR client performance data request

Table 8.22.3.2-1 shows the notification sent by the AIMLE server to the VAL server for the ML model performance degradation.

Table 8.22.3.2-1: ML model performance monitoring notify

| Information element | Status | Description |
| --- | --- | --- |
| AIMLE client ID | M (NOTE) | Indicates the ID of AIMLE client |
| Performance Metrics | O (NOTE) | Updated performance metrics |
| Rogue Indication | O (NOTE) | Indication if a UE is rogue or not |
| Action | O (NOTE) | Indicate termination to subscribers or warn about low performance, request historical performance data |
| NOTE: At least one of these IEs shall be present. |  |  |

#### 7.XX.3.4	MLR client performance data response

Table 8.22.3.2-1 shows the notification sent by the AIMLE server to the VAL server for the ML model performance degradation.

Table 8.22.3.2-1: ML model performance monitoring notify

| Information element | Status | Description |
| --- | --- | --- |
| AIMLE client ID | M (NOTE) | Indicates the ID of AIMLE client |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| Performance Metrics | O (NOTE) | Updated historic performance data, performance score |
| NOTE: At least one of these IEs shall be present. |  |  |

* * * Second Change * * * *

#### 7.XX.1.2	Impact to existing AIMLE client selection subscription and notification

#### 8.13.2.2	AIMLE client selection subscription and notification

Figure 8.13.2.2-1: AIMLE client selection subscription and notification

1.	A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria.

2.	The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request.

3.	The AIMLE server sends the AIMLE client selection subscription response to the VAL server.

4.	The AIMLE server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring. The AIMLE server utilizes SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.11, or 3GPP TS 23.434 [5] clause 9.3.12) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to establish monitoring of UEs entering or present in the target location provided in the location information in the selection criteria. The AIMLE server monitors AIMLE clients for performance metrics with ML repository

5.	The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE server uses the location monitoring for selecting UEs that fulfil the location criteria and removing UEs which cease to fulfil the location criteria as provided in the location information in the AIMLE client selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

The AIMLE server may determine the application QoS parameters based on the VAL Service ID.

If a desired service in the selection criteria is ceased to be provided by the client or its profile change so that it no longer meets the selection criteria or performance criteria, the AIMLE server removes the AIMLE clients which ceases to fulfil the criteria and selects other clients that fulfil selection or performance criteria.

NOTE 1:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1.

NOTE 2:	In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is up to implementation.

6.	The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

### 8.13.3	Information flows

#### 8.13.3.1	General

The following information flows are specified for AIMLE client registration:

-	AIMLE client selection subscription and notification request and response;

-	AIMLE client selection subscription update request and response; and

-	AIMLE client selection unsubscribe request and response

#### 8.13.3.2	AIMLE client selection subscription request

Table 8.13.3.2-1 shows the request sent by a VAL server to an AIMLE server for the Selected AIMLE Client selection subscription.

Table 8.13.3.2-1: AIMLE client selection subscription requst

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client selection criteria | M | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| AIMLE client performance criteria | O | AIMLE client performance requirements |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

#### 8.13.3.5	AIMLE client subscription update request

Table 8.13.3.5-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection subscription update request.

Table 8.13.3.5-1: AIMLE client selection subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the existing subscription for which the update request applies. |
| AIMLE client selection criteria | O | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| AIMLE client performance criteria | O | AIMLE client performance requirements |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | O | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

# *** End of Changes ***