3GPP TSG-SA6 Meeting #68	S6-253157

Stor-Göteborg, Sweden, 25th Aug 2025 – 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to ML model information

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

The AIMLE server assigns an AIML task to UE which include model and operations. The client registers with its availability schedule. If a task is assigned to an AIML client, the client may update its registration with new availability schedule. In case of multiple task configuration on client, the client may serve the tasks in FIFO basis. This implementation on client is unregulated and these tasks can be prioritized on UE.

Clause 7.3a.1.2.4 of  TS 28.105 discusses the management aspects of ML Training process and it also includes prioritizing AIML tasks.

The task priority needs to be defined during participation request so that UE can schedule tasks as per their priority. The AIMLE server may need to maintain a list of ongoing tasks and update task priority when required. An AIMLE client may need to do a task transfer for emergency training as indicated by priority parameter of new task. In case of multiple AIMLE server creating AIML tasks on same UE/AIMLE client, a static profile of AIMLE client with scheduled AIML task can be created on ML repository.

2. Reason for Change

To enhance task management capability of AIMLE framework by introducing task priority feature

3. Conclusions

This paper proposes solution to configure task priority when multiple tasks are configured on an AIMLE client.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0 and implement these changes in TS 23.482.

* * * First Change * * * *

## 7.XX	Solution#XX: Introduction of Task Priority

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

The task scheduled on an AIMLE client can be executed as per their time configurations. In case of overlapping schedules or any emergency task requires training, the setting of task priority can help an AIMLE client to decide order of AIMLE task execution.

#### 7.XX.1.2	Impact to existing ML Model training request

### 8.3.1	General

This clause describes the procedure for supporting the ML model training by the AIMLE server. The VAL server requests the AIMLE server for the ML model training. VAL server requests the AIMLE server to train a ML model by specifying the type of learning, details about the ML model to be trained or requirements to select a ML model for training and criteria for selecting the members who can participate in the ML model training. The VAL server may additionally provide a priority to ML tasks to indicate which task needs to be executed on priority.

### 8.3.2	Procedure for ML model training

Figure 8.3.2-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server.

Figure 8.3.2-1: ML model training

1.	The VAL server sends an ML model training request to AIMLE server, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model training request. If no model information is provided but only the model requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for training based on the ML model requirement information using the procedure as specified in clause 8.11.3.

3.	If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure.

4.	The AIMLE server notifies the VAL server to update the list FL/ML clients selected or de-selected for the ML model training or to share the training output or any errors during the training process.

### 8.3.3	Information flows

#### 8.3.3.1	ML model training request

Table 8.3.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.3.3.1-1: ML model training request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE 1) | List of member clients to be utilized for training the ML model. |
| Member selection criteria | O (NOTE 1) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | O (NOTE 2) | Identifies the ML model that has to be distributed to the selected member clients for training. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML task priority | O | Indicates how early a ML task needs to be executed on AIMLE client among other scheduled tasks. |
| ML model requirement information | O (NOTE 2) | Identifies the requirement for selecting a model to be trained and this information contains the filtering criteria for selecting the model as specified in Table 8.11.4.1-2. |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new member clients selected or de-selected. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE 1: At least one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

* * * Second Change * * * *

#### 7.XX.1.3	Impact to existing AIML Client Participation request

### 8.10.1	General

The following clauses specify procedures, information flows, and APIs for AIMLE client participation.

### 8.10.2	Procedure

#### 8.10.2.1	AIMLE client participation

Figure 8.10.2.1-1: AIMLE client participation

1.	An AIMLE server sends an AIMLE client participation request to an AIMLE client to verify participation in AI/ML operation(s). The request includes information as described in Table 8.10.3.1-1.

2.	The AIMLE client acknowledge its willingness to be part of (or removed from) the AIMLE client set in an AIMLE client participation response sent to the AIMLE server. The response includes information as described in Table 8.10.3.1-2.

### 8.10.3	Information flows

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| ML model ID | M | The ML model ID to use for the AI/ML operation. |
| ML task priority | O | Indicates how early a ML task needs to be executed on AIMLE client among other scheduled tasks. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.10.3.2	AIMLE client participation response

Table 8.10.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.10.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Status | M | The status for the request: success or failure. |
| Cause | O | The cause for the request failure. |

* * * Third Change * * * *

#### 7.XX.1.5			Impact to existing HFL training

### 8.12.3	Information flows

#### 8.12.3.1	HFL training subscription request

Table 8.12.3.1-1 shows the request sent by the AIMLE server to AIMLE clients for the HFL training subscription procedure.

Table 8.12.3.1-1: HFL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AI/ML model and model parameters | M | Information about the AI/ML model and model parameters for use in HFL training. |
| Dataset identifier | M | The dataset identifier associated with the dataset used for HFL training. |
| ML task priority | O | Indicates how early a ML task needs to be executed on AIMLE client among other scheduled tasks. |
| Number of data samples | M | The number of data samples required for a round of HFL training. |
| Operational schedule | O | A schedule for when training is to occur. |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event- triggered (e.g. based on percentage completion), upon completion of each training round. |

# *** End of Changes ***