3GPP TSG-SA WG6 Meeting #68	S6-253163

Stor-Göteborg, Sweden, 25th Aug 2025 – 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to AIML model retrieval

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

The AIMLE server gives URL to VAL server and AIMLE client in order to fetch a ML model. The AIMLE client may require to download ML models as specified in section 8.2.2.2 of TS 23.482. The ML model may be huge in size. In case of non-critical AIML tasks, this download can be performed during non-peak hours. The AIMLE server can coordinate with network to derive non-peak hours and accordingly choose AIMLE clients as per their availability. Moreover, the VAL server may get a URL of a nearby repository holding ML model if location information is provided to AIMLE server.

2. Reason for Change

To optimize model downlad for non-critical AIMLE tasks during non peak hours and to provide a URL of model repository nearest to requestor i.e. VAL server or AIMLE client. An AIML model will be trained by thousands of UE in order to leverage power of federated learning. Downloading of huge ML model or data for processing will require  good chunk of network bandwidth and traffic on repository server. If AIMLE clients of same gNB are selected to train a model on RAN parameters, then downloading models in peak hours may cause congestion in network. Moreover, the proposal will also allow VAL server to fetch models from a nearby repository.

3. Conclusions

This paper proposes solution to optimize ML model download for non-critical ML operations during non-peak hours so as to avoid congestion in network

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0 and its inclusion in TS 23.482.

* * * First Change * * * *

## 7.XX	Enhancements to ML model retrieval using BDT procedure

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

The ML model retrieval can be optimized to be performed during non-peak hours during BDT retrieved time slots. This section describes the steps involved in making a client download ML model or data during non-peak hours using BDT procedure.

#### 7.XX.2.1	Changes to existing ML Model Retrieval

### 8.2.2	Procedure

#### 8.2.2.1	General

Following procedures are supported for ML model retrieval:

-	ML model retrieval request-response procedure;

-	ML model subscribe-notify procedures for retrieval of ML models, comprising:

-	Subscription procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure;

- ML Model retrieval with BDT

#### 8.2.2.XX	ML model retrieval with BDT

Figure 8.2.2.2-1: ML model retrieval with BDT

- The AIMLE client register for ML model retrieval request as per clause 8.2 with requestor identity. The requestor may optionally send location information. If the requestor is VAL server, the location information will help AIMLE server to give a nearby endpoint to VAL server to have minimum latency. In case, requestor information is of AIMLE client and client sends location information in request, AIMLE server shall use it for further steps.

- If client has not sent any location information, t, AIMLE server can fetch location of the client using NEF monitoring event “Location Reporting” as defined in clause 4.4.2 of TS 29.522 or it may use client’s PLMN, IP etc details for further processing.

- If the requestor is AIMLE client, the AIMLE server, based on its location, initiate BDT procedure as defined in section 4.4.4 of TS 29.522 with NEF to fetch time window when network is least congested.

- The AIMLE server sends ML Model Retrieval response to VAL server with URL of nearest ML repository along with timing information when download shall start. In case of AIMLE client, the AIMLE server shall send timing information as received from NEF using BDT procedure.

### 8.2.3	Information flows

#### 8.2.3.1	ML model retrieval request

Table 8.2.3.1-1 describes information elements for the ML model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.1-1: ML model retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Requestor Location Info | O | Location information of requestor |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

#### 8.2.3.2	ML model retrieval response

Table 8.2.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.2.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the ML model retrieval request was successful. |
| >List of ML models | M | List of retrieved ML models; the nested information elements are provided for each ML model in the list. |
| >> ML model identifier | M | The ML model identifier. |
| >> ML model retrieval endpoint (NOTE 1) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. (NOTE3) |
| >> ML model (NOTE 1) | O | The ML model containing a retrieved ML model. |
| Download time window | O | Optimized time for AIMLE client when network is least congested |
| Failure response | O (NOTE 2) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE 1: At least one of these information elements is needed. NOTE 2: One of the IEs shall be present. NOTE 3: This URL corresponds to an endpoint nearby to requestor location. Can be retrieved using SDN information of MEC configurations |  |  |

#### 8.2.3.3	ML model retrieval subscribe request

Table 8.2.3.3-1 describes information elements for the ML model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.3-1: ML model retrieval subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Requestor Location Info | O | Location information of requestor |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

#### 8.2.3.5	ML model retrieval notification

Table 8.2.3.5-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.5-1: ML model retrieval notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| List of ML models | O | List of retrieved ML models; the nested information elements are provided for each ML model in the list. |
| > ML model identifier | O | The ML model identifier. |
| Download time window | O | Optimized time for AIMLE client when network is least congested |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| NOTE: At least one of these elements is needed. The URL is of a nearest server as per location info provided in request |  |  |

# *** End of Changes ***