3GPP TSG-SA6 Meeting #68	S6-253177

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0050 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining client actions on its removal from an AIMLE task. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.482, mechanisms have been specified for registeration and de-registeration of AIMLE clients and FL members. The specification also mentions about client participatation request between AIMLE server and AIMLE client. The client participation request contains option add/remove AIMLE client from participating AIMLE client set but no message is currently defined from VAL server to update participation of AIMLE client (or remove) form AIML task and define actions on removal like task transfer or tranfer the last update etc. This CR attempts to fix this issue by letting AIMLE server remove AIMLE client from an ongoing task in update participation request specifying what needs to be done by AIMLE client (like sending the last training data or discarding it) when it’s removed from client set. Similar implementations are required in HFL training unsubscribe request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding action on removal of AIMLE client from client set in newly added participation update request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If AIMLE server removes AIMLE client from a ML client set or HFL training then the action on UE to treat last training data will remain undefined. Discarding it may cause resource wastage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10, 8.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 8.10	AIMLE client participation

### 8.10.1	General

The following clauses specify procedures, information flows, and APIs for AIMLE client participation.

### 8.10.2	Procedure

#### 8.10.2.1	AIMLE client participation

Figure 8.10.2.1-1: AIMLE client participation

1.	An AIMLE server sends an AIMLE client participation request to an AIMLE client to verify participation in AI/ML operation(s). The request includes information as described in Table 8.10.3.1-1.

2.	The AIMLE client acknowledge its willingness to be part of (or removed from) the AIMLE client set in an AIMLE client participation response sent to the AIMLE server. The response includes information as described in Table 8.10.3.1-2.

3.	Removal of client from client set shall cause termination of participation of AIMLE client. The AIMLE server can fetch the list of clients in client set or it can initiate the removal of only some clients if VAL server requests or due to operator policy.

4.	The AIMLE server shall also inform AIMLE client about final action to be performed by client for an ongoing task on its removal from ML training task. The client can discard the output of ongoing task on termination or it may transfer the last output of training on termination to AIMLE server.

#### 8.10.2.1	AIMLE client participation update request

This clause describes procedure to update participation request of a client. The update request can be initiated due to change in model training schedule, dataset requirement service requirement

Figure 8.10.2.1-1: AIMLE client participation update

- AIMLE server sends client participation update request to AIMLE client on order to update parameters related to model ID, AIML operations, operational schedule, dataset requirements or service requirements.

- AIMLE clients performs authorization check and validates requestor ID and model ID for the update request.

- The client can accept the AIMLE server’s update request or it may discard the updates. On failure of update request, AIMLE server can initiate client rmoval from client set.

- The client shall send training notifications to AIMLE server with training output as per updated client participation request.

### 8.10.3	Information flows

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| ML model ID | M | The ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Action on client removal | O | Discard training output, initiate task transfer or send last training output and then terminate client participation. |
| Service requirement | M | Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.10.3.2	AIMLE client participation response

Table 8.10.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.10.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Status | M | The status for the request: success or failure. |
| Cause | O | The cause for the request failure. |

* * * * Second Change * * * *

## 8.12	HFL training

### 8.12.1	General

AI/ML training is a highly iterative process and is performed over many training rounds. In the case of federated and distributed learning, the training is performed with many AIMLE clients. Due to the repetitive nature of AI/ML training, the AIMLE server can be configured to manage the training process over multiple training rounds for VAL servers. Note that the HFL training procedure supports horizontal federated learning, distributed learning, transfer learning, and split AI/ML.

The following clauses specify procedures, information flows, and APIs to support HFL training.

### 8.12.2	Procedure

Pre-conditions:

1.	AIMLE client discovery and selection have been performed.

2.	Datasets are available and prepared for AI/ML training at the AIMLE clients. The datasets are assigned identifiers for use in HFL training.

Figure 8.12.2-1: HFL training

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2.

2.	The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1.

3.	For the HFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training.

4.	Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the HFL training as described in the clause 8.10.

5.	The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1.

6.	Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 8.

7.	Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule.

8.	Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an VAL service ID for the HFL training operation.

9.	If errors were encountered, the procedure skips to step 10.

If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients.

If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 8 are repeated for the next training round.

When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository.

10.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

#### 8.12.2.1	HFL training unsubscribe request

The AIMLE server/operator may decide to unsubscribe to HFL training due to some scenarions like detection of rogue UE or AIMLE server getting blacklisted by network or due to changes in dataset or any other parameters of HFL training subscription. The AIMLE server may ask the UE to transmit the output of last training round before unsubscription.

Figure 8.12.2-1: HFL training unsubscribe request

- On getting ML model training cancellation request from VAL server, the AIMLE server can initiate HFL training unsubscribe request.

- The AIMLE server send HFL training unsubscribe request to AIMLE client containing requestor ID and other parameters.

- On receiving the HFL training unsubscribe request, client performs authorization check if AIMLE server is authorized to perform unsubscription based on requestor ID. If authorization check is successful. Client checks if output for ongoing training round shall be generated or discerded as per HFL training unsubscribe request “Notification settings” parameter.

- If server demands to transmit output of last training round the client sends the last training parameters and terminates HFL training.

### 8.12.3	Information flows

#### 8.12.3.1	HFL training subscription request

Table 8.12.3.1-1 shows the request sent by the AIMLE server to AIMLE clients for the HFL training subscription procedure.

Table 8.12.3.1-1: HFL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AI/ML model and model parameters | M | Information about the AI/ML model and model parameters for use in HFL training. |
| Dataset identifier | M | The dataset identifier associated with the dataset used for HFL training. |
| Number of data samples | M | The number of data samples required for a round of HFL training. |
| Operational schedule | O | A schedule for when training is to occur. |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event- triggered (e.g. based on percentage completion), upon completion of each training round. |

#### 8.12.3.2	HFL training subscription response

Table 8.12.3.2-1 shows the response sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.2-1: HFL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |
| VAL service ID | M | The VAL service identifier for the AIMLE HFL training operation. |

#### 8.12.3.x	HFL training unsubscribe request

Table 8.12.3.x-1 shows the request sent by the AIMLE server to AIMLE clients for the HFL training unsubscribe procedure.

Table 8.12.3.x-1: HFL training unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Subscription ID | M | An identifier for the subscription. |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event- triggered (e.g. based on percentage completion), upon completion of each training round. Configuration to Send last training round output/notification or discard it |

#### 8.12.3.2	HFL training unsubscribe response

Table 8.12.3.2-1 shows the response sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.2-1: HFL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |
| VAL service ID | M | The VAL service identifier for the AIMLE HFL training operation. |

#### 8.12.3.3	HFL training subscription notification

Table 8.12.3.3-1 shows the notification sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.3-1: HFL training subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| VAL service ID | M | The VAL service identifier for the AIMLE HFL training operation. |
| HFL training output | M | ML model parameters from HFL training. |
| Errors list | O | A list of errors encountered during a HFL training round. |
| Timestamp | O | A timestamp for the notification. |

* * * * End of Changes * * * *