3GPP TSG-SA WG6 Meeting #68	S6-253198

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0313 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on CAPIF-1/1e events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S6-253009, LS has been received from CT3 to SA6 with following questions: - Q-1: What are the information elements of the subscription request triggered by AMF to subscribe the “CAPIF-1/1e interaction events”? - Q-2: What are the information elements of the notification provided by CCF to AMF for the“CAPIF-1/1e interaction events”? This paper provides clarification based on the qeustions from the CT3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - IE table for subscription request has bee modified to add event criteria realted to CAPIF-1/1e events. - IE table for notification has been modified to add CAPF-1/1e related event details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage#3 work may not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.1, 8.8.2.3, 8.8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.2.1	Event subscription request

Table 8.8.2.1-1 describes the information flow for event subscription request from the subscribing entity to the CAPIF core function.

Table 8.8.2.1-1: Event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity |
| Event criteria | M | The event criteria include event type information like failure API invocation event, new API available event, API version change event, API location change event, API invoker status, Onboarded API invokers enrolled for particular service APIs, particular service APIs requested in "Discover service APIs" requests or "Open Discover service APIs" requests or discovered in the related responses, etc and other query information like service API identifier, service API name, onboarding criteria, reporting frequency of the event etc. |
| Notification reception information | O | The information of the subscribing entity for receiving the notifications for the event. |

* * * Next Change * * * *

#### 8.8.2.3	Event notification

Table 8.8.2.3-1 describes the information flow for event notification from the CAPIF core function to the subscribing entity. A notification about an event is sent to a subscribing entity if the event criteria in the related subscription match the corresponding attributes of the event content.

Table 8.8.2.3-1: Event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription |
| Event identifier | M | The unique identifier for the event. For the list of events, refer subclause 8.8.6 |
| Event related information | M | The event related information (e.g. time at which the event originated, location of event, statistics on number of CAPIF-1/1e events like number of onboarded API invokers enrolled for particular service APIs, number of times particular service APIs are requested in "Discover service APIs" requests or "Open Discover service APIs" requests or discovered in the related responses) |
| Event content | M | The content of the event information. |

* * * Next Change * * * *

### 8.8.6	List of CAPIF events

Table 8.8.6-1 provides a non-exhaustive list of CAPIF events.

Table 8.8.6-1: List of CAPIF events

| Events | Events Description |
| --- | --- |
| Availability of service APIs | Availability events of service APIs (e.g. active, inactive) |
| Service API updated | Events related to change in service API information |
| Monitoring service API invocations | Events corresponding to service API invocations |
| API invoker status | Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.) |
| API topology hiding status | Events related to API topology hiding status in CAPIF (created, revoked) |
| System related events | Alarm events providing fault information |
| Performance related events | Events related to system load conditions |
| CAPIF-1/1e interaction related events | Events related to CAPIF-1/1e interactions for a service API (e.g. number of onboarded API invokers enrolled for particular service APIs, number of times particular service APIs are requested in "Discover service APIs" requests or "Open Discover service APIs" requests or discovered in the related responses) |

* * * End of Change * * * *