3GPP TSG-SA WG6 Meeting #68	S6-253213

Gothenburg, Sweden, 25th – 29th August 2025

Source:	ZTE Corporation

Title:	Key Issue on support enhancements to application enablement services by utilizing Sensing results

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo (luo.wei11@zte.com.cn)

1. Introduction

According to 3GPP TS 22.137, SA1 requirements indicate that “the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met”, and that “a trusted third-party shall be able to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and  receive the corresponding sensing results.”. In addition, SA1 has studied 32 sensing related use cases in 3GPP TR 22.837 including smart home/city, UAS, traffic management, etc. SA6 also has defined a series of vertical application layer enablement services, such as 3GPP TS 23.255 (Application layer support for Uncrewed Aerial System (UAS)) and 3GPP TS 23.286 (Application layer support for V2X services). These application enablers target the UE as the end-user device, and leverage traditional 3GPP system capabilities, such as location services.

However, use cases for vertical applications using the enablement layer are limited because the current network lacks sensing capability for all objects, especially those without 3GPP network access, e.g. UAV without 3GPP network access, obstacles, etc. If 3GPP Application Enablement layer (e.g. 3GPP TS 23.255, 3GPP TS 23.286, etc.) can support acquisition of object information, especially the information of those without 3GPP network access could be considered, better service can be provided e.g., by covering more use cases, such as detecting (and potentially predicting) the position and trajectory of objects which are not connected to 3GPP network. Providing such information to 3GPP-registered UAVs, assistance can be provided for obstacle avoidance and the prevention of mid-air collisions.

2. Reason for Change

This solution proposes a new key issue for enhancing application enablement services  by utilizing sensing results.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15.

* * * First Change * * * *

X.X	Key Issue #X:Support enhancements to application enablement services by utilizing Sensing results

x.x.1	Description

According to 3GPP TS 22.137, SA1 requirements indicate that “the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met”, and that “a trusted third-party shall be able to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and  receive the corresponding sensing results.”.

NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area.

Meanwhile, a series of vertical application layer enablement services have been defined, such as 3GPP TS 23.255 (Application layer support for Uncrewed Aerial System (UAS)) and 3GPP TS 23.286 (Application layer support for V2X services). These application enablers target the UE as the end-user device, and leverage traditional 3GPP system capabilities, such as location services. However, use cases for vertical applications using the enablement layer are limited because the current network lacks sensing capability for all objects, especially those without 3GPP network access, e.g. UAV without 3GPP network access, obstacles, etc. If 3GPP Application Enablement layer (e.g. 3GPP TS 23.255, 3GPP TS 23.286, etc.) can support acquisition of object information, especially the information of those without 3GPP network access could be considered, better service can be provided e.g., by covering more use cases, such as detecting (and potentially predicting) the position and trajectory of objects which are not connected to 3GPP network.

x.x.2	Open Issues

This key issue will study:

- Whether and how to enhance the application enablement architecture and related functions to expose sensing results.

- Whether and how to enhance the vertical applications to request sensing service.

* * * End of Changes * * * *