3GPP TSG-SA6 Meeting #68	S6-253214

# Gothenburg, Sweden, 25th Aug 2025 - 29th Aug 2025               (revision of S6-253xxx)

Source:	Airbus

Title:	Solution to KI#3 – both parties as recording targets

Spec:	3GPP TR 23.700-39 V0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This pCR proposes a solution to KI#3: Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets.

2. Reason for Change

No solution exists for KI#3.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.2.0.

* * * First Change * * * *

6.x	Solution x (for KI#3): Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets

6.x.1	Description

6.x.1.0	General

As describe in TS 23.280 clause 10.18.3.2 (Procedure for recording communication sessions of an MC service user), a (SIPREC) recording session is started when a communication session involving recording target user is initiated.

If both parties of a private cal / one-to-one session are recording targets there are two options – record them using the same (SIPREC) recording session or start two separate recording sessions.

Both options have their pros and cons. However, for this issue 3GPP specifications should not require a specific solution i.e. this detail shall be left for implementation.

However, to guide CT1 and also to avoid this issue popping up again in few years, we should add a NOTE indicating that the issue has been concluded.

6.x.1.1	Functional model and reference points

No changes.

6.x.1.2	Configurations

No changes.

6.x.1.3	Procedures

A new NOTE is proposed to TS 23.280, see below:

TS 23.280 / 10.18.3.2	Procedure for recording communication sessions of an MC service user

Figure 10.18.3.2-1 below illustrates the recording procedure for communication sessions of an MC service user (MCPTT ID, MCVideo ID).

Pre-conditions:

-	The user (MCPTT ID or MCVideo ID) has been configured as a target for recording in the MC service user profile (see 3GPP TS 23.281 [12] clause A.3 and 3GPP TS 23.379 [16] clause A.3) and the user profile has been uploaded to the MC service server.

-	The target MC service user has performed MC user authentication and service authorization (see 3GPP TS 33.180 [25], clause 5.1.1 and clause 5.1.2) and has fetched its user profile(s) from the configuration management server.

Figure 10.18.3.2-1: Recording procedure for a target MC service user

1.	An MC communication session (MCPTT/MCVideo private call) involving the target user is initiated.

2.	A Session Recording Client (SRC) in the MC service server starts a recording session (see IETF RFC 7866 [40]) with the recording server.

NOTE 1:	If both parties are recording targets, the SRC may start either one recording session or two recording sessions (one for each party). This decision is out of scope of 3GPP.

3.	The MC communication session ends.

4.	The recording session ends.

5.	The recording server stores the recording session, and it is available for download and/or replay.

NOTE 2:	Steps 1-5 are repeated as long as none of the conditions of step 6 is true.

6.	When the user (MCPTT ID / MCVideo ID) is removed from the recording targets or the user logs off from the MC service or the UE is switched off, an ongoing recording session (if exists) is terminated.

6.x.2	Impacts on existing functional entities and reference points

None.

6.x.3	Solution evaluation

FFS.

* * * End of changes * * * *