3GPP TSG-SA WG6 Meeting #68	S6-253220

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution on Northbound interface of DCAR

Spec:	3GPP TR 23.700-57 v0.2.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This new key issue covers the Objective b) III. of FS_MMTel_Ph2_APP, i.e.

support of the application enablement aspects for A2P and P2A avatar communication, e.g. providing capabilities to application providers/Vertical service provider to leverage  avatar communication specified by SA2 in NG_RTC_Ph2 WID.

This pCR proposes this new Key Issue for TS23.700-57.

2. Reason for Change

Contribution with new Key Issue.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.2.0.

* * * First Change * * * *

6.x	Solution #x: Northbound interface of DCAR

6.x.1	Description

This solution resolves Key Issue #4: Northbound interface of DCAR.

This solution uses the enhanced architecture of MMTel Enabler as specified 3GPP TS 23.392 [2]. The architecture is illustrated in Figure 6.x.1-1

Figure 6.x.1-1: MMTel Enabler architecture supports Northbound interface of DCAR

The interactions between the MMTel Enabler Server and DCAR are provided by the MMTel Enabler Server via MMTel-4 reference point. This reference point is the northbound interface of DCAR and supports:

-	the MMTel Enabler Server to manage the DC application and profiles needed in MMTel service on DCAR; and

-	the MMTel Enabler Server to get the DC application and profile from the DCAR.

.

6.x.2	Procedures

Figure 6.x.2-1 below illustrates the procedure of uploading DC application and profile from the MMTel Enabler Server to DCAR. This procedure is a detailed procedure for the step 3 in the clause 8.2.2 of TS 23.392.

Pre-conditions:

1.	The DC application and profile have been configured to MMTel Enabler Server as specified in 3GPP TS 23.392 [2].

Figure 6.x.2-1: MMTel Enabler Client in the UE search the DC application list

1.	The MMTel Enabler Server sends a DC application and profile upload request to the DCAR. The request message includes information elements as specified in Table 6.x,2-1.

Table 6.x.2-1: Information elements in DC application and profile upload request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| MMTel Enabler Server ID | M | The identity of the MMTel Enabler Server |
| Security credentials | O | Security information required by the DCAR. |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application list are listed in the Table 8.2.2.3.1-2 of 3GPP TS 23.392 [2]. |

2.	Upon receiving the request, if required by the mmtel service provider, the DCAR validates if the requester is authorized for the request based on the MMTel Enabler Server ID. Then the DCAR stores the DC application and the related profile.

3.	The DCAR sends a DC application and profile upload response to the MMTel Enabler Server. The response message includes information elements as specified in Table 6.x,2-2.

Table 6.x.2-2: Information elements for DC application and profile upload response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile configuration response list | M | A list of DC application and profile configuration response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.2.3.2-2 of 3GPP TS 23.392 [2]. |

6.1.3	Impacts on Existing Nodes and Functionality

In this solution, a new reference point MMTel-4 between the MMTel Enabler Server and DCAR is introduced to the architecture of MMTel Enabler. The interaction  via MMTel-4 reference point in this solution is needed to be supported by both MMTel Enabler Server and DCAR.