3GPP TSG-SA WG6 Meeting #68	S6-253229

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0319 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving Editor’s Notes on CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor’s Notes related to CAPIF interconnection exist which can be solved as follows considering agreed S3-252428: Clause 6.4.15: Editor's note: Reference to the appropriate SA3 specification is needed. TS 33.122 clause 6.13 specifies the security procedures for CAPIF interconnection. The Editor’s Note can be removed, and the NOTE can be updated accordingly. Clause 8.25.2.11 and 8.25.2.13: Editor's note: The additional security aspects for the protection of the identity information of the API invoker are FFS in SA3. TS 33.122 clause 6.13 does not specify additional security aspects for the protection of the identity information of the API invoker or introduces an Editor’s Note in this regard. The Editor’s Notes can be removed. Clause 8.25.2.16: Editor's note: The information exchanged between CCF-B and CCF-A in the interconnection obtain API authorization procedure will be addressed by SA3. Editor's note: The information exchanged between CCF-B and CCF-A in the interconnection to obtain API invoker authentication and authorization procedure will be addressed by SA3. TS 33.122 clause 6.13.3 specifies the Authentication and authorization procedures between the API invoker onboarded into CCF-B and AEF registered in CCF-A, so the issue(s) are already addressed and the Editor’s Notes can be translated into Notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solution of the Editor’s Notes as indicated in the Reason for change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solved Open Issues in SA3 about interconnection remain as not solved in TS 23.222 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.14, 6.4.15, 8.25.2.11, 8.25.2.13, 8.25.2.16, 8.25.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.4.14	Reference point CAPIF-6 (between the CAPIF core functions of the same CAPIF provider)

The CAPIF-6 reference point exists between the CAPIF core functions within the same trust domain of CAPIF provider.

The CAPIF-6 reference point supports:

-	Publishing the service APIs information;

-	Discovering the service APIs information;

-	Obtaining security information for the API invoker upon accessing the service API; and

-	Providing authorization for the API invoker prior to accessing the service API.

NOTE:	The security aspects of CAPIF-6 are specified in clause 6.10  and 6.13 of 3GPP TS 33.122 [12].

* * * Second Change * * * *

### 6.4.15	Reference point CAPIF-6e (between the CAPIF core functions of different CAPIF providers)

The CAPIF-6e reference point exists between the CAPIF core function within the 3rd party trust domain and the CAPIF core function within the PLMN trust domain.

The CAPIF-6e reference point supports all the functions of CAPIF-6.

NOTE:	The security aspects of CAPIF-6e are specified in 3GPP TS 33.122 [12], clause 6.10 and 6.13.

* * * Third Change * * * *

#### 8.25.2.11	Interconnection revoke API invoker authorization request

Table 8.25.2.11-1 describes the information flow interconnection revoke API invoker authorization request from one CCF to another CCF.

Table 8.25.2.11-1: Interconnection revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the requesting CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

* * * Fourth Change * * * *

#### 8.25.2.13	Interconnection revoke API invoker authorization notify

Table 8.25.2.13-1 describes the information flow interconnection revoke API invoker authorization notify from one CCF to another CCF.

Table 8.25.2.13-1: Interconnection revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the notifying CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

* * * Fifth Change * * * *

#### 8.25.2.16	Interconnection API invoker obtaining authorization to access service API

NOTE:	The security aspects of this procedure are specified in clause 6.13 of 3GPP TS 33.122 [12].

* * * Sixth Change * * * *

#### 8.25.2.17	Interconnection Authentication and authorization between the API invoker and the AEF

NOTE:	The security aspects of this procedure are specified in clause 6.13 of 3GPP TS 33.122 [12].

* * * End of Changes * * * *