3GPP TSG-SA WG6 Meeting #68	S6-253236

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0233 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN in 5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An EN about how to handle SEAL Client functionality in a constrained MSGin5G UE is existing in clause 5.2 of Rel-18 version of TS23.554. Since the constrained MSGin5G UE has the same capabilities as unconstrained MSGin5G UE except that it cannot connect to the 3GPP network directly, the SEAL Client in the constrained MSGin5G UE is same with the the SEAL Client in the unconstrained MSGin5G UE. Therefore, the EN can be removed with some clarification on architecture. This EN is solved in Rel-19 in SP-240311. This EN does not impact the procedures in Rel-18, therefore the Rel-19 update can also be applied in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to remove the EN in clause 5.2 and update the architecture accordingly in Rel-18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN exists in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2	Application Architecture

Figure 5.2-1 shows the application architecture of the MSGin5G service. The MSGin5G service shall fulfil the service requirements which are enumerated in 3GPP TS 22.262 [2] and the architecture requirements enumerated in clause 4.

Figure 5.2-1: Application Architecture of the MSGin5G Service

The Application Client resides on the same UE with the MSGin5G Client as shown in MSGin5G UE-1, or resides on a different UE and interacts with the MSGin5G Client over the MSGin5G-5 reference point as shown in UE-2.

The MSGin5G Client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The MSGin5G Server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [5].

The MSGin5G UE-1 communicates with MSGin5G Server over MSGin5G-1 reference point.

The Legacy 3GPP Message Gateway interacts with MSGin5G Server over MSGin5G-2 reference point on behalf of Legacy 3GPP UE (e.g., SMS, NIDD).

The Non-3GPP Message Gateway interacts with MSGin5G Server over MSGin5G-4 reference point on behalf of Non-3GPP UE.

The Broadcast Message Gateway interacts with MSGin5G Server over MSGin5G-7 reference point.

NOTE 1:	A SEAL Group Management Server and a SEAL Configuration Management Server (both specified in 3GPP TS 23.434 [5]) may be collocated in the MSGin5G Server. A SEAL Configuration Management Client specified in 3GPP TS 23.434 [5] may be collocated in the MSGin5G Client, Legacy 3GPP Message Gateway and Non-3GPP Message Gateway. The implementation of such deployment option is out of scope of the present specification.

NOTE 2:	Depending on the non-3GPP message service, the interaction between Non-3GPP message client and Non-3GPP Message Gateway may involve 3GPP Core Network.

NOTE 3:	3GPP Core Network may not be involved for the interaction between the Non-3GPP Message Gateway and the MSGin5G Server.

NOTE 4:	If the Application Client on UE-2 communicates with the MSGin5G Client on UE-1 over the MSGin5G-5 reference point then the transport layer communication is based on the Unicast mode 5G ProSe Direct Communication specified in 3GPP TS 23.304 [18] or is implementation specific.

The architecture shown in Figure 5.2-2 and Figure 5.2-3 illustrate architectural options for providing MSGin5G services for constrained devices. In these figures, the MSGin5G UE-2 is a constrained device which does not connect to the 3GPP network directly for message exchange with MSGin5G Server (e.g. UE-2 is out of 3GPP RAN coverage, with or without authorization to use UE-to-Network relay). If allowed by configuration, the MSGin5G UE-2 can use the options listed below to communicate with the MSGin5G Server:

-	the MSGin5G UE-2 uses an UE-1 as relay;

-	the MSGin5G UE-2 interacts with an MSGin5G Gateway UE which supports MSGin5G Gateway Client.

Figure 5.2-2 shows the application architecture of MSGin5G UE-2 using an UE-1 as a relay. The SEAL Client(s) residing on the UE-1 also acts as relay for the SEAL Client(s) residing on the MSGin5G UE-2 as specified in 3GPP TS 23.434 [5].

NOTE 5:	In this option, MSGin5G UE-2 discovers the UE-1 and then uses the UE-1 as a ProSe UE-to-Network Relay by using the procedure specified in 3GPP TS 23.304 [18].

NOTE 6:	The UE-1 can be any UE supporting ProSe UE-to-Network Relay capability and may not be an MSGin5G UE.

Figure 5.2-2: MSGin5G UE-2 using UE-1 as a relay

Figure 5.2-3 shows the application architecture of MSGin5G UE-2 with MSGin5G Client interacts with an MSGin5G Gateway UE over the MSGin5G-6 reference point. The SEAL Client(s) residing on the UE-2 communicates with the SEAL Client(s) residing on the UE-1 via SEAL-PC5 reference as specified in 3GPP TS 23.434 [5].

NOTE 7:	Both MSGin5G Client functionality and MSGin5G Gateway service functionality are internal functionalities of MSGin5G Gateway Client. The interaction between MSGin5G Client functionality in the MSGin5G Gateway Client and the MSGin5G Gateway service functionality is implementation specific.

Figure 5.2-3: MSGin5G UE-2 with MSGin5G Client interacts with an MSGin5G Gateway UE

NOTE 8:	MSGin5G-6 reference point is based on the Unicast mode 5G ProSe Direct Communication specified in 3GPP TS 23.304 [18].

Figure 5.2-4 illustrates the functional model for interconnection between MSGin5G servers.

Figure 5.2-4: Interconnection between MSGin5G Servers

To support distributed SEAL server deployment in one PLMN, and delivering MSGin5G messages between different PLMNs, the MSGin5G Server interacts with another MSGin5G Server over MSGin5G-8 reference point.