3GPP TSG-SA WG6 Meeting #68	S6-253242

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0060 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group management – update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently the spatial anchor group management functionality covers the procedures for spatial anchor group creation and deletion but the procedure for spatial anchor group update is not captured. This CR adds the missing spatial anchor group update procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proedure to update the spatial anchor group |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of spatial anchor group management will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.3.7.2.x	Spatial anchor group update

Figure 8.3.7.2.x-1 illustrates the spatial anchor group update procedure through which the requestor SAn client or the VAL server can request the SAn server to update a spatial anchor group.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.7.2.x-1: Spatial anchor group update

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group update request to the SAn server. This request includes a requestor identifier, security credentials, identifier of the spatial anchor group, list of spatial anchors identified by the spatial anchor IDs to be associated as part of the spatial anchor group, list of spatial anchors to be dis-associated as part of the spatial anchor group , updated service area etc as specified in Table 8.3.7.3.x-1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the updating the spatial anchor group. If the requestor is authorized, the SAn server updates the stored spatial anchor group and its associated information based on the request received in step 1.

3.	The SAn server sends a spatial anchor group update response as specified in Table 8.3.7.3.y-1 to the requestor. If the SAn server updated spatial anchor group, the response includes an indication of success otherwise, the response includes an indication of failure and reason for failure.

* * * Next Change * * * *

##### 8.3.7.3.x	Spatial anchor group update request

Table 8.3.7.3.x-1 describes information elements for the spatial anchor group update request from the SAn client or VAL server to the SAn server.

Table 8.3.7.3.x-1: Spatial anchor group update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor group ID | M | Identity of the spatial anchor group to be updated |
| Group description | O | Human readable description about the spatial anchor group. |
| Service area | O | Service area where the spatial anchor group is visible or accessible. |
| VAL service information | O | Information about the application service associated with the spatial anchor group. |
| Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor group. |
| Service ID | O | The identifier of the application service. |
| List of spatial anchors IDs | O | List of spatial anchors IDs to be associated with the group. |
| List of spatial anchor IDs | O | List of spatial anchor IDs to be dis-associated from the spatial anchor group |

##### 8.3.7.3.y	Spatial anchor group update response

Table 8.3.7.3.y-1 describes information elements for the spatial anchor group update response from the SAn client or VAL server to the SAn server.

Table 8.3.7.3.y-1: Spatial anchor group update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the spatial anchor group create request was successful. |
| >Spatial anchor group ID | O | The unique identifier of the updated spatial anchor group. |
| >List of Spatial anchor IDs | O | List of spatial anchor IDs associated with the spatial anchor group |
| >List of spatial anchor IDs | O | List of spatial anchor IDs dis-associated/de-linked from the spatial anchor group |
| Failure response | O (NOTE) | Indicates that the spatial anchor update request failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |

* * * End of Changes * * * *