3GPP TSG-SA WG6 Meeting #68	S6-253248

Gothenburg, Sweden 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0479 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call modify by an authorised user using functional alias(es) as participant list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, NOKIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Usage of functional alias(s) instead of MCX Service ID(s) is a major functionality for the Railways to adopt MCX Services. The following stage 1 requirements on ad hoc group calls using functional alias(es) as defined in 22.280 19.7.0 are not covered by stage 2: [R-6.15.5.2-001b] The participant of an MCX Ad hoc Group Communication who has activated several Functional Alias(es) shall receive only one communication based on the unique MCX Service ID. [R-6.15.5.2-001c] The MCX Service shall provide a mechanism for the participant of an Ad hoc Group Communication to determine the Functional Alias, through which this participant is addressed by the communication. The present CR clarifies usage of functional alias when a user manually selects a list of participant (using functional alias(es) when requesting to modify an ad hoc group call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding functional alias(es) in the ad hoc group call modify of participant by an authorised user |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements of ad hoc group calls are not fully specified. In addition, the Railways needs are not fully covered to enable adoption of MCX services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.11, 10.19.3.1.4, 10.19.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Change #1*****************************************************************************

#### 10.19.2.11	Modify ad hoc group call participants request (MCPTT client – MCPTT server)

Table 10.19.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCPTT client to the MCPTT server.

Table 10.19.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| Functional alias | O | The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call participants |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants needs to be modified |
| MCPTT ID list (see NOTE 1, NOTE 2) | O | List of additional MCPTT users to be added to the on-going ad hoc group call |
| Functional alias list (see NOTE 2) | O | List of additional functional alias(es) of the MCPTT users to be added to the on-going ad hoc group call |
| MCPTT ID list (see NOTE 1, NOTE 3) | O | List of MCPTT users to be removed from the on-going ad hoc group call |
| Functional alias list (see NOTE 3) | O | List of functional alias(es) of the MCPTT users to be removed from the on-going ad hoc group call |
| NOTE 1: Either one or both of these information elements is present NOTE 2: At least one identity shall be present. If both are present, the functional alias is just for information. NOTE 3: At least one identity shall be present. If both are present, the functional alias is just for information. |  |  |

***** End of Change #1 ***********************************************************************

***** Change #2*****************************************************************************

##### 10.19.3.1.4	Modification of ad hoc group call participants by an authorized user

Figure 10.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by an authorized user.

Pre-conditions:

1.	An ad hoc group call which is setup with participants list provided by the initiating MCPTT user as described in clause 10.19.3.1.1 or based on the criteria provided by the initiating MCPTT user as described in clause 10.19.3.1.3 is already in progress.

2.	The participants of the MCPTT ad hoc group call belong to the single MCPTT system.

3.	The MCPTT users on MCPTT client 1, MCPTT client 3 to MCPTT client n are on an ongoing ad hoc group call.

4.	The MCPTT user at MCPTT client 1 determines to remove the user of MCPTT client 3 from the ad hoc group call and add user of MCPTT client 2 into the on-going ad hoc group call.

Figure 10.19.3.1.4-1: Modification of ad hoc group call participants by an authorized user

1.	The MCPTT user at the MCPTT client 1 is authorized and requests to modify ad hoc group call participants. The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server in order to remove MCPTT client 3 from the ongoing ad hoc group call and add MCPTT client 2 into it.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call regardless of the original group call setup parameters. If the request to modify ad hoc group call participants contains a list of functional alias(es) instead of a list of MCPTT ID(s), the MCPTT server shall resolve the functional alias(es) to the corresponding MCPTT ID(s) for which each functional alias is active. The MCPTT server shall also check whether MCPTT client 1 is allowed to use the functional alias of MCPTT client 2 to be added to the MCPTT ad hoc group call and whether MCPTT client 2 is allowed to receive an MCPTT ad hoc group call from MCPTT client 1 using the functional alias. When the group participants were initially determined by the MCPTT server with criteria and MCPTT users are to be removed, the MCPTT server removes MCPTT clients and marks them so that the MCPTT server will not add them back to the ad hoc group call based on the criteria. Participants to be added shall be marked and kept in the ad hoc group call and shall not be removed by the MCPTT server even if they do not meet the call criteria.

NOTE 1:	In the above case participants being removed can be added back and the participants being added can be removed by the authorized user via the modify ad hoc group call participants request.

3.	The MCPTT server sends modify ad hoc group call participants response to the MCPTT client 1.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server.

7.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE 2:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

9.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server. The MCPTT server considers the MCPTT user as implicitly affiliated to the ad hoc group.

10.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

11.	The MCPTT server may notify the participants about the change in the participants of on-going ad hoc group call.

***** End of Change #2 ***********************************************************************

***** Change #3*****************************************************************************

##### 10.19.3.2.6	Modification of ad hoc group call participants by an authorized user

Figure 10.19.3.2.6-1 below illustrates the MCPTT user modifying the ad hoc group call participants procedure involving multiple MCPTT systems.

Pre-conditions:

1.	An MCPTT ad hoc group call is already in progress and the participants list provided by the originating MCPTT user while initiating the MCPTT ad hoc group call.

2.	The participants of the MCPTT ad hoc group call belongs to the multiple MCPTT system.

3.	The MCPTT user at MCPTT client 1 determines that the MCPTT client 3 needs to be invited to on-going ad hoc group call and MCPTT client 2 needs to be removed from the on-going ad hoc group call from the primary MCPTT system.

4.	The MCPTT user at MCPTT client 1 determines that the MCPTT client 5 needs to be invited to on-going ad hoc group call and MCPTT client 4 needs to be removed from the on-going ad hoc group call from the partner MCPTT system.

Figure 10.19.3.2.6-1: Modification of ad hoc group call participants by an authorized user

1.	The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server 1 to remove both MCPTT client 2 and MCPTT client 4 from the on-going ad hoc group call and to add both MCPTT client 3 and MCPTT client 5 into on-going ad hoc group call.

2.	The MCPTT server 1 verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call. If the request to modify ad hoc group call participants contains a list of functional alias(es) instead of a list of MCPTT ID(s), the MCPTT server 1 shall resolve the functional alias(es) to the corresponding MCPTT ID(s) for which each functional alias is active. The MCPTT server 1 shall also check whether MCPTT client 1 is allowed to use the functional alias of MCPTT client 5 to be added to the MCPTT ad hoc group call and whether MCPTT client 5 is allowed to receive an MCPTT ad hoc group call from MCPTT client 1 using the functional alias.

3.	The MCPTT server 1 sends modify ad hoc group call participants response to the MCPTT client 1.

4.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 3 from the primary MC system be invited to an on-going ad hoc group call and successfully added to the call according to the procedure defined in clause 10.19.3.1.4.

5.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 2 from the primary MC system be removed from the on-going ad hoc group call and successfully removed from call according to the procedure defined in clause 10.19.3.1.4.

6.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 5 from the partner MC system be invited to an on-going ad hoc group call. The MCPTT server 1 sends the ad hoc group call request to the MCPTT client 5.

7.	The MCPTT client 5 notifies the user about the incoming ad hoc group call.

8.	The MCPTT client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCPTT server 1.

9.	The on-going ad hoc group call is updated by adding MCPTT client 5 which is based on the modify participant list provided by the MCPTT client 1.

10.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 4 from the partner MC system be removed from the on-going ad hoc group call. The MCPTT server 1 sends the ad hoc group call leave request to the MCPTT client 4.

11.	The MCPTT client 4 notifies the user of the ad hoc group call leave request.

12.	The MCPTT client 4 sends the ad hoc group call leave response to the MCPTT server 1.

13.	The on-going ad hoc group call is updated by removing MCPTT client 4, which is based on the modify participant list provided by the MCPTT client 1.

NOTE:	The ad hoc group call request and response exchanged between MCPTT server 1 of primary MC system and MCPTT client 4/MCPTT client 5 will always traversal through the MCPTT server 2.

14.	The MCPTT server 1 may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

15.	The MCPTT server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group call.

***** End of Change #3 ***********************************************************************