3GPP TSG-SA WG6 Meeting #68	S6-253251

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0158 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to SEALDD rate control to support rejected clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_PH2 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD enabled bandwidth control for different VAL users procedure accepts or rejects the VAL users request based on the available bandwidth capacity between SEALDD server and VAL server. If the SEALDD server cannot meet the VAL user bandwidth requirements due to lack of capacity then it rejects the request. This leads to negative impact on the VAL user application QoS and QoE. The VAL server is not aware of the bandwidth utilization at the SEALDD server and because of insufficient bandwidth capacity, its VAL UEs are rejected. And since VAL server lack this information, VAL server cannot perform any adjustments like modifying the policy if required to accommodate more VAL users and reduce the failure rate and thereby increasing the QoS and QoE of its users. The proposed solution provides notification to the VAL server based on the request. For example notify the VAL server when 70% of the total bandwidth utilization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides enhancements to reduce the connection failure rate for VAL users. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL users cannot establish SEALDD connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.2.1, 9.2.3.x(new), 9.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.8.2.1	SEALDD enabled bandwidth control for different VAL users

The SEALDD layer provides the differentiated data delivery service with different bandwidth experience for VAL users, where the VAL server provides the bandwidth limit (i.e., minimum bandwidth requirement and maximum bandwidth limit) for VAL users. Figure 9.8.2.1-1 illustrates the procedure for bandwidth control for different VAL users.

Pre-conditions:

1.	The VAL server has discovered and selected the SEALDD server by CAPIF functions as specified in clause 9.4.2.

2. The SEALDD server has subscribed to 5GC for QoS monitoring of the specific UE related to the VAL user, as defined in clause 5.2.6.9 in 3GPP TS 23.502 [6].

3. The SEALDD policy (i.e, the bandwidth control policy) has been configured in SEALDD server, as described in clause 9.10.

Figure 9.8.2.1-1: SEALDD enabled bandwidth control transmission procedure

- The VAL server sends a Sdd_regularTransmission request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), the VAL server’s total bandwidth limit and the bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, request for bandwidth limit utilization notification. The request for bandwidth limit utilization notification provides the criteria to notify the VAL server for the bandwidth limit utilization. e.g., threshold based percentage of the VAL server’s total bandwidth limit

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server.

3.	The VAL client sends a SEALDD service request to SEALDD client.

4.	The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client gets the SEALDD server's address.

5.	The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the VAL user or UE identity, XR application device capability information as per Table 9.2.3.3-1. The SEALDD client retrieves the capability information as per the 3GPP TS 26.118 [20] clause 9.2.3 and clause 9.2.6.

6.	The SEALDD server performs bandwidth limit check according to the VAL user’s bandwidth limit, Bandwidth control policy as described in Table 9.10.3.1-1, the current SEALDD traffic delivery status, the VAL server’s total bandwidth limit and/or the related UE’s current network status (i.e. via QoS monitoring report from the 5GC). As per the Bandwidth control policy, the SEALDD server may release connections to allocate the available bandwidth for VAL users. If the available bandwidth (i.e. the remaining bandwidth that is used for the VAL user without exceeding the VAL server’s total bandwidth limit) fails to meet the VAL user’s minimum bandwidth requirement, the SEALDD server will reject the SEALDD client’s connection establishment request.

7.	When the available bandwidth meets the VAL user’s requirement, the SEALDD client establishes the SEALDD connection with the SEALDD server. The SEALDD server calculates the suggested traffic transmission bandwidth to the SEALDD client according to the VAL user’s bandwidth limit and the related UE’s current network status (i.e. via QoS monitoring report or ECN marking for L4S report from the 5GC) and XR application device capability information (e.g., codec).  The SEALDD server also calculates the remaining bandwidth available for the new VAL Users. If used bandwidth is above the threshold criteria mentioned in Request for bandwidth limit utilization notification in step 1, then the SEALDD server calculates and notifies the VAL server with the number of VAL UEs connected and their allocated bandwidth, number of VAL UEs can meet the minimum bandwidth and number of VAL UEs can meet the maximum bandwidth using SEALDD regular transmission notify message.

8.	If the bandwidth limit check is failed (i.e., the available bandwidth fails to meet the VAL user’s minimum bandwidth requirement) in step 6, the SEALDD server sends Sdd_RegularTransmissionConnection_Establish response with the failed result (i.e., reject the connection establishment) and the pending timer to trigger the re-connection from SEALDD client.

 If the bandwidth limit check is successful (i.e., the available bandwidth meets the VAL user’s requirement) in step 6, the SEALDD server sends Sdd_RegularTransmissionConnection_Establish response with the successful result and/or the suggested traffic transmission bandwidth.

9.	If used bandwidth is above the threshold criteria mentioned as indicated in the request message in step1 to send bandwidth limit utilization notification, then the SEALDD server calculates and notifies the VAL server with the available remaining bandwidth, number of VAL UEs connected and their allocated bandwidth, number of VAL UEs can meet the minimum bandwidth and number of VAL UEs can meet the maximum bandwidth using SEALDD regular transmission notify message.

NOTE:	The SEALDD server can re-allocate the available bandwidth resource to different VAL users according to the configured bandwidth control policy, as described in clause 9.10.

If the connection establishment is rejected, the SEALDD client re-establishes SEALDD connection by performing steps 5-8, when the pending timer is expired.

For the uplink application traffic, the SEALDD client buffers or drops some packets when the uplink traffic from VAL client exceeds the suggested traffic transmission bandwidth. Similarly, for the downlink application traffic, the SEALDD server buffers or drops some packets when the downlink traffic from VAL server exceeds the suggested traffic transmission bandwidth.

* * * Next Change * * * *

### 9.2.3	Information flows

#### 9.2.3.1	SEALDD enabled regular transmission request

Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service.

Table 9.2.3.1-1: SEALDD enabled Regular transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the traffic from the SEALDD server |
| QoS information | O | QoS information provided by VAL server |
| VAL server’s total bandwidth limit | O (See NOTE) | The total bandwidth limit of VAL server, including UL/DL |
| VAL users’ bandwidth limit | O (See NOTE) | The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL |
| Request for bandwidth limit utilization notification | O (See NOTE) | Indicates the criteria to notify the VAL server for the bandwidth limit utilization. e.g., threshold based percentage of the VAL server’s total bandwidth limit |
| NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. |  |  |

#### 9.2.3.x	SEALDD regular transmission notify

Table 9.2.3.x-1 describes the information flow from the SEALDD server to the VAL server to notify of updates in the regular application transmission request.

Table 9.2.3.x-1: SEALDD regular transmission notify

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information connection information | O | Address information (e.g., IP address and/or port, URL) of the SEALDD server |
| Bandwidth utilization status | O | Indicates the bandwidth utilization limits status |
| > Available remaining bandwidth | O | Indicates the available remaining bandwidth |
| > Current number of VAL UEs | O | Indicates the number of connected VAL UEs and allocated bandwidth in SEALDD server |
| > Minimum number of VAL UEs supported | O | Indicates the number of VAL UEs can be supported with maximum bandwidth limit |
| > Maximum number of VAL UEs supported | O | Indicates the number of VAL UEs can be supported with minimum bandwidth limit |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * Next Change * * * *

### 9.10.3	Information flows

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID). |
| Identities | O | Identifier of the VAL UEs or VAL users, or VAL UE group ID(as defined in clause 7.5 of 3GPP TS 23.434 [4]), or VAL UE identity list for which SEALDD policy applies |
| Slice identifier | O | The slice identifier (S-NSSAI) for which SEALDD policy applies. |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity. |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL, reuse and reallocate. Bandwidth for inactive VAL UEs to admit new VAL users |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service. |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked. |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy. |
| > Non-3GPP access measurement policy | O | Indicates the non-3GPP access (like WLAN) measurement policy (e.g. WLAN SSIDs/BSSID, signal strength thresholds(high/low), time-based threshold for offload to WLAN). |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2]. |
| > Multi-modal flows alignment policy | O | Indicates the information to do the multi-modal flows alignment, e.g., Multi-modal Service ID, maximum acceptable duration for traffic flow alignment. |
| > Policy expiration time | O | Indicates the validity period of the policy. |
| > UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS as specified in ITU-T P.1203.3 [18]) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

* * * END of Changes * * * *