3GPP TSG-SA WG6 Meeting #68	S6-253275

Gothenburg, Sweden 25th – 29th Aug 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New KI on architecture and functional model for AIoT services

Spec:	3GPP TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new KI on architecture/functional model and related functionalities for AIoT services.

2. Reason for Change

AIoT devices are the new type of IoT devives which are different from the traditional cellular IoT devices (e.g. NB-IoT/eMTC devices). Whether a new architecture/functional model or existing IoT related application enablers could support the AIoT devices should be studied further.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

5	Key issues

5.x	Key issue #x: Architecture and functional model for AIoT services

5.x.1	Description

Based on the definition in 3GPP TS 22.369 [22369], the AIoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor), and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. An AIoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). And AIoT devices can be maintenance free and can have long life span (e.g. more than 10 years). Obviously, the AIoT devices are the new type of IoT devices which are different from the traditional cellular IoT devices (e.g. NB-IoT/eMTC devices).

In SA6, there are already some IoT services/devices that have been studied in FS_ACE_IOT and PINAPP which the related normative work has been specified in 3GPP TS 23.434[23434] and 3GPP TS 23.542[23542]. Especially for PINAPP, a new application enabler (i.e. PIN enabler) has been developed to support the PIN service and personal IoT devices. As the AIoT devices are the new type of IoT devices, whether a new architecture/functional model or the existing IoT related application enablers could support the AIoT devices should be studied further.

5.x.2	Open Issues

Based on the above analysis, the following open issues need to be studied in the application enabled layer:

-	Whether and how a new architecture/functional model needs to be developed to support the AIoT services.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[23434]	3GPP TS 23434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows ".

[23542]	3GPP TS 23542: "Application layer support for Personal IoT Network".

* * * End of Change * * * *