3GPP TSG-SA WG6 Meeting #68	S6-253326

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-251xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0251 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location information for private and group call requests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Information Flow for the listed call requests (Group Call Request, Emergency Group Call Request, Imminent Peril Group Call Request, Private Call Request, Emergency Private Call Request) is already defined for all relevant communication directions. For Group Call Request, the optional Location Information is already included. However, for the other call types the optional Location Information is missing for all communication directions, which creates inconsistencies and may prevent location data from being available when needed. Adding Location Information ensures consistent availability across all call types and communication directions, thereby improving operational capability in situation-critical scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the optional Location Information attribute to these information flows for all directions: - Emergency Group Call Request - Imminent Peril Group Call Request - Private Call Request - Emergency Private Call Request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not added, there is an inconsistency between call types and communication directions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.18, 7.1.2.2.22, 7.2.2.2.1, 7.2.2.2.2, 7.2.2.2.6, 7.2.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.1.2.2.18	MCVideo emergency group call request

Table 7.1.2.2.18-1 describes the information flow emergency group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.18-1: MCVideo emergency group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Emergency indicator | M | Indicates that the group call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested priority | O | Priority level requested for the call |
| Implicit transmit media request (see NOTE) | O | Indicates that the originating client requests the permission to transmit media. |
| Location information | O | Location of the calling party |
| NOTE: This element shall be included only when the originating client requests the the permission to transmit media. |  |  |

# * * * * Second change * * * *

##### 7.1.2.2.22	MCVideo imminent peril group call request

Table 7.1.2.2.22-1 describes the information flow MCVideo imminent peril group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.22-1: MCVideo imminent peril group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Requested priority | O | Priority level requested for the call |
| Imminent peril indicator | M | Indicates that the group call request is an imminent peril call |
| Location information | O | Location of the calling party |

# * * * * Third change * * * *

##### 7.2.2.2.1	MCVideo private call request (MCVideo client – MCVideo server)

Table 7.2.2.2.1-1 describes the information flow MCVideo private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.1-1: MCVideo private call request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID (see NOTE) | O | The MCVideo ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request | O | An indication that the user is also requesting the permission to transmit video |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| Requested priority | O | Application priority level requested for this call |
| Location information | O | Location of the calling party |
| NOTE: Either an MCVideo ID or a functional alias shall be present, but not both. |  |  |

# * * * * Fourth change * * * *

##### 7.2.2.2.2	MCVideo private call request

Table 7.2.2.2.2-1 describes the information flow MCVideo private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.2-1: MCVideo private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request (see NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| Location information | O | Location of the calling party |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |

# * * * * Fitfh change * * * *

##### 7.2.2.2.6	MCVideo emergency private call request (MCVideo client to MCVideo server)

Table 7.2.2.2.6-1 describes the information flow MCVideo emergency private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.6-1: MCVideo emergency private call request (MCVideo client to MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested priority | O | Priority level requested for the call |
| Location information | O | Location of the calling party |

# * * * * Sixth change * * * *

##### 7.2.2.2.7	MCVideo emergency private call request

Table 7.2.2.2.7-1 describes the information flow MCVideo emergency private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.7-1: MCVideo emergency private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media. |
| SDP offer | M | Media parameters of MCVideo client. |
| Location information | O | Location of the calling party |

# * * * * End of changes * * * *