3GPP TSG-SA WG6 Meeting #68	S6-253332

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Lenovo

Title:	AIMLAPP architecture enhancements

Spec:	3GPP TR 23.700-83 v0.3.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides architecture enhancements for AIMLAPP considering roaming and multi-operator scenarios.

2. Reason for Change

This paper provides architecture enhancements for AIMLAPP considering roaming and multi-operator scenarios.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

### 6.2.1	AI/ML enablement functional model for multi-operator services

In this clause the architecture enhancements relate to the support for multi-operator services. There are two cases for the multi-operator service support:

- An AIMLE server connected to AIMLE clients over different PLMNs

- Each AIMLE server connected to AIMLE clients of single PLMN, whereas the AIMLE server interact each other via AIML-E.

In the fist scenario, the AIML service (VAL service, ASP service) runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN.

The AIML service can be an application enablement service (AIMLE client to AIMLE server) where AIMLE-UU can be over multiple MNO networks. Here, the VAL UEs may connect in a local fashion (UE-to-UE) via AIMLE-PC5.

In Figure 6.2.1-1, the VAL UE#1 is connected to PLMN#1 and VAL UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. In this scenario, sdifferent VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server. One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks.

Figure 6.2.1-1 architecture for multi-operator support with common AIMLE server

In the second scenario, as depicted in Figure 6.2.1-2, the VAL UEs are assumed to have AIMLE clients installed and active and connect to different AIMLE servers provided by different MNOs or trusted 3rd parties of the MNOs.

In Figure 6.2.1-2, each AIMLE server connects to each PLMN. The VAL UE 1 is connected to PLMN1 and AIMLE server 1 and VAL UE 2 is connected to PLMN 2 and AIMLE server 2. The ML repository is accessible to both AIMLE servers via ML-R, and there might be also multiple ML repositories corresponding to different PLMNs based on the implementation. In this case, MLR-E is the new interface corresponding to interactions among repositories for synchronizing the registries when applicable (e.g. ML member, models,..).

One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks, and AIMLE server 1 is provided by a service provider /MNO in an edge area, whereas AIMLE server 2 is provider by a regional network operator. One of the AIMLE servers may be also a cloud provider or 3rd party VMNO connecting to one of the MNO1 and MNO2.

Figure 6.2.1-2 architecture for multi-operator support with distributed AIMLE servers

### 6.2.2	AI/ML enablement functional model for roaming scenarios

This clause describes the AIMLAPP architecture for roaming UEs. This architecture is aligned with network roaming architecture, since it re-uses existing network roaming principles and describes the AIML service continuity while VAL UEs are roaming; hence how AIMLE spans across HPLMN and VPLMN is discussed. Such architecture applies to both roaming models (LBO and HR roaming architectures).

More specifically, a VAL UE is roaming or expected to roam from PLMN1 (denoted as HPLMN) to PLMN2 (denoted as VPLMN) while having ongoing application session (AIML service). AIMLE server is mapped to single PLMN; hence the roaming may lead to update of the serving AIMLE server for the UE. More specifically, the AIMLE client 1 needs to change AIMLE server while roaming to the target area covered by different PLMN.

There are two variants for the change of the AIMLE server:

- Scenario 1: AIMLE server changes and the ML/FL server role for the ML/FL process changes. So, another AIMLE Server needs to undertake the ML/FL server role/ functionality for the ongoing ML/FL process

- Scenario 2: AIMLE server changes but the ML/FL server role stays the same in AIMLE server. This doesn’t require context transfer for the role/capability change, and only the serving AIMLE server changes.

While the AIMLE server changes due to the UE roaming, the ML repository which stores the ML models, and the ML/FL member information may also need to change based on the deployment. There are two cases which may require further interactions among ML repositories: 1) each AIMLE server is deployed at the edge/cloud by the edge/cloud provider, and the ML repository is a single entity which is expected to store roaming info (multi-MNO info), and there is no update required based on the switching of the AIMLE server for the ongoing AI/ML service 2) each AIMLE server is deployed by each MNO, and the ML repository is different per PLMN.

In Figure 6.2.2-1, the AIMLAPP architecture for enabling AI/ML services for roaming scenarios is showcased. In this architecture, AIML-UU is the interface between the AIMLE client and the AIMLE server, which can be over HPLMN or VPLMN which VAL UE is roaming. AIML-E supports the interaction among AIMLE servers deployed by different MNOs, whereas the MLR-E is the new interface supporting the interaction among repositories for aligning the information on the ML models and ML/FL members to allow the continuity of the AIML service while the VAL UE is roaming.

Figure 6.2.2-1 architecture for roaming support

### 6.2.3	AI/ML enablement functional model for federation

Editor’s Note: The enhancements for federation support are FFS.

## 6.3	Functional Elements

The functional elements corresponding to the architecture will be presented in this clause.

## 6.4	Reference Points

The reference points corresponding to the architecture will be presented in this clause.

* * * Next Change * * * *