3GPP TSG-SA WG6 Meeting #68	S6-253344

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	SA6 Rel-20 6G SID Moderator

Title:	Discussion on work areas of SA6 6G Application Enablement study

Agenda Item:	11.1

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

Abstract: Proposed work areas and work tasks of SA6 6G Application Enablement study

# 1	Discussion

Several input papers were discussed during SA6 Rel-20 Workshop on 6G-study and the agenda is in the following link:

https://www.3gpp.org/ftp/tsg_sa/WG6_MissionCritical/Informal_ConfCalls/2025/IWS_20250625and26_SA6_Rel-20_6G-study_planning/chairs_notes/inbox/SA6_6G-study_WS-Agenda_v4.docx

Subsequently, it was decided that moderator shall provide a summary of potential Work Areas / Work Tasks for the 6G-study on application enablement including questions for potential Work Areas / Work Tasks to be used for moderated discussion. During SA6#68 it is proposed to discuss the initial set of Work Areas / Work Tasks listed in this paper and figure out the contentious aspects that need further moderated discussion, and identify any missing aspects.

Moderated discussion on the 6G SID is expected after SA6#68 meeting. Based on the moderated discussion the moderator will provide a summary and a revised proposal of 6G SID for SA6#69 meeting.

The following are moderator proposals on the work areas, moderated discussion questions and work plan.

# 2.	Work Area Descriptions

It is proposed to endorse the following work area for moderated discussion.

NOTE: the work area description within each work area is just for companies to provide moderated input on. Whether each area needs to be studied or not will be determined based on the moderated discussion and the final scope of study.

/*************************** Start ***************************/

## 1.0	System and Operational Aspects

Moderator proposal:

| Work Area Description | System and Operational Aspects facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to application enablement e.g. support of legacy services on 6G, 6G enhancements of legacy/existing services and capabilities. |
| --- | --- |
| Work Tasks | - 6G Enabler integrated within 3GPP System for better positioning in 6G Landscape - Intent driven capability exposure - Data, Location, Sensing, AI as new capabilities for exposure. - Secure, Privacy and Regulatory compliant management and exposure - Local exposure​​, ​​user-plane exposure​​, ​​modular or nested exposure​​, and ​​multi-party collaborative exposure - 3GPP wide API framework including usage by CN, OAM and RAN groups - Framework to support wider ecosystem players (e.g. Aggregators, IT players, edge/cloud providers, verticals) - Framework to support latest API paradigms (AI-powered, Streaming services, Realtimeness, Handling large data etc) |

## 1.1	AI

Moderator proposal:

| Work Area Description | In the context of AI-driven 6G systems, two concepts will be important for shaping the future systems: AI for 6G System and 6G System for AI. |
| --- | --- |
| Work Tasks | - Explore the Generative AI capabilities and its applications in 6G architecture - Handling of scarce training data for analytics services, using synthetic data - Handling LLMs and large data - AI as a service in Agentic AI era |

## 1.2	Integrated Sensing and Communication

Moderator proposal:

| Work Area Description | Integrated Sensing and Communication facilitates new applications and services that require sensing capabilities. |
| --- | --- |
| Work Tasks | TBD |

## 1.3	Ubiquitous Connectivity

Moderator proposal:

| Work Area Description | Ubiquitous Connectivity is intended to enhance connectivity to bridge the digital divide and enhance user experience, to address presently uncovered or scarcely covered areas. |
| --- | --- |
| Work Tasks | TBD |

## 1.4	Immersive Communication

Moderator proposal:

| Work Area Description | Immersive Communication requires support for mixed traffic of video, audio, haptic and other environment data in a reliable and synchronous manner, combining low latency and high data rates. |
| --- | --- |
| Work Tasks | - Avatars in Real-Time Communication Services - Enhancements to Metaverse and MMTel - To identify what needs to be specified beyond the support enabled by SA2 |

## 1.5	Massive Communication

Moderator proposal:

| Work Area Description | Massive Communication: involves connection of massive number of devices or sensors for a wide range of use cases and applications. |
| --- | --- |
| Work Tasks | - New compute plane tailored to ASP/vertical use cases, while considering the network situation - Compute handling on UE and offloading outside UE - Quantifying compute requirements and discovering nodes for handling - Compute co-ordination with CN - Network resources vs compute resources at edge/cloud |

## 1.6	Specific Industry and Verticals

Moderator proposal:

| Work Area Description | Specific Industry and Verticals: various capabilities and use cases needed to support the specific needs of different vertical markets |
| --- | --- |
| Work Tasks | - Digital twins for Factories, Network - Application Digital Twins different from Network Digital Twins - Enhancements to Metaverse, AIML and Sensing - Enabler Layer supported Service Digital Twins to enable rapid service development, testing, optimization, and deployment of services |

## 1.7	Other Considerations

Moderator proposal:

| Work Area Description | Other Considerations (e.g. inputs directly to stage-2, inputs based on industry initiatives) |
| --- | --- |
| Work Tasks | - Holistic 6G Enabler that is simple to use by Verticals - Blockchain/DLT enablement to perform automated transactions in the future without much human intervention e.g. in self-driving cars - Distributed Network operates independently from the PLMN of the same network operator to provide local services - RAN capability exposure via CAPIF enhancements - Energy driven enablement features - Harmonized 3GPP wide User Consent Framework |

# 3.	Moderated discussion proposed questions

It is proposed to endorse the following questions for each work area and work tasks for moderated discussion.

Q1: Do you support this work area to be included for the study? Answer with good justification.

Q2: What dependency does this work area have? (with other work areas and/or with other WGs)

Q3: Which aspects of the work area do you support for the study? Answer with good justification.

Q4: Which of the proposed aspects do you not support for the study? Answer with good justification.

Q5: What objectives do you suggest for this work area? The answer is expected to be on a level which allows to formulate the objectives of the SID.

# 4.	Work plan Proposal

It is proposed to endorse the following work plan:

|  | SA6#68 | Moderated Discussion#1 | SA6#69 | Moderated Discussion#2 | SA6#70 | Post SA6#70 untill end of Study |
| --- | --- | --- | --- | --- | --- | --- |
| Input | 1. Company proposed work tasks and initial analysis SA1 TR 22.870. 2. Discussion on Working methods (NWM, email, conf calls) for Moderated discussions. 3. Discuss the questions for Moderated Discussions | Company proposed Work Tasks | 1. Moderator collated Work Tasks 2. Company proposed Work Tasks | Company proposed Work Tasks | 1. Moderator collates Work Tasks 2. Company proposed Work Tasks | Company proposed key issues, solutions etc. to the agreed Work-Tasks |
| Output | 1. Populate SID with agreed Work Tasks 2. Finalize tool for Moderated Discussions. 3. Agree to questions for Moderated Discussions | Moderator collates Work Tasks and prepares discussion summary | Intermediate update SID based on the agreed Work Tasks (Round 1) | Moderator collates Work Tasks and prepares discussion summary | Finalize Work Tasks (Round 2). Finalize SID considering agreed Work Tasks and submit for SA plenary approval | Finalize TR with relevant Key Issues, Solutions and Conclusions for all Work Tasks by end of release. The conclusions of this study will form the basis for normative 6G application enablement work. |