3GPP TSG-SA WG6 Meeting #68	S6-253357

Gothenburg, Sweden 25th – 29th August 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new KI on individual managing of SEALDD-S and SEALDD-UU

Spec:	3GPP TR 23.700-52 v0.2.0

Agenda item:	9.7

Document for:	Approval

Contact:	changlinhui@huawei.com

1. Introduction

Add new KI on individual managing of SEALDD-S and SEALDD-UU.

2. Reason for Change

As clarified in discussion slides of S6-253309.

On the one hand, there are some misalignments within the SEALDD architecture. For instance, the VAL server in SEALDD policy configuration is not the same as the one interacting with the VAL client—this is explicitly stated in Clause 9.10.1: "In clause 9.10, the VAL server is a specific server for configurating SEALDD policy, and is different from the VAL server used for VAL application processing in other clauses." However, several descriptions of the SEALDD architecture in the specification are not fully consistent with this concept, including but not limited to:

- The architecture in clause 7.2 appears to indicate that the VAL server must have both VAL-UU and SEALDD-S;

- The VAL server description is missing in clause 7.3;

- The SEALDD-S reference point description in clause 7.4.4 seems indicate the control plane and the user plane are terminated at the same VAL server.

On the other hand, the VAL server may not require the VAL-UU connection during data transport as illustrated in figure 2-1. However, such descriptions are absent from the current specification. In certain existing procedures, the SEALDD-S connection is always created for certain VAL UE/VAL UE group, this is depicted in figure 2-2.

Figure 2-1: Illustrative Decoupling of SEALDD-S and SEALDD-UU

Figure 2-2:  The issue of coupling between SEALDD-UU and SEALDD-S

So, the support on managing SEALDD-S and SEALDD-UU individually need to be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v0.2.0.

* * * First Change * * * *

5.x	Key issue #x: Individual managing of SEALDD-S and SEALDD-UU

5.x.1	Description

The VAL server dedicated to configuring SEALDD policies may be distinct from the one used for VAL application processing. However, in the current specification, the VAL server interacting with the SEALDD server (via SEALDD-S) is required to be the same as the one communicating with the VAL UE. Furthermore, the SEALDD-S is always created for certain VAL UE/VAL UE group in some procedures.

In summary, whether and how to manage SEALDD-S and SEALDD-UU individually need to be studied.

5.x.2	Open issues

- Whether and how to manage SEALDD-S and SEALDD-UU individually rather than in a conjunction way.

* * * End of Change * * * *