3GPP TSG-SA WG6 Meeting #68	S6-253368

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	KPN N.V

Title:	Solution for Key Issue #7: VFL Sample Alignment between VAL Servers

Spec:	3GPP TR 23.700-83 0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution introduces a VFL sample alignment solution when VAL servers who are involved in VFL process as VFL participants need to perform sample alignment either for training or inference purposes.

2. Reason for Change

To achieve VFL process, VAL servers which are VFL participants require sample space alignment where the samples are UEs identified by local identifiers in each VAL server (for instance, AF specific UE identifiers and IP addresses). Sample space alignment allows matching corrsponding samples in a sample space in privacy-preseving maner. As identified by Key Issue # 3 the AIMLE server which has the ability to know the identity of UEs, can play a crucial role in sample alignment wihout revealing the identity of the UEs. This contribution proposes proposes to enhance the AIMLE Server to support sample alignment among VAL server VFL particpants.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V0.2.0.

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Vertical Federated Learning (VFL): A federated learning technique in which multiple VFL participants jointly perform training and inference on a common sample space with different feature sets, without exchanging raw data, thereby preserving their local data privacy.

VFL process: A process of performing training or inference using the VFL technique.

VFL participant: An entity that takes part in a VFL process, either as a VFL server or a VFL client.

VFL Server: A VFL participant that coordinates the VFL process by aggregating results from VFL clients and propagating updates to them during training.

VFL client: A VFL participant that performs local training and inference, sends results to the VFL server, and updates local parameters based on information received from the VFL server.

VFL Sample alignment: A stage in a VFL process during which samples are aligned among VFL participants, ensuring that training and inference are performed consistently across participants on the same sample space.

* * * Second Change * * * *

## 7.x Solution #x: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

### 7.x.1 Solution Description

#### 7.x.1.1 General

This solution addresses key issue # 7 which identifies VFL sample alignment among open issues. In a VFL process, a VAL server may act as a VFL server (active party) coordinating training and inference by aggregating results from other VAL servers acting as VFL clients (passive parties). In this context, the sample space consists of user equipment (UEs), each represented by local identifiers specific to the VAL servers.

The VFL process begins by first aligning the samples among VFL participants so that inference and trainingare performed properly. This is performed without sharing the UE identifiers with other VFL participants. Each UE in in the sample space may be identified with different identifications in each of the VFL participants such as different AF specific UE IDs or different public IP address of the UE. Therefore, the process of VFL sample alignment should be able to align the UEs that are involved in a VFL process without sharing the UE identifiers while resolving the potential difference in how the VFL participants identify the UEs.

This solution proposes to enable the AIMLE server to support sample alignment among VFL participants while preserving the privacy of the UEs and while resolving the potential differences in how each VFL participant identifies the UEs. To enable this the following procedures are defined:

- VFL participant registration: a procedure allowing VFL participants (VAL servers or AIMLE servers) to register with the AIMLE server, providing metadata including supported feature sets, operational modes (passive, iteration, and mediation modes), and local UE identifiers (e.g., AF-specific UE IDs or IP addresses). The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (such as MSISDN) via interaction with the Core Network.

- VFL client discovery: a procedure which is used by the VFL server to discover VFL participants for a VFL process having the desired sample space and feature set. The AIMLE server discovers VFL clients that match the desired feature set and sample space using previously mapped UE identifiers, ensuring privacy preservation.

- VFL client selection: a procedure which is used by the VFL server to select the desired VFL participants which may also can trigger assignment of sample correlation IDs associated to each UE ID in the sample space if the selected mode of operation by the VFL server is passive mode.

- VFL training request: a procedure which is used by the VFL server to initiate the VFL training process by instructing the VFL clients to perform training on a specified set of samples (i.e. UEs) and send the results to the VFL server which aggregates the results and determines the parameter updates to improve the models in each VFL participants. The update is propagated towards each VFL client so that the VFL clients can update their model locally.

- VFL inference request: a procedure which is used by the VFL server to initiate the VFL inference process by instructing the VFL clients to perform inference on a specified set of samples and send the result to the VFL server which aggregate the results and produce the inference result.

The AIMLE server can enable different modes of operation for the inference and training depending on the modes of operation supported by the VFL participants and the mode of operation selected by the VFL server during the VFL client selection procedure. These modes of operation are:

- Passive mode: In this mode of operation, the AIMLE server, during the training and inference request, sends to each VFL participant the correlation ID for each UE in the sample space which can be used to refer to the sample UE in a sample space during training and inference. The VFL server can use this correlation ID to indicate to each selected VFL client in each iteration to align the corresponding samples. This mode is passive because the AIMLE server is not involved in the actual training and inference process once it provided the correlation IDs in the sample space for each VFL participant.

- Iteration mode: In this mode of operation the AIMLE server is involved in the actual VFL process by receiving an iteration request from the VFL server which include the samples (i.e. UEs) in the current VFL process iteration and then sending an initiation request to the VFL clients with the samples (i.e. UE) to perform an inference or training on. Once the iteration is complete, the VFL server continues with the next iterations of the VFL process until all iterations are completed. This mode is iteration mode because the AIMLE server enables the sample alignment at each iteration of the VFL process.

- Mediation mode: This mode of operation is similar to the iteration mode, but instead of the VFL clients sending the results to the VFL server, each iteration is mediated by the AIMLE server which receives the results from the VFL clients and sends it to the VFL server. During training, it also receives updates from the VFL server and propagates to the VFL clients.

#### 7.x.1.2 Procedure

Figure 7.x.1.2-1: VFL process sample alignment procedure

Editor’s Note: Further details of the registration, discovery, selection, training, and inference procedures are FFS.

7.x.2 Architecture Impacts

There is no architectural impact from this solution.

7.x.3 Corresponding APIs

Editor’s Note: The corresponding APIs are FFS.

7.x.4 Solution Evaluation

Editor’s Note: The evaluation of the solution is FFS.

* * * End of Change * * * *