3GPP TSG-SA WG6 Meeting #68	S6-253402

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253012)

Source:	Motorola Solutions

Title:	Solution for Supporting Non session based MCDATA Recording.

Spec:	3GPP TR 23.700-39 V0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Adinarayana Setty (Adinarayana.setty@motorolasolutions.com)

Harish N (harish.negalaguli@motorolasolutions.com)

1. Introduction

This contribution proposes solution for key issue X for supporting Non session based MCData recording.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.2.0.

* * * First Change * * * *

## 6.x	Solution X: Non session based MCData recording support

### 6.x.1	Description

The 3GPP standard technical specifications TS 23.280 [2], TS 23.379 [3], TS 23.281[4], and TS 23.282[5] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCData), Mission Critical Video (MCVideo), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means, capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure in section 6.x.1.3 details solution for supporting non session based MCData recording.

#### 6.x.1.1	Functional model and reference points

As described in Key issue X and scenario 4.1.x, existing REC-4 interface as defined in 3GPP TS 23.280 [2], clause 7.5.2.34 shall be extended to support Non session based MCData recording.

#### 6.x.1.2	Configurations

None.

#### 6.x.1.3	Procedures

The 3GPP specifications TS 23.282 [4] and TS 24.282 [5] outline the architecture, flows, and procedures for supporting MCData communication. TS 23.282 [4] clause 7.4, outlines procedures for supporting MCData SDS, TS 23.282 [4] clause 7.5 outlines procedures for supporting MCData FD, TS 23.282 [4] clause 7.9 outlines procedure for supporting Enhanced status messaging.

The following procedure outlined in diagram 6.x.1.3-1 proposes Non session based MCData recording support flow.

Pre-conditions:

1.	MCData users involved in communication are already registered with MCData servers for receiving MCData service.

2.	MCData payload data size is below the configured maximum payload data size for providing MCData service over signalling control plane.

3.	All MCData clients involved in the communication are in the same MCData system.

4.	All MCData Groups are hosted in the same MCData system.

Figure 6.x.1.3-1: MCData recording flow.

1.	 MCData service procedures are invoked between set of MCData users.

2.	Upon the completion of MCData service procedures between group of MCDATA users or Private users, MCDATA server or MCData Message Store will initiate the transfer of MCDATA conversation information to the recording server. This transfer SHALL include metadata, MCData messaging content, optionally will include file attachments (if any) etc.

NOTE: Procedure used for transferring recording content from MCData server or MCData Message Store server will be FFS.

3.	On the receipt of MCData specific recorded information, Recording server shall push this information to Recording mass storage server (for later retrieval).

4.	As per the procedures defined in TS 23.280 [2] clause 10.18.4.2, MC Replay UE shall fetch and replay MCData recordings from Recording server.

### 6.x.2	Impacts on existing functional entities and reference points

None

### 6.x.3	Solution evaluation

TBD.

* * * End of Changes * * * *