3GPP TSG-SA WG6 Meeting #68	S6-253403

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253013)

Source:	Motorola Solutions

Title:	Scenarios for Non session-based Recording support for MCPTT, MCVIDEO messages.

Spec:	3GPP TR 23.700-39 V0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Adinarayana Setty (Adinarayana.setty@motorolasolutions.com)

Harish N (harish.negalaguli@motorolasolutions.com)

1. Introduction

This contribution proposes scenarios for supporting Non session based recording support for MCPTT, MCVideo messages.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.2.0.

* * * First Change * * * *

# 2	References

[10]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control.”

[11]	TS 24.281: “Mission Critical Video (MCVideo) signalling control.”

* * * Next Change * * * *

### 4.1.x Scenario X1: Recording support for Non session based MCPTT messages.

Figure 4.1.x-1: Functional entities for supporting recording of Non-session based MCPTT messages.

Procedure to support Non session based MCPTT messages such as User authentication and authorization for MCPTT services, Affiliation and de-affiliation to or from MCPTT group(s), MCPTT group selection, Location information (on-network), Subscription and notification for functional alias are described in 3GPP TS 23.379 [3] clause 10.2, 3GPP TS 23.379 [3] clause 10.3, 3GPP TS 23.379 [3] clause 10.4, 3GPP TS 23.379 [3] clause 10.13, 3GPP TS 23.379 [3] clause 10.18 respectively.

This scenario describes the recording support for Non session based MCPTT service between MC users.

- MC user A, MC user B (MCPTT ID A, MCPTT ID B) has/have been set as the target for the recording by a Recording admin user.

- The procedure described in 3GPP TS 23.280 [2], clause 10.18.2.2 will be used to update target user’s MCPTT profile in the CMS to support recording.

NOTE 1:	Parameters required for supporting recording in MCPTT user profile is FFS.

- Target MC users can be part of Non-session based MCPTT Services such as, MC user authentication and authorization for MCPTT services, Affiliation and De-affiliation to/from MCPTT Groups, Subscription and for functional alias etc.

- When recording target MC users are involved in any Non session based MCPTT Service communication, such  MCPTT Service communications needs to be recorded.

NOTE 2:	Above provided Non session based MCPTT services is not an exhaustive list, but it should be noted that, this scenario is applicable for all Non session based MCPTT services recording.

### 4.1.y Scenario X2: Recording support for Non session based MCVideo messages.

Figure 4.1.y-1: Functional entities for supporting recording of Non-Session MCVideo messages.

Procedure to support Non session based MCVideo messages such as Affiliation and de-affiliation to/from MCVideo group(s), Functional alias management, User authentication and authorization for MCVideo service, Location information (on-network) are described in 3GPP TS 23.281[4] clause 7.9, 3GPP TS 23.281[4] clause 7.9a, 3GPP TS 23.281[4] clause 7.12, 3GPP TS 23.281[4] clause 7.14.

This scenario describes the recording support for Non session based MCVideo Service messages between MC user A, MC user B.

- MC user A, MC user B (MCVideo ID A, MCVideo ID B) has/have been set as the target for recording by a Recording admin user.

- The procedure described in 3GPP TS 23.280 [2], clause 10.18.2.2 will be used to update target user’s MCVideo profile in the CMS to support recording.

NOTE 3:	Parameters required for supporting recording in MCVideo user profile is FFS.

- Target MC users can be part of Non-session-based MCVideo Services such as Affiliation and de-affiliation to/from MCVideo group(s), Functional alias management, User authentication and authorization for MCVideo service, Location information (on-network) etc.

- When recording target MC users are involved in any Non session based MCVideo Service communication, such MCVideo service communications needs to be recorded.

NOTE 4:	Above provided Non session based MCVideo services is not an exhaustive list, but it should be noted that, this scenario is applicable for all Non session based MCVideo services recording.

* * * End of Changes * * * *