3GPP TSG-SA WG6 Meeting #68	S6-253405

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253013)

Source:	Motorola Solutions

Title:	Solution for supporting Non session-based Recording for MCPTT, MCVideo messages.

Spec:	3GPP TR 23.700-39 V0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Adinarayana Setty (Adinarayana.setty@motorolasolutions.com)

Harish N (harish.negalaguli@motorolasolutions.com)

1. Introduction

This contribution proposes solution for key issue X1, X2 to support Non session based recording for MCPTT, MCVideo messages.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.2.0.

* * * First Change * * * *

# 2	References

[10]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control.”

[11]	TS 24.281: “Mission Critical Video (MCVideo) signalling control.”

* * * Next Change * * * *

## 6.x	Solution X1: Non session based MCPTT recording support

### 6.x.1	Description

The 3GPP standard technical specifications TS 23.280 [2], TS 23.379 [3], TS 23.281[4], and TS 23.282[5] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCData), Mission Critical Video (MCVideo), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure 6.x.1.3 lists non session based recording procedures for MCPTT Service.

#### 6.x.1.1	Functional model and reference points

As described in key issue X1, mentioned in scenario X1 clause 4.1.x existing REC-4 interface as defined in 3GPP TS 23.280 [2], clause 7.5.2.34 shall be extended to support Non session based MCPTT recording.

#### 6.x.1.2	Configurations

None.

#### 6.x.1.3	Procedures

The 3GPP specifications TS 23.379 [3] and TS 24.379 [10] outline the architecture, flows, and procedures for supporting MCPTT communication.

TS 23.379 [3] clause 10.6, TS 23.379 [3] clause 10.7, TS 23.379 [3] clause 10.14, TS 23.379 [3] clause 10.19 outlines procedures for supporting MCPTT Group call, MCPTT Private call, Ambient listening call, MCPTT Ad hoc group call respectively.

The following procedure outlined in diagram 6.x.1.3-1 proposes non session based MCPTT recording flow.

Pre-conditions:

1.	MCPTT users involved in communication are already registered for receiving MCPTT service.

2.	All MCPTT clients involved in the communication are in the same MCPTT system.

3.	All MCPTT Groups are hosted in the same MCPTT system.

Figure 6.x.1.3-1: MCPTT recording flow.

1.	 Non session based MCPTT service procedures are invoked between set of MCPTT users.

2.	Upon the completion of Non session based MCPTT service procedures between group of MCPTT users, MCPTT server will initiate the transfer of MCPTT Non session-based messaging information to the recording server. This transfer SHALL include Non session based MCPTT Messaging content, metadata etc.

NOTE: Procedure used for transferring recording content from MCPTT server to Recording server will be FFS.

3.	On the receipt of Non session based MCPTT recording information, recording server shall push this information to Recording mass storage server (for later retrieval).

4.	As per the procedures defined in TS 23.280 [2] clause 10.18.4.2, MC Replay UE shall fetch and replay MCPTT recordings from recording server.

### 6.x.2	Impacts on existing functional entities and reference points

None

### 6.x.3	Solution evaluation

TBD.

## 6.y	Solution X2: Non session based MCVideo recording support

### 6.y.1	Description

The 3GPP standard technical specifications TS 23.280 [2], TS 23.379 [3], TS 23.281[4], and TS 23.282[5] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCData), Mission Critical Video (MCVideo), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure 6.y.1.3 lists non session based recording procedures for MCVideo Service.

#### 6.y.1.1	Functional model and reference points

As described in key issue X2 and mentioned in scenario X2 clause 4.1.y, existing REC-4 interface as defined in 3GPP TS 23.280 [2], clause 7.5.2.34 shall be extended to support Non session based MCVideo recording.

#### 6.y.1.2	Configurations

None.

#### 6.y.1.3	Procedures

The 3GPP specifications TS 23.281 [4] and TS 24.281 [11] outline the architecture, flows, and procedures for supporting MCVideo communication.

TS 23.281 [4] clause 7.1, TS 23.281 [4] clause 7.2, TS 23.281 [4] clause 7.3, TS 23.281 [4] clause 7.4, TS 23.281 [4] clause 7.19 outlines procedures for supporting MCVideo Group call, MCVideo Private call, MCVideo Pull, MCVideo Push, MCPTT Ad hoc group call respectively.

The following procedure outlined in diagram 6.y.1.3-1 proposes non session based MCVideo recording flow.

Pre-conditions:

1.	MCVideo users involved in communication are already registered for receiving MCVideo service.

2.	All MCVideo clients involved in the communication are in the same MCVideo system.

3.	All MCVideo Groups are hosted in the same MCVideo system.

Figure 6.x.1.3-1: MCVideo recording flow.

1.	 Non session based MCVideo service procedures are invoked between set of MCVideo users.

2.	Upon the completion of Non session based MCVideo service procedures between group of MCVideo users, MCVideo server will initiate the transfer of MCVideo Non session based messaging information to the recording server. This transfer SHALL include Non session based MCVideo Messaging content, Metadata etc.

NOTE: Procedure used for transferring recording content from MCVideo server to Recording server will be FFS.

3.	On the receipt of Non session based MCVideo recording information, recording server shall push this information to Recording mass storage server (for later retrieval).

4.	As per the procedures defined in TS 23.280 [2] clause 10.18.4.2, MC Replay UE shall fetch and replay MCVideo recordings from recording server.

### 6.x.2	Impacts on existing functional entities and reference points

None

### 6.x.3	Solution evaluation

TBD.

* * * End of Changes * * * *