3GPP TSG-SA WG6 Meeting #68	S6-253467

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253153)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0477 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the modify ad hoc group call criteria procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections are needed to clarify the “Modify ad hoc group call criteria” procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Make corrections to the relevant information flows to clarify what IEs are for and included in procedure. - Align the affected procedures with the corrected infromation flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.10, 10.19.2.20, 10.19.2.20a, 10.19.2.21, 10.19.2.21a, 10.19.3.1.6, 10.19.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.19.2.10	Ad hoc group call notify (MCPTT server – MCPTT client)

Table 10.19.2.10-1 describes the information flow ad hoc group call notify from MCPTT server to MCPTT client.

Table 10.19.2.10-1: Ad hoc group call notify

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of ad hoc group call participant |
| Functional alias | O | The associated functional alias of the MCPTT user of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| MCPTT ID list(s) | O | The list of the invited MCPTT users who did not acknowledge the ad hoc group call request within a configured time or the list of the invited MCPTT users who acknowledged the ad hoc group call request and joined or the list of the MCPTT users who joined or left the ongoing MCPTT ad hoc group call. |
| Call resulting criteria for determining the participants (NOTE) | O | Call resulting criteria used by the MCPTT server to determine the list of participants to be invited. |
| NOTE: This information Element shall only be included for the authorized MCPTT user or the ad hoc group call initiator. It shall be included if the MCPTT server modified the criteria in the modify ad hoc group call request message. |  |  |

# * * * * Next change * * * *

#### 10.19.2.20	Modify ad hoc group call criteria request (MCPTT client – MCPTT server)

Table 10.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCPTT client to the MCPTT server.

Table 10.19.2.20-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | M | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area. |

#### 10.19.2.20a	Modify ad hoc group call criteria request (MCPTT server – MCPTT server)

Table 10.19.2.20a-1 describes the information flow Modify ad hoc group call criteria request from the MCPTT server to the MCPTT server.

Table 10.19.2.20a-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | M | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area or the modified criteria by the primary MCPTT server. |

#### 10.19.2.21	Modify ad hoc group call criteria response (MCPTT server – MCPTT client)

Table 10.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCPTT server to the MCPTT client.

Table 10.19.2.21-1: Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria |
| Functional alias | O | The functional alias of the requesting MCPTT user who is authorized to modify the ad hoc group call criteria |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call criteria request (success or failure) |
| Call resulting criteria for determining the participants (NOTE) | O | Contain the call resulting criteria used by the MCPTT server to determine the list of participants to be invited |
| NOTE: This Information Element shall only be included if the Result is success. |  |  |

#### 10.19.2.21a	Modify ad hoc group call criteria response (MCPTT server – MCPTT server)

Table 10.19.2.21a-1 describes the information flow Modify ad hoc group call criteria response from the MCPTT server to the MCPTT server.

Table 10.19.2.21a-1: Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call criteria request (success or failure) |

# * * * * Next change * * * *

##### 10.19.3.1.6	Modification of ad hoc group call criteria by an authorized user

Figure 10.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure by an authorized user.

Pre-conditions:

1.	The MCPTT user at the MCPTT client 1 is authorized to modify the criteria.

2.	Both the MCPTT server and the MCPTT client 1 are aware of the criteria related to the ongoing ad hoc group call.

Figure 10.19.3.1.6-1: Modification of ad hoc group call criteria by the authorized user

1.	The MCPTT user at the MCPTT client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCPTT client 1 sends the modify ad hoc group call criteria request to the MCPTT server which contains an updated criteria to determine the list of participants.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group voice communication. The MCPTT server determines the list of ad hoc group call participants based on the criteria provided and the added/removed participant by the authorized user as marked in clause 10.19.3.1.4.

If MCPTT client 3 satisfies the new criteria to be removed and

-	has been already added by an authorized user, the MCPTT server does not remove it from the call and skips the step 4 to step 6.

-	has not been already added by an authorized user, the procedure continues with step 4.

If MCPTT client 2 satisfies the new criteria to be added and

-	has been previously removed by an authorized user, the MCPTT server does not add it back to the call and skips the step 7 to step 9.

-	has not been already removed by an authorized user, the procedure continues with step 4.

Depending on the criteria provided and based on local policy, the MCPTT server may modify the content of the criteria received in step 1 to determine the list of participants.

3.	The MCPTT server sends modify ad hoc group call criteria response containing the call resulting criteria used by the MCPTT server to determine the list of participants to be invited to MCPTT client 1.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server.

7.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE 1:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

9.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server. The MCPTT server considers the MCPTT user as implicitly affiliated to the ad hoc group.

10.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to or removed from the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCPTT server to determine the list of participants to be invited.

11.	The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call.

The MCPTT server continuously checks whether other MCPTT clients meet or if participating MCPTT clients no longer meet the criteria for the ad hoc group emergency call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * Next change * * * *

##### 10.19.3.2.7	Modification of ad hoc group call criteria by an authorized user

Figure 10.19.3.2.7-1 illustrates the modification of ad hoc group call criteria procedure by the initiator of the ad hoc group call between multiple MC systems.

Preconditions:

-	The MCPTT user at the MCPTT client 1 is authorized to modify the criteria.

-	The MCPTT server A and MCPTT server B are aware of the criteria related to the ongoing ad hoc group call.

-	MC system A and MC system B are interconnected.

Figure 10.19.3.2.7-1: Modifying the criteria for determining the participants during an ongoing ad hoc group call between multiple MC systems

1.	An ad hoc group call has been established based on criteria sent by an authorized MCPTT client upon initiating the ad hoc group call.

2.	The MCPTT user at the MCPTT client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group call participants.

3a.	MCPTT client 1 sends a modify ad hoc group call criteria request to MCPTT server A.

3b.	MCPTT server A determines that the modify ad hoc group call criteria request received in step 3a has impact on users in MCPTT server B. Depending on the criteria provided and based on local policy, the MCPTT server A may modify the content of the criteria received in step 3a to determine the list of participants.

MCPTT server A sends a modify ad hoc group call criteria request to MCPTT server B containing the resulting criteria to be used by MCPTT server B.

NOTE 1:	If MCPTT server A identifies that the criteria require to add participants from another MC system then MCPTT server A sends an ad hoc group call request to the MCPTT server in that MC system.

4a/b.	The MCPTT server A receives the modify ad hoc group call criteria response and forwards the response to MCPTT client 1 in MC system A.

5.	MCPTT server B detects that MCPTT client 3 meets the criteria for the ongoing ad hoc group call initiated at MC system A.

5a.	MCPTT server B sends an ad hoc group call add user notification message towards MCPTT server A.

5b.	MCPTT server A sends an ad hoc group call request towards MCPTT client 3.

5c.	The MCPTT user 3 is notified of entering an ongoing ad hoc group call.

5d.	MCPTT client 3 sends an ad hoc group call response towards MCPTT server A.

5e.	MCPTT server B sends an ad hoc group call remove user notification message towards MCPTT server A.

5f.	MCPTT server A sends an ad hoc group call leave request towards MCPTT client 4.

5g.	The MCPTT user 4 is notified of leaving an ongoing ad hoc group call.

5h.	MCPTT client 4 sends an ad hoc group call leave response towards MCPTT server A.

6.	The MCPTT server A may notify the authorised MCPTT user of all the users who are added to or removed from the on-going ad hoc group call. This notification may be sent to the authorised MCPTT user by the MCPTT server A more than once during the call when MCPTT users join or leave the ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCPTT servers to determine the list of participants to be invited.

7.	The MCPTT server A adds MCPTT client 3 to the ad hoc group call and removes MCPTT client 4 from the ad hoc group call.

The MCPTT servers continuously check whether other MCPTT clients meet or if participating MCPTT clients no longer meet the criteria for the ad hoc group call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of the ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * Next change * * * *