3GPP TSG-SA WG6 Meeting #68	S6-253513

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253226)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0316 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on granularity of service authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 6.12 of TS 33.122 Specifies Authorization for finer level service API access, to limit access to specific services, service operations and/or resources. Therefore, the dependency with SA3 for the details of finer level service API access in the existing ENs in TS 23.222 can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.31.3, remove the dependency with SA3 for the detailed specification of finer level of serice API access. Annex E, remove the EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Finer level of Service API access is not solved in TS 23.222. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31.3, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function (ROF) can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner (RO).

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain authorization information to invoke the service API exposed by the API exposing function (AEF) and to access information owned by the resource owner at the AEF through the invocation of an obtain service API authorization request to the CCF. The request contains API invoker information required for authorization, the information identifying the service API, the purpose for data processing and any information required for authentication of the API invoker. The request may include finer level service API access requirements (e.g., access per service API operation or access per API service resource) and resource owner-related information required to obtain resource owner authorization information.

NOTE 1:	The details of the information provided by the API invoker for obtaining service API access are specified in 3GPP TS 33.122 [12].

2.	The CCF performs the authentication of the API invoker (using authentication information). Then the Authorization function determines the authorization by checking the authorization information available in the CCF and by checking the authorization information provided by the RO via the ROF. The request to the ROF contains application service information (e.g. the application service provider and application identifier), the purpose for data processing and the resource owner data information for which the API invoker requests access grants.

NOTE 2:	The detailed procedure to obtain the RO's authorization information is specified in 3GPP TS 33.122 [12].

NOTE 3:	Step 1 can occur when the API invoker receives a failure in Service API invocation response indicating that authorization information from resource owner(s) is required.

3.	Based on the RO authorization information obtained via the ROF, the CCF sends an obtain service API authorization response to the API invoker.

4.	The API invoker sends service API invocation request to the API exposing function with the authorization information received from CCF (Authorization function) in step 3.

5.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information received from CCF (Authorization function).

* * * Second Change * * * *

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. |  |

* * * End of Changes * * * *