3GPP TSG-SA WG6 Meeting #68	S6-253518

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253232)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0322 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of authorization revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S3-252426 agreed by SA3 develops RO authorization revocation and includes information related to the revoked token in the Revoke_Authorization service operation of the CAPIF_Security API. The Revoke API invoker authorization notify needs to include the Access Token Identification. And these changes need to be brought to TS 23.222. In addition, by mistake, Rel19 added that the revoke API invoker authorization notify service operation can be sent by the CCF to the API Exposing Function. This mistake needs to be corrected. Other minor clarifications are added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 8.23.2.1 includes as parameter of the Revoke API invoker authorization request the Revoked auhorization data identification, applicable only to the revocation requests from the CCF to the AEF. - Clause 8.23.2.3 removes the revoke notification to the AEF and includes the revoke authorization data identification parameter as in 8.23.2.1. - Clause 8.23.4, procedure is updated to include the revoked authorization data identification information. - Interconnect revoke API invoker authorization, clauses 8.25.2.11, 8.25.2.13 includes IE about revoked authorization data identification. - 8.25.3.8 clarifies the triggers for interconnection revocation. - 8.35.3 corrects the Revoke resource authorization notify by the Revoke resource authorization request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Authorization Revocation procedure, missing finer level of access authorization included in the revoked authorization data identification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.23.2.1, 8.23.2.3, 8.23.4, 8.25.2.11, 8.25.2.13, 8.25.3.8, 8.35.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.23.2.1	Revoke API invoker authorization request

Table 8.23.2.1-1 describes the information flow revoke API invoker authorization request from the API exposing function to the CAPIF core function or from the CAPIF core function to the API exposing function.

Table 8.23.2.1-1: Revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification list | M | The identification information of the service API(s) for which the authorization is revoked. |
| Security information | O (NOTE 1,2) | Security information (as specified in 3GPP TS 33.122 [12]) for authorization revocation. |
| Cause | M | The cause for revoking the API invoker authorization (e.g., RO authorization revocation). |
| NOTE 1: The security information may be included in the Revoke API invoker authorization request from the CAPIF core function to the API exposing function (i.e., it is not used in the request from the API exposing function to the CAPIF core function). NOTE 2: If the security information is not included, the revocation applies to all the permissions granted for the service API. |  |  |

* * * Next Change * * * *

#### 8.23.2.3	Revoke API invoker authorization notify

Table 8.23.2.3-1 describes the information flow revoke API invoker authorization notify from the CAPIF core function to the API invoker.

Table 8.23.2.3-1: Revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker whose authorization has been revoked |
| Service API identification list | M | The identification information of the service API(s) for which the authorization is revoked. |
| Security information | O (NOTE) | Security information (as specified in 3GPP TS 33.122 [12]) for authorization revocation. |
| Cause | M | The cause for revoking the API invoker authorization (e.g., RO authorization revocation). |
| NOTE: If the security information is not included, the revocation applies to all the permissions granted for the service API(s). |  |  |

* * * Next Change * * * *

### 8.23.4	Procedure for CAPIF revoking API invoker authorization initiated by CAPIF core function

Figure 8.23.4-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the CAPIF core function. This procedure is also used for revoking API invoker authorization supporting RNAA scenarios.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 8.23.4-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

1.	The CAPIF core function is triggered to revoke the API invoker authorization.

2.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API, and optionally, the security information as specified in clause 6.5.3.4 of TS 33.122 [12].

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API or, if provided, the API invoker authorization corresponding to the received security information.

4.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API or, if provided, to the received security information.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * Next Change * * * *

#### 8.25.2.11	Interconnection revoke API invoker authorization request

Table 8.25.2.11-1 describes the information flow interconnection revoke API invoker authorization request from one CCF to another CCF.

Table 8.25.2.11-1: Interconnection revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the requesting CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification list | M | The identification information of the service API(s) for which the authorization is revoked. |
| Security information | O (NOTE 1,2) | Security information (as specified in 3GPP TS 33.122 [12]) for authorization revocation. |
| Cause | M | The cause for revoking the API invoker authorization |
| NOTE 1: The security information may be included in the Interconnect Revoke API invoker authorization request from the CAPIF core function to which the API invoker is onboarded to the CAPIF Core Function where the API exposing function is registered (i.e., it is not used in the request from the CAPIF Core Function where the API exposing function is registered). NOTE 2: If the security information is not included, the revocation applies to all the permissions granted for the service API. |  |  |

Editor's note:	The additional security aspects for the protection of the identity information of the API invoker are FFS in SA3.

* * * Next Change * * * *

#### 8.25.2.13	Interconnection revoke API invoker authorization notify

Table 8.25.2.13-1 describes the information flow interconnection revoke API invoker authorization notify from one CCF to another CCF.

Table 8.25.2.13-1: Interconnection revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the notifying CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification list | M | The identification information of the service API(s) for which the authorization is revoked. |
| Security information | O (NOTE) | Security information (as specified in 3GPP TS 33.122 [12]) for authorization revocation. |
| Cause | M | The cause for revoking the API invoker authorization |
| NOTE: If the security information is not included, the revocation applies to all the permissions granted for the service API. |  |  |

Editor's note:	The additional security aspects for the protection of the identity information of the API invoker are FFS in SA3.

* * * Next Change * * * *

#### 8.25.3.8	Procedure for CAPIF revoking API invoker authorization in CAPIF interconnection

Pre-conditions:

1.	The API invoker is authenticated by CCF-B and authorized by CCF-B and CCF-A to use the service API. The API invoker has obtained authorization to use the service API as specified in clause 8.25.3.7.

2.	The CCF-B may have available access control policies corresponding to the service API.

3.	The CCF-A is triggered to revoke API invoker authorization for service API access, e.g. AEF requested the revocation of API invoker authorization.

Figure 8.25.3.8-1: Procedure for revoking API invoker authorization in CAPIF interconnection

1.	If the CCF-B keeps service API authorization information for the API invoker, and the CCF-A is triggered to revoke API invoker authorization information for service API access, the CCF-A sends an interconnection revoke API invoker authorization request to the CCF-B with the details of the API invoker, the AEF and the service API.

2.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CCF-B invalidates the API invoker authorization corresponding to the service API.

3.	The CCF-B sends an interconnection revoke API invoker authorization response to the CCF-A. The CCF-B sends the Revoke API invoker authorization notify described in step 6.

4.	Instead of step 1 to 3, if the CCF-A keeps service API authorization information for the API invoker, and the authorization information is revoked, the CCF-A sends an interconnection revoke API invoker authorization notify to the CCF-B.

5.	The CCF-A invalidates the API invoker authorization corresponding to the service API.

6.	The CCF-B sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * Next Change * * * *

### 8.35.3	Procedure

Figure 8.35.3-1 illustrates the procedure for revoking resource owner authorization to access service API initiated by the resource owner function.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

Figure 8.35.3-1: Procedure for revoking resource owner authorization

1.	The RO triggers the revocation of the resource authorization via ROF.

2.	The ROF requests resource authorization revocation to the CAPIF core function.

NOTE:	The detailed procedure of resource authorization revocation is to be specified in TS 33.122 [12].

3.	The CAPIF core function determines the revoked authorization information for the API Invoker and Service API as specified in clause 6.5.3.4 of TS 33.122 [12].

4.	The CAPIF core function sends a revoke resource authorization notification to the API invoker according to the procedure in clause 8.23.4.

5.	The CAPIF core function sends a revoke resource authorization notification to the API exposing function  according to the procedure in clause 8.23.4.

* * * END of Change * * * *