3GPP TSG-SA WG6 Meeting #68	S6-253523

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253064)

Source:	InterDigital, China Mobile

Title:	New Key Issue on using sensing results for spatial maps

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Quang Ly <Quang.Ly@InterDigital.com>

1. Introduction

This paper proposes a new key issue for supporting the use of sensing results for spatial maps.

2. Reason for Change

The SA6 SID FS_Sensing_App (SP-250875) was approved for studying the use of sensing results for vertical applications. Objective a) provides two requirements for the study: analyze SA1 use cases associated with sensing and identify potential enhancements of enablement services by utilizing the sensing results.

a)	Analysis of SA1 use cases associated with Sensing and identify potential enhancements to SA6 application enablement services (e.g., 3GPP TS 23.255, 3GPP TS 23.286, 3GPP TS 23.437) by utilizing Sensing results;

In Rel-19, SA1 studied and defined use cases for use of 3GPP sensing technology. The use cases were described and captured in 3GPP TR 22.837. Specifically, the use cases described in clause 5.19 and 5.27 mentioned the use of sensing data for metaverse applications. In 3GPP TR 22.856, the SA1 FS_Metavese study captured use of sensing data in various metaverse use cases described in clauses 5.2, 5.5, 5.6, etc.

In Rel-19, SA1 described in 3GPP TS 22.137 the capabilities of a 5GS to provide the ability to identify and track objects (e.g. UAV, human, vehicle or animal). Service requirements were agreed to allow for the reporting of sensing results to a trusted third-party requesting information about a target object. The sensing results exposed to a third party may be derived from a joint processing of 3GPP and non-3GPP sensing data.

In Rel-19, SA6 defined procedures to create and update spatial maps in 3GPP TS 23.437 for metaverse applications. There is a note in the Create Spatial Map Request procedure described in clause 9.3.1 that specified processed sensor data is obtained from and stored at the VAL layer (note is copied below for reference).

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope.

It is proposed to use sensing results to enhance the capability of spatial map service defined in 3GPP TS 23.437. The spatial map service can use the sensing results to determine and identify the presence of objects in a coverage area of a spatial map. The spatial map service may further use the sensing results to track movement of the object. Finally, the spatial map service may expose object information obtained from sensing results to third-party consumers.

4. Proposal

It is proposed to agree on this new key issue for 3GPP TR 23.700-15 v 0.0.0.

* * * * First Change * * * *

## 4.x	Key issue #x: Key issue on use of sensing results for spatial maps

### 4.x.1	Description

This KI aims to study how to use sensing results to manage object information in spatial maps. Objects in spatial maps can be stationary or mobile. As a result, tracking mobile objects within spatial maps can be more efficient with the use of sensing technology. A sensing service can be triggered to generate sensing results for an area of interest in a spatial map and the sensing results can be used to determine and identify objects within the spatial map. As an object moves, sensing results can be used to track the object and allow a spatial map server the ability to keep the spatial map current. The information obtained from sensing results for the spatial map can then be exposed to third-party consumers.

In Rel-19, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope. In Rel-20, exposure of sensing results (with or without the sensing contextual information) to support sensing services will be provided by 3GPP core network to enabler layer.

### 4.x.2	Open Issues

The key issue will study:

1. Whether and how the enabler layer can use sensing results for determining the presence of objects (e.g. humans, animals, and vehicles) within an area of interest in a spatial map.

2. Whether and how the enabler layer can associate a sensing result with an object within a spatial map.

3. Whether and how the enabler layer can localize objects and detect object movements within a spatial map from sensing results.

4. How the enabler layer can support SM server with creating and updating SM using sensing results.

* * * * End of Change * * * *