3GPP TSG-SA WG6 Meeting #68	S6-253612

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253287)

Source:	Ericsson

Title:	Enhancements to support flow synchronization for XR applications

Spec:	3GPP TR 23.700-51 V0.2.0

Agenda item:	9.5

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This paper introduces a solution for the KI#1 to enhance the SEALDD layer to provide  robust flow synchronization and QoS control for the XR application.

2. Reason for Change

The current procedure in clause 9.12.2.1.2 enables the VAL server to provide the QoS requirements to the SEALDD server for the XR application. Based on SEALDD-S 5-tuple information, it is not sufficient to understand the QoS requirements of the different flows.  Therefore, this paper proposes a type of the multimodal XR flow to identify the associated media types of the given XR application. The SEALDD server can derive the QoS requirements for the individual flows(e.g. audio, video, haptic) of the XR application based on the type and configure them in the network.

The multi-modal XR application has different types of flows like haptic, audio, video, and each flow requires different QoS. Moreover, the multi-modal application requires synchronization between flows as compared to other conventional traffic types like browsing or streaming, etc.  And if this synchronization requirements are not met by the network then it leads to negative impact on the QoS and QoE to end users, like drowsiness in VR headset, lag, etc.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V0.2.0

* * * First Change * * * *

##### 9.12.2.1.2	SEALDD enabled XR data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled XR data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, type of multi-modal flow, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for XR application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. If QoS information for each flow is not provided, then the SEALDD server derives the flow-specific QoS requirements based on the type of multi-modal flow(e.g., audio, video, haptic).

NOTE: The SEALDD server can use KPIs described in clause 7.11 TS 22.261[2] to derive the individual multi-modal flow QoS requirements.

2.	Same as step 2 of clause 9.2.2.2.  For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes XR transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates.

7-8.	Same as step 8-9 of clause 9.2.2.2.

NOTE :	If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1.

The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

* * * Next Change * * * *

#### 9.12.3.1	SEALDD enabled XR transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting XR transmission service.

Table 9.12.3.1-1: SEALDD enabled XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| > Type of multi-modal XR flow | O | Indicates the type of the flow of the multi-modal XR application. It includes audio, video, tactile or haptic |
| >QoS information of multi-modal XR flow | O | QoS information provided by VAL server for each specific flow(e.g. audio, video, haptic/tactile) |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |

* * * End of Changes * * * *