3GPP TSG-SA WG6 Meeting #68	S6-253652

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-253265

Source:	Ericsson

Title:	Updates to Solution#5 on Analytics Performance Information Collection

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

Solution #5 introduces the monitoring of ML-enabled analytics correctness. In this solution, the AIMLE server requests the ADAE the ML-enabled analytics performance to detect potential model performance degradation. This pCR proposes to update the solution #5 by including the option for the AIMLE server and/or ADAE server to collect analytics performance information via A-DCCF.

In addition to that, such degradation may be caused by different factors, such as data drift and it is essential that the ML-enabled analytics performance report considers the underlying reasons for this performance degradation.

2. Reason for Change

To improve solution #5 by including the option of the AIMLE server and/or ADAE server to collect analytics performance information via A-DCCF, and adding data drift as one of the reasons for ML model degradation.

3. Conclusions

This paper proposes to update the solution #5 on monitoring ML-enabled analytics correctness.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness

### 7.5.1	Solution description

This solution addresses Key Issue #2 and open issue 3.

In this solution, the ADAES is providing the analytics performance monitoring where the AIMLE server detects an expected or predicted model degradation performance based on the ML-enabled analytics performance feedback. Such ADAE analytics (as captured in 3GPP TS 23.436) can be about the edge load or VAL server or session performance analytics which utilizes ML methods (training by AIMLE) for predicting the end-to-end performance.

Pre-conditions:

1.	One or more ADAE services (at the ADAE server or clients) using the given ML model training service from AIMLE server are ongoing.

2.	The AIMLE server (or VAL server) detects a requirement to monitor the analytics performance. Such trigger may be based on VAL server request for the ML model performance monitoring capability as specified in 3GPP TS 23.482 clause 8.22.2.

Figure 7.5.1-1: Procedure for ML-enabled analytics monitoring

1.	The AIMLE server sends an ADAE analytics performance monitoring subscription request to the ADAE server (ADAES). The subscription request includes the following parameters: an ADAE analytics ID or type, a VAL server ID, an area of interest and time of validity, a monitoring reporting configuration (thresholds for triggering a monitoring event, e.g. minimum accuracy, confidence level, whether the reporting is one time or periodical or event-based), a KPI for the analytics service, VAL policies for triggering an action based on a monitoring event (e.g. if analytics degradation is detected, re-start the analytics process).

2.	The ADAE server authorizes the subscription request and sends an ADAE analytics performance monitoring subscription response to the AIMLE server. The response is a positive or negative acknowledgement of the subscription and includes a subscription ID in case of success and a cause in case of failure (e.g. failure to authorize).

3.	The ADAE server discovers based on the analytics ID the ADAECs or other entities (e.g. edge ADAES) which are undertaking an analytics task for the given analytics ID (e.g. local prediction at UE side or ADAES side).

4.	The ADAES server sends a request to the discovered entities (ADAEC or edge ADAES) indicating a monitoring requirement for the analytics task. This request includes the analytics ID, the requestor ID, the event trigger criteria for providing a monitoring report (e.g. reaching a threshold), the target KPIs to be monitored (e.g. accuracy), the area and time of interest, whether this is a predicted parameter or actual parameter to be monitored. Such request can be in form of an analytics correctness feedback request either real time or when the analytics task finishes.

5. The discovered ADAEC(s) and/or edge ADAES start monitoring the performance of the analytics task and in particular the attributes which are requested (accuracy, delay, evaluation vs ground truth data). This involves the continuous checking of the performance (accuracy, latency) of the analytics service for providing a notification / trigger event to the AIMLE server identifying a deviation in terms of ADAE analytics performance. Such identification can be a detection of a mismatch in the analytics performance with a possible cause (e.g. data producer, ML model, UE/ channel conditions, data drift).

6. The discovered ADAEC(s) and/or edge ADAES based on the trigger criteria, send to the ADAES a monitoring response, indicating an expected or predicted degradation of the performance of the analytics task. Such message may also include the possible cause as in step 6 (e.g. data producer, ML model, UE/ channel conditions, data drift).

7. The ADAES identifies or predicts an ADAE service degradation or performance (against the KPIs for that analytics ID) and sends a notification to the AIMLE server detecting a degradation / mismatch at the target performance for the analytics task.

8. The AIMLE server based on the received monitoring notification from the one or more AIMLE clients, it identifies an expected or predicted ML model degradation using the AIMLE capability for ML model performance monitoring as specified in 3GPP TS 23.482 clause 8.22.2 and can optionally use the ADAE capability for ML model degradation detection in clause 8.19.2.

Alternatively, the AIMLE server after step 2 and/or the ADAE server after step 3 send requests to A-DCCF for collecting analytics performance information. The procedure for ADAE Server to request data collection via A-DCCF are as specified in clause 8.12 of 3GPP TS 23.436 [4].

Editor’s Note: Whether the procedure specified in clause 8.12 of 3GPP TS 23.436 [4] can be reused with enhancements for AIMLE Server to request data collection via A-DCCF is FFS.

# *** End of Changes ***