3GPP TSG-SA6 Meeting #68	S6-253662

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S6-253190)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 744 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Leveraging N6 measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20, EDGEAPP |  |  |  |  |  | Date: |  |  | 2025-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed to leverage the N6 measurement capabilities specified by SA2 for AF influence on traffic routing (TS 23.501, 23.502 & 23.548), which is a capability of the feature that TS 23.558 does not currently utilise. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add List of N6 delay measurement assistance information to EAS profile to enable the EES to provide EAS Deployment Information (EDI) to the core network. Highlight when an EES is expected to provide EAS deployment information to the core network. Describe when “Indication of considering N6 delay” can be leveraged by the EES when performing AF influence on traffic routing. Further details are provided in the companion discussion paper S6-253189. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.558 does not currently specify usage of “Indication of considering N6 delay” when performing AF influence on traffic routing or how an EES can provide EDI to the core network on behalf of an EAS to enable that. Without this enhancement, the specification is underutilising the capability to which could be critical in achieving e2e application client service requirement KPIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.4.3.1, 8.6.7.2.1 and 8.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In the revision, replaced normative nomenclature in the proposed NOTE 2, NOTE and NOTE 4 in clause 8.4.3.1, 8.6.7.2.1 & 8.10.3 respectively. The Category is changed to B. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.4	EAS Profile

An EAS Profile includes information about an EAS used to describe services and service characteristics offered.

NOTE 1:	Information elements in the EAS Profile are provided by the ASP.

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > Bundle ID (NOTE 3) | O | Bundle ID as described in clause 7.2.10. |
| > List of EAS IDs (NOTE 2, NOTE 3) | O | List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > Main EASID | O | Indicate which EAS in a bundle takes the main EAS service role. |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| Allowed MNO information (NOTE 4) | O | Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS content synchronization support | O | Indicates if the EAS supports content synchronization between EASs. |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS coordinate ACR. |
| EAS Transport layer service continuity support | O | This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation |
| General context holding time duration (NOTE 1) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| List of N6 delay measurement assistance information | O | Includes parameters enable N6 delay monitoring corresponding to each EAS DNAI as described in clause 6.2.3.4 of 3GPP TS 23.548 [20]. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, overload warning etc.) |
| List of associated devices | O | List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type. |
| NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator. |  |  |

NOTE 2:	The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

* * * Second Change * * * *

#### 8.4.3.1	General

The EAS Registration procedure allows an EAS to provide its information to an EES in order to enable its discovery.

If there is a change in the requirements or the information of an EAS, it uses the EAS registration update procedure to update the EES.

The EAS uses the EAS de-registration procedure to remove its information from the EES.

EAS registration at the EES can be time bound. So, to maintain the registration, the EAS needs to send a registration update request prior to the registration expiration time. If a registration update request is not received prior to the registration expiration time, the EES treats the EAS as implicitly de-registered.

NOTE 1:	For registered EAS(s), the EES can request AF traffic influence for any UE, which is implementation specific.

NOTE 2:	If N6 delay measurement assistance information is provided by the EAS in its profile at registration or registration update, the EES makes the corresponding EAS Deployment Information available to the 3GPP core network via the Nnef_EASDeployment service as specified in 3GPP TS 23.502 [3] thereby enabling the EES to provide “Indication of considering N6 delay” when requesting AF traffic influence. The same service is used to remove the EAS Deployment Information at EAS de-registration.

* * * Third Change * * * *

##### 8.6.7.2.1	Procedure of application traffic influence trigger from EAS

Figure 8.6.7.2.1-1 illustrates the procedure of application traffic influence trigger from EAS.

Figure 8.6.7.2.1-1: Application traffic influence trigger from EAS

1.	The EAS sends an EAS traffic influence request.

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence.

3.	Upon successful authorization, the EES includes transaction ID, target DNAI, traffic descriptor information and N6 routing information at target DNAI in the Nnef_TrafficInfluence_Create Request to the NEF, or Npcf_PolicyAuthorization_Create Request to the PCF, to influence the traffic for EAS as described in 3GPP TS 23.501 [2], clause 5.6.7.1.

NOTE:	If EAS Deployment Information has been made available to the 3GPP core network via the Nnef_EASDeployment service as specified in 3GPP TS 23.502 [3], including N6 delay measurement assistance information, the EES provides “Indication of considering N6 delay” in the request to influence the traffic for EAS.

* * * Fourth Change * * * *

### 8.10.3	Capabilities utilized by EES and CES

When required, the EES and CES may utilize:

-	AF traffic influence functionality, including the user plane path management event notifications of the UE, from the 3GPP core network either directly (e.g. via PCF) as described in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3]; or indirectly (i.e. NEF/SCEF+NEF) as described in 3GPP TS 23.501[2] ,3GPP TS 23.502[3], 3GPP TS 29.522 [4];

-	the location information from the API exposed by 3GPP core network, e.g. SCEF/NEF/SCEF+NEF or LCS (Location Service) as specified in 3GPP TS 23.682 [17], 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10] to obtain the UE's location from the 3GPP Core Network;

-	capabilities exposed by the 3GPP core network, e.g. NEF or PCF, to establish an AF session with QoS, and QoS related event notifications subscribed with the 3GPP core network as specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [12];

-	capabilities exposed by the 3GPP core network, e.g. NEF or NWDAF, to analyse UE expected behaviour as specified in 3GPP TS 23.288 [18];

-	the monitoring capability exposed by the 3GPP core network as specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3];

-	application triggering service as specified for Nnef_Trigger_Delivery service in clause 4.13.2 of 3GPP TS 23.502 [3] or the device triggering procedure via T8 in clause 5.17 of 3GPP TS 23.682 [17] for the EEC Triggering service described in clause 8.16;

-	AF specific UE ID retrieval as specified in clause 4.15.10 of 3GPP TS 23.502 [3] to obtain an identifier that can be used when invoking further NEF provided services (e.g., location monitoring); and

-	EAS deployment functionality via the Nnef_EASDeployment service as specified in 3GPP TS 23.502 [3], where the related procedures are specified in clause 6.2.3.4 of TS 23.548 [20], to provide the 3GPP Core Network with EAS Deployment Information which can include N6 delay measurement assistance information to enable N6 delay monitoring.

NOTE 1:	While obtaining UE's location or analytics information using 3GPP core network capabilities for the request over EDGE-1 interface, if the EEC registration request from the UE indicates the UE mobility support requirement, the EES as per ECSP policy and EAS requirements may decide whether to subscribe to NEF or NWDAF for UE location information or its analytics.

NOTE 2:	If required, EES can determine UE’s serving PLMN information through procedures described in clause 8.18. Further, as required, EES can use this information to utilize 3GPP capabilities provided by UE’s serving PLMN.

NOTE 3:	When utilizing 3GPP core network capabilities, the EES, acting as an Application Function (AF) as defined in TS 23.503 [12], may use its EESID as the AF Identifier. Alternatively, when an EES is acting on behalf of an EAS that is also considered an AF, the EES may pass the EASID as the AF identifier in place of its EESID if it is authorized to do so.

NOTE 4:	When utilizing 3GPP core network AF traffic influence functionality, the edge or cloud enabler server can provide “Indication of considering N6 delay” in the AF traffic influence request, see clause 5.6.7 of 3GPP TS 23.501 [2], if the EAS Deployment Information has been made available to the 3GPP core network including N6 delay measurement assistance information.

* * * End of Change * * * *