3GPP TSG-SA WG6 Meeting #68	S6-253663

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253222)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0020 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription request sent to the MMTel Enabler server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The details of how the MMTel Enabler Client subscribes to the MMTel Enabler Server to receive notifications are not specified in 3GPP TS 23.392. However, this requirement is needed for Stage 3 when defining 3GPP TS 24.392. Two ENs in clauses 6.2.2.1.2 and 6.2.2.1.3 respectively state: Editor’s note: the details of subscription request sent to the MMTel Enabler server needs to be defined in stage 2. Therefore, this CR is proposed to add this detailed requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to add the details of how the MMTel Enabler Client subscribes to the MMTel Enabler Server to receive notifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details of how the MMTel Enabler Client subscribes to the MMTel Enabler Server for notifications are not specified, which will impact Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2.1, 8.3.3.2.2, 8.3.3.3.x(new), 7.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: added the definition of MMTel Enabler Client ID. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.3.2.1	The MMTel Enabler Client request DC application profiles based on notification

The MMTel Enabler Server can notify the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the DC application profiles, the detailed procedure is illustrated in Figure 8.3.3.2.1-1.

NOTE:	The MMTel Enabler Server can instruct the MMTel Enabler Client to request root application and DC application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.1-1: MMTel Enabler Client request DC application profiles based on notification

1.	The notification channel has been established between notification management client and notification management Server as per step 1 to step 6 specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications per step 7 specified clause 17.3.3 of 3GPP TS 23.434 [6]. This request includes the information elements as specified in Table 8.3.3.3.x-1 and the callback URL returned from the notification management server as specified  in step 4 of clause 17.3.3 of TS 23.434 [6].

If the MMTel Enabler Server provides the notification service, step 1 and step 2 are skipped.

3.	The MMTel Enabler Server sends a Root application and profile update notification to notify the MMTel Enabler Client in the UE to get the Root application or DC application profile. The Root application and profile update notification includes information elements as specified in Table 8.3.3.3.1-1.

The step 4 to step 8 are same as steps 1 through 5 in clause 8.3.2.2.

* * * Next Change * * * *

8.3.3.2.2	The MMTel Enabler Server notifies the DC application profile to the MMTel Enabler Client

The MMTel Enabler Server notifies the DC application profiles to the MMTel Enabler Client is illustrated in Figure 8.3.3.2.2-1.

Pre-condition:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.2-1: MMTel Enabler Server notifies the DC application profile list to UE

1.	The notification channel has been established between notification management client and notification management server as per step 1 to step 6 specified in clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications. This request includes the information elements as specified in Table 8.3.3.3.x-1 and the callback URL returned from the notification management server as specified  in step 4 of clause 17.3.3 of TS 23.434 [6].

3.	The MMTel Enabler Server checks the provision of Root application and DC application profile list is needed, e.g. the UE has IMS DC capability, an application version or application profile update is available, etc.

NOTE:	The detail additional checklists in this step are out of scope of the present document.

4.	The MMTel Enabler Server sends a Root application notification message to the MMTel Enabler Client in the UE. The notification message includes information elements as specified in Table 8.3.3.3.2-1.

* * * Next Change * * * *

8.3.3.3.x	MMTel Enabler Client subscribes to the MMTel Enabler Server request

Table 8.3.3.3.x-1 describes information elements for the MMTel Enabler Client subscribes to the MMTel Enabler Server request.

Table 8.3.3.3.x-1: Information elements for MMTel Enabler Client subscribes to the MMTel Enabler Server request

| Information element | Status | Description |
| --- | --- | --- |
| MMTel Enabler Client ID | M | Identity of the requesting MMTel Enabler Client |
| MMTel Enabler Client version (see NOTE) | M | The version of MMTel Enabler Client. |
| Validity Duration | M | Indicates how long the subscription will last (i.e. subscription lifetime) as requested by the MMTel Enabler client. |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different DC application profile list based on this IE. |  |  |

7	Identities

7.1	General

The following clauses list identities that are used in this technical specification.

7.2	Application ID (APPID)

The APPID is a globally unique value that identifies a Data Channel Application. It is assigned by the MMTel Enabler Server. The value of this IE is also used to set the "req-app-id" parameter in "a=3gpp-req-app" media-level SDP attribute as specified in 3GPP TS 26.114 [4] when the application requests application data channel(s).

7.3	MMTel Enabler Client ID (MMTECID)

The MMTECID is a used to identify an MMTel Enabler Client within the MMTel Service. Its value is the same as the identity of the DCMTSI Client that interacts with the MMTel Enabler Client.