3GPP TSG-SA WG6 Meeting #68	S6-253671

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253333)

Source:	Lenovo

Title:	Solution on support for Federated AIML services

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a solution to KI# 3 on support for federated AIML services.

2. Reason for Change

This paper proposes a solution to KI# 3 on support for federated AIML services. This solution addresses the key issue on how to configure and enforce federation agreements for AI/ML operations requiring joint or split processing by platforms operated by different providers. In particular, the solution provides a mechanism to translate a request for an AI/ML service to a requirement for a federated AI service, given the resource conditions and the federation agreements in place.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: Solution on support for federated AI services

### 7.x.1	Solution description

This solution addresses Key Issue #3.

In the context of AI/ML services, a federated AIML service can have different flavour based on the scenario.

- Scenario 1: an FL training operation in different Edge servers (EAS) or cloud servers (CAS): EAS#1 of EDN1 is an FL client, EAS#2 of DN#2 is another FL client and CAS of DN#3 is an FL server/aggregator.

- Scenario 2: an ML model training or inference operation split in different EDNs/DNs. In such scenario, the AIML operation (e.g. inference) is split among UE and network entities. If such network entities belong to different operators, then a federation agreement is in place and the AIML operation is split among different stakeholders /operators.

- Scenario 3: If an ML operation like model training is provided per ADAE analytics event, then the analytics service comprises the ML model training, model inference and the analytics operation which is provided by an analytics engine. Such ML-enabled analytics service may be distributed among different network or server entities provided by different network operators. One example can be the MNO #x is performing ADAES analytics on VAL performance in an analytics engine in the telco platform, and the ML model training is requested and provided by an external entity (e.g. AWS) using data provided by 3rd party. In certain use cases, the ML model training may be performed by a CAS in a less granular fashion, and the analytics is locally performed by one or more ECSPs at the respective edge platforms which may have overlapping or orthogonal coverage.

For the above scenarios, a federated AIML service can be defined as an AI/ML service which requires the joint operation of subsequent AI operations provided by different providers (MNO, ECSP, CSP, Device owner, App developer).

This solution addresses the key issue on how to configure and enforce federation agreements for AI/ML operations requiring joint or split processing by platforms operated by different providers. In particular, the solution provides a mechanism to translate a request for an AI/ML service to a requirement for a federated AI service, given the resource conditions and the federation agreements in place.

The procedure for the configuration of the federated AIML service based on the VAL request for an AI/ML task (ML model training or inference request) is shown below.

Pre-condition:

- AIMLE consumer such as AIMLE client or VAL server has ML model or address to ML model.

Figure 7.x.2-1: Procedure for registration on FL member registry

- An AIMLE server of ECSP/CSP #1 receives an ML model training request from a VAL server as described in clause 8.3.2 of TS 23.482 or an ML model inference request (as described in Solution #1).

- The AIMLE server determines to trigger a federated AIML service, which can be an AIMLE service such as federated training or inference spanning across different ECSP/CSPs. The determination can be based on the capability or availability limitations of ECSP/CSP #1 for providing the AI/ML service; hence support from another ECSP/CSP is needed.

- The AIMLE server performs an ML model information discovery, to discover ML model support for federated AI scenarios, where the discovery request (as specified in TS 23.482 clause 8.11.4.3) is enhanced to include federation information, i.e. list of information for different federation agreements related to the ML model, in particular the role of other edge/cloud providers in accessing or operating the ML model (or part of it). This includes information on partner ECSP/CSPs, permissions, capabilities and availability of the ECSP/CSP offering part of the AIML service.

- In this step, the AIMLE server obtains the identity and address of the target ECSP/CSP which is planned to undertake part of the AI/ML service based on step 3.

The AIMLE server determines to select the target AIMLE server of ECSP#2 to form the Federated AIML service and generates and Federated Service ID. The AIMLE server of ECSP#1 provides a configuration of the federated AIML service including the service area of interest, time validity, operation expected per lead and partner OP, the dataset requirements by the partner OP.

Then, AIMLE server sends a Federated AIML service configuration request to the partner ECSP/OP (e.g. AIMLE server in target ECSP) to provide the configuration parameters.

- The target AIMLE server after authorizing the request, it checks the federation agreement (with its ML repository) and confirms or negotiates the configuration parameters. Then, the target AIMLE server provides a positive or negative response to the source AIMLE server (or ECSP#1).

In case of negotiation (which can be due to the load/resource/energy situation in the target edge platform’s OP), the target AIMLE server may provide an alternative set of configuration parameters (e.g. limited coverage or time availability or time to accomplish the ML model training, or data availability) and further confirmation from source AIMLE server may be provided.

- The AIMLE server sends an ML model training/inference response including also the trained/inferred model and optionally a notification that the training/inference was outcome of a federation support (this may be needed to require more fees for the charging of the consumer).

- The AIMLE server performs an ML model information storage with the updated mapping of the ML model or AI service with the federation information.

### 7.x.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *