3GPP TSG-SA WG6 Meeting #68	S6-253672

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253336)

Source:	Lenovo

Title:	Solution on support for FL member registration and events notifications

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

The paper proposes a new solution on FL member registration and events notifications for KI#3.

2. Reason for Change

In TS 23.482 there are two capabilities (including procedures and APIs) related to the registration and events notifications related to FL members (or participants which may be FL clients). In particular:

- Support for FL member registration: This functionality covers the registration and registration update of the candidate FL member to the ML repository which is keeping the FL member registrations. Such candidate member can be a VAL server functionality or an enabler layer functionality (e.g. AIMLE server) which is registering to the ML repository/registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server).

- Support for FL events subscription and notification: This functionality enables a consumer (who can be the AIMLE server or a VAL server e.g. acting as FL server) to subscribe for FL related events and getting notified on changes on the availability of the FL members which are to be used for the FL-related task (e.g., training). This capability at the ML repository acting as an AIML service registry supports the subscription for events related to FL members and the notification to the consumer in case of changes. This feature assumes that such FL members (AIMLE or VAL server or AIMLE clients) have previously registered to this registry their availability and capabilities.

The capability for FL member registration and events notification in case of unavailability and other causes related to the registered FL members currently assumes that the entities which play role in FL process are connected to a single operator’s network (PLMN or NPN).

One practical issue is what happens if there are not many available or fully capable FL members, e.g. due to load or energy consumption in server side or due to poor channel conditions or out-of-coverage or idle VAL UEs, to undertake an FL task in a service area (as defined by the ASP or edge / cloud provider or from MNO).

As possible remedy to this issue, a service provider (e.g. SEAL provider or edge/cloud provider) may have in parallel service agreements with multiple MNOs in a target service area. For example, for an edge hotspot area (e.g. stadium, shopping street) MNO x and MNO y have overlapping coverage, and SEAL clients are deployed in VAL UEs connected to one or both MNOs, and also edge/cloud services (e.g. AIMLE or VAL apps) have a service agreement with one or more MNOs. Such extended support can be considered as multi-operator use case which is common in verticals like V2X (e.g. multi-operator eV2X services).

The solution provides a mechanism for supporting registration for candidate FL member (e.g. FL clients) which are application and/or application enablement layer entities (AIMLE server, VAL server, AIMLE clients, VAL clients), where the candidate FL members aim to operate in a service area which is covered by multiple networks operated by different operators, and the candidate FL members are connected to more than one network (either some members are connected to PLMN1 and some to PLMN2 or both).

Some assumptions related to this solution:

- An FL member can be a VAL server or client provided by the VAL service provider or an AIMLE server or client (e.g. AIMLE client) provided by the SEAL service provider.

- If the FL member is part of the VAL service provider or SEAL service provider at the server side, then the FL member can have interactions with one or more PLMNs via different PLMN agreements. In that case, the “serving PLMN” can be explained as the PLMN being served by the VAL/SEAL server for communicating the VAL/SEAL service (via VAL-UU or SEAL-UU). The “preferred PLMN” or “allowed PLMN” can be another network which may have less priority or limited subscription apart the serving PLMN and may serve the purpose of being allowed to utilize resources for reaching VAL/SEAL clients connected via different PLMN.

- If the FL member is at the VAL UE, then serving PLMN is more straightforward and is applicable to the case that the VAL UE connects to that PLMN. The “preferred” or “allowed” PLMN is for cases of dual connectivity or roaming.

The figure below shows some high-level business model:

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: Solution of FL member registration in multi-operator scenarios

### 7.x.1	Solution description

This solution addresses Key Issue #3.

The solution provides a mechanism for supporting registration and event notification for candidate FL member (e.g. FL clients) which are application and/or application enablement layer entities (AIMLE server, VAL server, AIMLE clients, VAL clients), where the candidate FL members aim to operate in a service area which is covered by multiple networks operated by different operators, and the candidate FL members are connected to more than one network (either some members are connected to PLMN1 and some to PLMN2 or both).

#### 7.x.1.1	Procedure for FL member registration

In cross-PLMN scenario, for registering the FL members using the multi-operator information, the following enhancements are provided in TS 23.482 clause 8.4.2 (for registration) and 8.4.3 (for registration update).

8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

NOTE: Such allowed MNO info is applicable to scenarios when the candidate FL member has limitations being grouped with other FL members being connected to other PLMNs. Such limitations are imposed based on the vendor providing the service in cross-PLMN scenarios, e.g. based on an interoperability indication. If the vendor is the same is the same across FL member connected to different PLMNs, then it may be possible that the FL members of different PLMNs collaborate e.g. in cases like VFL.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving or allowed PLMN.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving, or allowed PLMN.  In case of registration update, the mapping to the one or more PLMNs and/or the roles may be updated accordingly.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

### 7.x.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *