3GPP TSG-SA WG6 Meeting #68	S6-253673

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253125)

Source:	China Mobile

Title:	Sol for KI#4 Support for split operation node registration involving multiple AIMLE clients

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This document proposes a solution for the Key Issue #4 split operation node registration involving multiple AIMLE clients.

2. Reason for Change

AIMLE client may provides its computing capability to act as a processing node, especially multiple AIMLE client collaborating scenario. AIMLE client can provide their idle computing resource to perform computing tasks of other UEs. Network may provide benefits for such sharing. For example, network can give a discount to the UE when UE request for network computing tasks. Take the Berkeley Open Infrastructure for Network Computing (BOINC) as an example. BONIC is developed by the Department of Computer Science at the University of California, Berkeley. It is a distributed computing system. Initially designed for the SETI@home project, it has gradually expanded to other fields, including mathematics, medicine, astronomy, meteorology, and more. BOINC allows devices (PC and mobile phones) to contribute idle computing resource to support scientific computing. To reward participants for their contributions, BOINC has established a Credit and a Badges system.

Based on discussion above, this document provides a mechanism to support AIMLE client to provides computing capability to AIMLE server. In this way, AIMLE client can participate in split AI/ML operations as a processing node.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83.

* * * First Change * * * *

## 7.X	Solution #X: Split operation node registration involving multiple AIMLE clients

### 7.X.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

AIMLE client may provides its computing capability to act as a processing node, especially multiple AIMLE client collaborating scenario. AIMLE client can provide their idle computing resource to perform computing tasks of other UEs. Network may provide benefits for such sharing. For example, network can give a discount to the UE when UE request for network computing tasks. In this solution, both AIMLE client and VAL server can registrater in AIMLE server to participate in split AI/ML operations as a processing node.

The Split operation node registration procedure as specified in clause 8.14 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

Editor's Note:	 The UE registering as a processing node for split operation is subject to the user’s consent, which is FFS.

#### 8.14.2.4 Split operation node registration ##### 8.14.2.4.1 General Clause 8.14.2.4.2 illustrates the split operation node registration procedure. Clause 8.14.2.4.3 illustrates the split operation node registration update procedure. Clause 8.14.2.4.4 illustrates the split operation node de-registration procedure. ##### 8.14.2.4.2 Split operation node registration Figure 8.14.2.4.2-1 illustrates the procedure for VAL server/AIMLE client to register with the AIMLE server to indicate split operation capabilities. Pre-conditions: 1. The VAL server/AIMLE client has received information (e.g. URI, IP address) related to the AIMLE Server; 2. The VAL server/AIMLE client has received security credentials authorizing it to communicate with the AIMLE server; Figure 8.14.2.4.2-1: Split operation node registration 1. The VAL server/AIMLE client sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server/AIMLE client. The request includes information defined in Table 8.14.3.6-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information. 3. The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.3 Split operation node registration update Figure 8.14.2.5-1 illustrates the procedure for VAL server/AIMLE client to update a split operation node registration with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.3-1: Split operation node registration update 1. The VAL server/AIMLE client sends a split operation node registration update request to the AIMLE server to update an existing split operation registration. The request includes information defined in Table 8.14.3.8-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the registration information. 3. The AIMLE server sends a split operation node registration update response to the requestor. If the AIMLE server has successfully updated the registration, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.4 Split operation node de-registration Figure 8.14.2.4.4-1 illustrates the procedure for VAL server/AIMLE client to de-register for split operation with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.4-1: Split operation node de-registration 1. The VAL server/AIMLE client sends a split operation node de-register request to the AIMLE server. The request includes information defined in Table 8.14.3.10-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server terminates the registration for split operation. 3. The AIMLE server sends a split operation node de-register response to the requestor. If the AIMLE server has successfully terminated the registration for split operation node, the response includes an indication of success. If the registration for split operation was not terminated, the response includes an indication of failure and may include a reason for failure.

The Information flow of split operation node registration as specified in clause 8.14.3 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

#### 8.14.3.6 Split operation node register request Table 8.14.3.6-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node register request. Table 8.14.3.6-1: Split operation node register request Information element; Status; Description. Requestor identifier; M; The identity of the requestor (e.g., VAL server ID, AIMLE client ID).. Security credentials; M; The security credentials of the requestor.. Node information; M; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; M; Split operation capabilities of the VAL server/AIMLE client.. > Model information; M; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.7 Split operation node register response Table 8.14.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request. Table 8.14.3.7-1: Split operation node register response Information element; Status; Description. Successful response; O; Indicates that the request was successful.. > Registration identifier; M; The identifier of the registration.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. #### 8.14.3.8 Split operation node registration update request Table 8.14.3.8-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node registration update request. Table 8.14.3.8-1: Split operation node registration update request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. Security credentials; M; The security credentials of the requestor.. Node information; O; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; O; Split operation capabilities of the VAL server.. > Model information; O; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.9 Split operation node registration update response Table 8.14.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request. Table 8.14.3.9-1: Split operation node registration update response Information element; Status; Description. Successful response; O; Indicates that the request was successful.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure respone is sent, but not both. #### 8.14.3.10 Split operation node de-register request Table 8.14.3.10-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation de-register request. Table 8.14.3.10-1: Split operation node de-register request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. #### 8.14.3.11 Split operation node de-register response Table 8.14.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request. Table 8.14.3.11-1: Split operation de-register response Information element; Status; Description. Successful response; O; Indicates that the request was successful.. Failure response; O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure respone is sent, but not both.

### 7.X.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.X.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.X.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End Change * * * *