3GPP TSG-SA WG6 Meeting #68	S6-253376

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	KPN N.V

Title:	Solution to Key Issue #3 Cross-PLMN/Domain AIMLE client discovery, selection, monitoring

Spec:	3GPP TR 23.700-83 0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution proposes to enhance AIMLE client discovery and selection to enable cross-PLMN and cross-domain collaboration.

2. Reason for Change

In TS 23.482, there is currently a capability to discover and select AIMLE clients. However, Key Issue #3 has identified that cross-domain and cross-PLMN scenarios are not covered in TS 23.482 Release 19. This solution proposes to enhance AIMLE discovery and selection to include cross-domain and cross-PLMN deployments.

Specifically, we propose to enhance AIMLE server collaboration across PLMNs and domains, allowing one AIMLE server to discover, select, and monitor AIMLE clients in another domain when there are not sufficient AIMLE clients in the current domain or PLMN. Furthermore, since cross-PLMN and cross-domain operations incur additional cost, this solution also proposes to prefer AIMLE clients in the current domain/PLMN over those in other domains/PLMNs. This is achieved by monitoring AIMLE clients and replacing clients in other domains with clients in the current domain when they become available.

In the case of mobility of AIMLE clients from one domain/PLMN to another, this solution also proposes to enhance service continuity by allowing AIMLE clients that were part of the selection in the current domain/PLMN to be discovered and remain in the current selection via the partner AIMLE server.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V0.2.0.

* * * First Change * * * *

## 7.x Solution #x: Cross-PLMN/Domain AIMLE client discovery, selection, monitoring

### 7.x.1 Solution Description

#### 7.x.1.1 General

In TS 23.482, there is currently a capability to discover and select AIMLE clients. Key Issue #3 has identified that cross-domain and cross-PLMN scenarios are not covered in TS 23.482 Release 19. This solution enhances AIMLE client discovery and selection to include cross-domain and cross-PLMN deployments. Figure 7.x.1.1-1 shows the deployment scanrio where the AIMLE servers (AIMLE Server-1 and AIMLE Server-2) are in different domain or PLMN and the AIMLE servers in these different domain or PLMN have federation agreement to achieve discovery, selection and monitoring.

Figure 7.x.1.1-1: Use case for cross-domain/cross-PLMN federation

The following enhancements are proposed to enable cross-domain and cross-PLMN discovery, selection, and monitoring:

- Cross-domain and cross-PLMN deployments have AIMLE servers in different domains. When a discovery request fails to find sufficient AIMLE clients, the AIMLE server can discover AIMLE clients from another AIMLE server in another domain and select the desired clients.

- The AIMLE server, as per 3GPP TS 23.482 clause 8.13, can monitor AIMLE clients in its own domain/PLMN. To monitor AIMLE clients in another domain/PLMN, the AIMLE server can subscribe to the AIMLE server in that other domain/PLMN to receive status updates about those AIMLE clients.

- Based on the status of AIMLE clients in the current domain/PLMN and notifications received from the other domain/PLMN, the AIMLE server can replace AIMLE clients in the other domain/PLMN with AIMLE clients in the current domain/PLMN when they become available, in order to minimize cost.

- During AIMLE client discovery and selection, the AIMLE server can include in its request to the other AIMLE server (in another domain/PLMN) specific AIMLE clients that have migrated, to enhance service continuity.

#### 7.x.1.2 Procedure

Figure 7.x.1.2-1: Cross-domain/cross-PLMN AIMLE client discovery, selection and monitoring

Editor’s Note: The further details of the procedure is FFS.

7.x.2 Architecture Impacts

Editor’s Note: Architectural impact of cross-PLMN and cross-domain deployment is FFS.

7.x.3 Corresponding APIs

Editor’s Note: The corresponding APIs are FFS.

7.x.4 Solution Evaluation

Editor’s Note: The evaluation of the solution is FFS.

* * * End of Changes * * * *