3GPP TSG-SA WG6 Meeting #68	S6-253677

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-253262

Source:	Ericsson

Title:	Updates to Solution#9 on Edge AIMLE Server Deployment

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

Solution #9 introduces AIMLE server registration in a hierarchical AIMLE deployment. The solution is based on the deployment models given in TS 23.482 clauses A.4. However, the deployment models in A.4 are unclear and misalign with the procedures in clauses 8.24 and 8.25 of TS 23.482.

In addition, “security credential” exist in the procedures, which should be removed according to the agreement in previous meetings.

This paper proposes to update solution #9 by updating the deployment model in description (e.g. change the figure to as AIMLE server deployed at EAS), update the information flows, and remove “security credential” in the procedures.

2. Reason for Change

To make the edge AIMLE server deployment model clear and align with TS 23.482, this pCR proposes to update the solution #9.

3. Conclusions

This paper proposes proposes to update the solution #9 by updating the descriptions and figure for edge AIMLE server deployment, updating the corresponding information flows, and removing “security crediential” in the procedures.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

#### 7.9.1.1	AIMLE server registration

Figure 7.9.1.1-1 describes a procedure for an AIMLE server to register with another AIMLE server. In such a scenario, an AIMLE server deployed as EAS, which is located at an EDN, registers to a central AIMLE server.

Pre-conditions:

1.	AIMLE clients have registered with an edge deployed AIMLE server (i.e. an AIMLE server deployed as EAS which is located at an EDN) as described in clause 8.7.2.2 of 3GPP TS 23.482. The edge deployed AIMLE server receives AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE server has been pre-configured with endpoint information of a central AIMLE server.

Figure 7.9.1.1-1: AIMLE server registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration request to a central AIMLE server. The registration request includes the requestor identifier, endpoint information and a list of AIMLE clients registered with the AIMLE server. The request may include EDN information and AIMLE server capabilities.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server stores the registration information and creates a registration identifier. The central AIMLE server sends a AIMLE server registration response to the requestor. If the central AIMLE server has registered the requestor, the response includes an indication of success, a registration identifier and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

#### 7.9.1.2	AIMLE server registration update

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.2-1: AIMLE server registration update procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration update request to the central AIMLE server. The request includes the registration identifier, and may include updated AIMLE server information as described in Table 7.9.3.3-1. If the request is to renew the registration, only the registration identifier is included in the request.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server updates the information associated with the registration identifier and may update the registration expiration time if the request only includes the registration identifier. The central AIMLE server sends a AIMLE server registration update response to the requestor. If the central AIMLE server has updates the registration, the response includes an indication of success and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

#### 7.9.1.3	AIMLE server de-registration

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.3-1: AIMLE server de-registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server de-registration request to the central AIMLE server. The de-registration request includes the registration identifier.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server terminates the registration indicated in the request. The central AIMLE server sends a AIMLE server de-registration response to the requestor. If the central AIMLE server has de-registered the requestor, the response includes an indication of success. If the requestor was not de-registered, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

#### 7.9.3.1	AIMLE server registration request

Table 7.9.3.1-1 shows the request sent by an edge deployed AIMLE server (i.e. an AIMLE server deployed as EAS which is located at an EDN) to a central AIMLE server for registering AIMLE server and clients in the EDN service area.

Table 7.9.3.1-1: AIMLE server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the registering AIMLE server. |
| AIMLE server endpoint | M | The endpoint information (e.g., URL, URI, IP address, port) of the registering AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registering AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O (NOTE 1) | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O (NOTE 1) | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registering AIMLE server. |
| EAS information | O (NOTE 2) | Indicates information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | M | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registering AIMLE server. |
| NOTE 1: At least one of the information elements is present. NOTE 2: The information element shall be present for edge deployed AIMLE server. |  |  |

* * * Next Change * * * *

#### 7.9.3.3	AIMLE server registration update request

Table 7.9.3.3-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server registration update request.

Table 7.9.3.3-1: AIMLE server registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of an existing registration. |
| AIMLE server endpoint | O | The endpoint information (e.g., URL, URI, IP address, port) of the registered AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registered AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O | The identifier of the EDN service area associated with the registered AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registered AIMLE server. |
| EAS information | O | The information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | O | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registered AIMLE server. |

# *** End of Changes ***