3GPP TSG-SA WG6 Meeting #68	S6-253678

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-253263

Source:	Ericsson

Title:	New Solution for KI#6 on Enhance AIMLE Client Discovery to Support Diverse AIMLE Deployments

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue#6, the study aspects on support enhancements related to diverse AIMLE deployments scenarios include:

- Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client discovery services are needed to support AIMLE service. To support diverse AIMLE deployments scenarios, this pCR proposes to enhance the AIMLE client discovery.

2. Reason for Change

To support diverse AIMLE deployments scenarios, this pCR proposes to enhance the AIMLE client discovery.

3. Conclusions

This paper proposes solution on enhance AIMLE client discovery for diverse AIMLE deployments scenarios.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.XX	Solution#XX: Enhance AIMLE Client Discovery

### 7.XX.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client discovery services are needed to support AIMLE service provisioning.

This clause specifies the impact to existing information flows for the enhancements of AIMLE client discovery services.

The AIMLE information flows in 3GPP TS 23.482 for AIMLE client discovery can be enhanced (highlighted in bold italics) as follows.

### 8.8.3 Information flows #### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID, DNN and DNAI(s)) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. Edge AIMLE server Information; O; Information about edge deployed AIMLE server(s).. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS, which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server.. NOTE: At least one of the IEs shall be provided when edge deployed AIMLE server.. #### 8.8.3.2 AIMLE client discovery response Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure. Table 8.8.3.2-1: AIMLE client discovery response Information element; Status; Description. Status; M; The status for the request: success or fail.. List of AIMLE client IDs; M; A list of AIMLE client IDs that matches the AIML discovery criteria.. > AIML supported tasks; O; Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks.. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS (which serving the AIMLE client), which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server which serving the AIMLE client.. NOTE: At least one of the IEs shall be provided when the AIMLE client is served by edge deployed AIMLE server..

### 7.XX.2	Architecture Impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.XX.3	Corresponding APIs

Editor’s Note:	The corresponding APIs for supporting the solution is FFS.

### 7.XX.4	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***