3GPP TSG-SA WG6 Meeting #68	S6-253680

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253368)

Source:	KPN N.V

Title:	Solution for Key Issue #7: VFL Sample Alignment between VAL Servers

Spec:	3GPP TR 23.700-83 0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution introduces a VFL sample alignment solution when VAL servers who are involved in VFL process as VFL participants need to perform sample alignment either for training or inference purposes.

2. Reason for Change

To achieve VFL process, VAL servers which are VFL participants require sample space alignment where the samples are UEs identified by local identifiers in each VAL server (for instance, AF specific UE identifiers and IP addresses). Sample space alignment allows matching corrsponding samples in a sample space in privacy-preseving maner. As identified by Key Issue # 3 the AIMLE server which has the ability to know the identity of UEs, can play a crucial role in sample alignment wihout revealing the identity of the UEs. This contribution proposes proposes to enhance the AIMLE Server to support sample alignment among VAL server VFL particpants.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V0.2.0.

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Vertical Federated Learning (VFL): A federated learning technique in which multiple VFL participants jointly perform training and inference on a common sample space with different feature sets, without exchanging raw data, thereby preserving their local data privacy.

VFL process: A process of performing training or inference using the VFL technique.

VFL participant: An entity that takes part in a VFL process, either as a VFL server or a VFL client.

VFL Server: A VFL participant that coordinates the VFL process by aggregating results from VFL clients and propagating updates to them during training.

VFL client: A VFL participant that performs local training and inference, sends results to the VFL server, and updates local parameters based on information received from the VFL server.

VFL Sample alignment: A stage in a VFL process during which samples are aligned among VFL participants, ensuring that training and inference are performed consistently across participants on the same sample space.

* * * Second Change * * * *

## 7.x Solution #x: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

### 7.x.1 Solution Description

#### 7.x.1.1 General

This solution addresses key issue # 7 which identifies VFL sample alignment among open issues. In a VFL process, a VAL server may act as a VFL server (active party), coordinating training and inference by aggregating results from other VAL servers acting as VFL clients (passive parties). In this context, the sample space consists of user equipment (UEs), each represented by local identifiers specific to the VAL servers.

The VFL process begins by first aligning the samples among VFL participants so that inference and training are performed properly. This is performed without sharing the UE identifiers with other VFL participants. Each UE in the sample space may be identified with different identifications in each of the VFL participants such as different AF specific UE IDs or different public IP address of the UE. Therefore, the process of VFL sample alignment should be able to align the UEs that are involved in a VFL process without sharing the UE identifiers while resolving the potential difference in how the VFL participants identify the UEs.

This solution proposes to enable the AIMLE server to support sample alignment among VFL participants while preserving the privacy of the UEs and while resolving the potential differences in how each VFL participant identifies the UEs.

#### 7.x.1.2 Procedure

Assumptions:

- VAL Server acting as VFL server coordinates other VAL servers acting as VFL clients to perfom VFL training/inference.

- The sample space is associated with UEs which are locally identified by AF Specific UE IDs, IP addresses, or similar identifiers\.

Preconditions:

- The VAL servers acting as VFL clients and VAL server acting as VFL server have registered with the AIMLE server, providing metadata including supported feature sets, and local UE identifiers.

- The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions.

Editors Note: Whether a new procedure is needed for registration or TS 23.248 clause 8.4 should be updated for this purpose is FFS.

- The VFL server has discovered the VFL participants with the desired feature set and sample space. The AIMLE server can also assist in discovering VFL clients that match the required feature set and sample space using previously mapped UE identifiers.

Editors Note: The detailed procedure for VFL participant discovery is FFS.

Figure 7.x.1.2-1: VFL process sample alignment procedure

- VFL training/inference request: the VFL server sends request including the desired VFL clients, feature set, and sample space.

- VFL client selection and UE mapping: The AIMLE server selects the required VFL clients fulfling the required feature set and sample space. The UEs in the sample space of the VFL clients and VFL server is matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into a common UE identifiers (e.g, GPSI).

- Sample correlation: Sample correlation ID is assigned for each UE in the sample space for consistent referencing during training and inference.

- Training initiation to VFL clients: The AIMLE server sends VFL training/inference initiation message to the selected VAL servers acting as VFL, including the assigned sample correlation IDs.

- VFL training/inference response: the AIME server sends VFL training/inference response to the requesting VFL server, including the sample correlation IDs corresponding to the aligned UEs.

- Training/inference execution: The VFL server and the VFL clients proceed with the training/inference using the aligned sample space, with sample correlation IDs ensuring consistent sample alignment across particpants.

Editor’s Note: Other modes of training/inference operations are FFS.

7.x.2 Architecture Impacts

There is no architectural impact from this solution.

7.x.3 Corresponding APIs

Editor’s Note: The corresponding APIs are FFS.

7.x.4 Solution Evaluation

Editor’s Note: The evaluation of the solution is FFS.

* * * End of Change * * * *