3GPP TSG-SA WG6 Meeting #68	S6-253171

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253171)

Source:	Ericsson

Title:	Pseudo-CR on overall evaluation for technical gap 4

Spec:	3GPP TR 23.700-35 v. 1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR introduces overall evalution to the available solutions related to technical gap 4.

2. Reason for Change

As the study has completed 75% in SA6#67, this pCR starts looking into the overall evaluation of solutions related to technical gap 4.

3. Conclusions

An overall evalution is introduced in this pCR.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v. 1.0.0

* * * First Change * * * *

### 9.1.4	Technical gap #4

Technical gap #4 focuses on NRM services supporting IoT devices for dedicated scenarios such as power saving. Both solution 9 in clause 8.1.9 and solution 11 in clause 8.1.11 address this gap.

Solution 9 focuses on the interaction between the NRM server and the 3GPP network to address the gap under consideration. The VAL server requests the power saving service by identifying the requested IoT power saving service (e.g., UE identity, external group identity and application and network pattern parameters) to the NRM server. As a result, the NRM server may invoke one or multiple network related APIs; namely, MonitoringEvent, CpProvisioning and NpProvisioning APIs. Hence, it introduces a single service which may aggregate multiple APIs towards the 3GPP network to achieve the requested power saving service indicated by the VAL consumer.

On the other hand, Solution 11 approaches the gap under consideration from an end-to-end perspective and potential implementation/ deployment considerations. Furthermore, it considers addressing the gap via two cases: Case 1 focuses on the scenario when the VAL server triggers the power saving assist service by identifying a desired power saving mode, whereas Case 2 describes the scenario when the VAL client triggers a desired power saving mode.

Whether the VAL server or the VAL UE is the entity to request the desired power saving mode, the NRM server determines the corresponding power saving configuration, network and communication patterns parameters for the target UE(s) and invokes CpProvisioning and NpProvisioning APIs towards the 3GPP network. As a result, the target UE interacts with the 3GPP network according to the existing procedures indicating the desired power saving configuration (i.e., DRX or eDRX configuration). Hence, this solution not only focuses on the interaction between the VAL consumer and the NRM server, but it rather describes an end-to-end behavior to achieve the power saving service for IoT devices for all possible scenarios: when the VAL server initiates the power saving service, or when the VAL UE initiates the power saving service. Therefore, solution 11 can be recommended to the normative phase.

* * * End of Changes * * * *