3GPP TSG-SA WG6 Meeting #68	S6-253746

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253709, S6-253650, S6-253366)

Source:	KPN N.V.

Title:	Solving EN: Multi-Client Split Operation Pipeline

Spec:	3GPP TR 23.700-83 0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Anthony Pages (Anthony.pages@tno.nl), Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution provides detailed procedures and corresponding API definitions to resolve the Editor’s Notes in Solution #8 Multi-Client Split AI/ML Operations Pipeline. These Editor’s Notes highlight the need to specify the subscription and notification mechanisms related to the creation and joining of a split operation pipeline. In response, this contribution introduces the procedural descriptions for the Split Operation Pipeline Create Subscription and Split Operation Pipeline Join Subscription, along with the associated notification behaviors.

2. Reason for Change

Clause 7.8.2 contains two unresolved Editor’s Notes indicating that the specification does not yet describe the procedures and notification mechanisms for subscription to split operation pipeline events. Without these definitions, the interaction model between clients and the edge in a multi-client split operation context remains incomplete. These procedures are also not well defined, especially on how to properly inform changes in the pipeline for participating clients in the AIML shared pipeline.

3. Conclusions

This contribution resolves the ENs by introducing complete descriptions of the Split Operation Pipeline Create Subscription and Join Subscription procedures. It also defines the associated APIs, including the relevant input and output parameters, notification behavior, and information elements.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V0.2.0.

* * * First Change * * * *

## 7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline

### 7.8.1	Solution Description

#### 7.8.1.1	General

This solution addresses key issues related to split AI/ML operations involving multiple AIMLE clients. The proposed solution enhances split AI/ML operations by facilitating multiple AIMLE client collaboration within the same pipeline. It allows the creation of multi-client AI/ML split operation pipeline when such a pipeline is not yet created and allows joining the pipeline when such a pipeline has already been created. The solution also keeps AIMLE clients in sync with the latest status of the pipeline through notifications as more AIMLE clients join and the pipeline is adjusted according to the resources of the newly joining AIMLE clients. The solution ensures efficient resource utilization and continuous service, even with mobility and varying device constraints.

Editor's Note: The feasibility of sharing a pipeline among AIMLE clients is FFS

Use Cases:

-	Enhanced Smart City Surveillance with Mobility: Surveillance cameras, both stationary and mobile, capture video footage and process raw data on the cameras, sending subsequent layers to an edge server for immediate processing. AIML split operations enable real-time improved surveillance and threat detection for public safety through a shared pipeline that allows collaborative data processing. The pipeline needs to be re-adjusted based on computing resource powers for the surveillance cameras. Pipeline re-adjustment is also needed as the mobile surveillance cameras change service areas. This results in efficient resource utilization, energy saving, and larger surveillance.

-	Collaborative Autonomous Driving: A group of autonomous vehicles collaborate by sharing and processing data together. A more concreate example is vehicles utilizing shared Local Dynamic Maps (LDM) which is collaboratively updated and used by each vehicle locally. The raw data of each vehicle is processed locally and it is sent for subsequent layers in the edge server for immediate processing. The shared AIML split operation enable real-time collaboration to optimize driving conditions (e.g., efficient routes) and reduces computational burden in the vehicles. Pipeline re-adjustment is needed based on the computing resource power of the participants. Pipeline-readjustment is also needed as the vehicles change service area during mobility. This results in a larger collaboration coverage.

#### 7.8.1.2	Multi-client pipeline operation

Figure 7.8.1.2-1: Multi-client AI/ML split operation

The solution involves a multi-client split operations pipeline facilitated by AIMLE clients and servers. Initially, an AIMLE client initiates a discovery request to the AIMLE server to participate in a split operation pipeline, as illustrated in Block A of the figure. This is similar to clause 8.14.2.2 in 3GPP TS 23.482 split operation pipeline discovery. A key addition for this solution is an expansion of the split operation discovery filters in clause 8.14.3.2-2 with additional criteria for multi-client split operations within the pipeline. Following the discovery request, the split operation profile in the discovery response, as in clause 8.14.3.3 Table 8.14.3.3-2, is also expanded to include information about a pipeline with multi-client split operation pipeline support, the usage of the multi-client split operation pipeline (in usage information), and whether such a pipeline has already been created or not.

If in the discovery stage, as in Block B, the discovered instance of the split operation pipeline is a pipeline that has not yet been created, the AIMLE client will then request the creation of a new pipeline, explicitly mentioning the option for multi-client operations in the split operation profile, as shown in Block B, and subscribes for the split operation event. This is similar to clause 8.14.2.3 in 3GPP TS 23.482 and clause 8.14.2.5 in 3GPP TS 23.482, with additional expansion of clause 8.14.3.4-1 in 3GPP TS 23.482 to include multi-client support in the pipeline, expansion of the usage information to include further details on the multi-client pipeline, and also the computing resources of the AIMLE client to be able to efficiently adjust the pipeline later on as more AIMLE clients join the pipeline. Furthermore, instead of 3GPP TS 23.482, which has two separate procedures for pipeline creation and subscription, this solution proposes a split operation pipeline create subscription, which will allow creation and subscription together. Although this is a useful procedure for a single AIMLE client case as well, it becomes more important for split AI/ML operations involving multiple AIMLE clients. This is because as other AIMLE clients join or leave and the pipeline is modified, notifications need to be sent to the AIMLE clients so that proper client-side adjustments are made as needed.

Subsequently, once a multi-client AI/ML split operation pipeline has been created, other AIMLE clients who seek to also engage in the multi-client AI/ML split operation pipeline begin by initiating a discovery procedure as in Block C, which is similar to Block A. If a pipeline is already created according to Block B, it identifies the existing created pipeline that supports multi-client AI/ML split operation, fulfilling the respective split operation discovery filters. Therefore, instead of a split operation pipeline create subscription, it will proceed with a split operation pipeline join subscription, which will allow the AIMLE client to join the created pipeline as in Block D. To enable adjustment of the pipeline considering the newly joining AIMLE clients, each client indicates its respective computing resources during the join. This process can be scaled up to accommodate an n-number of AIMLE clients, allowing for extensive multi-client collaboration in split AI/ML operations.

As AIMLE clients with different computing resources join the split operation pipeline, the pipeline may need to be adjusted, and the participating AIMLE clients need to be notified of changes to the pipeline so that they can make client-side adjustments. For this purpose, as shown in Block E, a split AI/ML operation notification is sent to all AIMLE clients as per clause 8.14.2.5.3 of 3GPP TS 23.482.

#### 7.8.1.3	Multi-client pipeline service continuity

A multi-client AI/ML split operation pipeline should be resilient to the mobility of the AIMLE clients. When edges are involved in the split operation pipeline, not only should the pipeline be adjusted, but AIMLE clients should also be enabled to collaborate in the pipeline even if they are in different service areas. This will enable better service continuity support. When an AIMLE client leaves a pipeline, the pipeline also needs to be adjusted to stay optimal.

This solution achieves service continuity in two ways:

1.	First, the pipeline is re-adjusted according to the computing resources of the AIMLE clients to obtain matching layers in the pipeline. For example, when AIMLE clients with low computing resources join, the pipeline is adjusted so that more parts of the pipeline are offloaded to the nodes. Conversely, when an AIMLE client with low resources leaves, the pipeline can be re-adjusted so that more parts of the pipeline can be handled by the remaining AIMLE clients.

2.	Second, when AIMLE clients change service areas, the pipeline can still be adjusted so that the common layers in the pipeline remain matched. When an AIMLE client moves to a different service area, adjustments are made based on the available AIMLE clients in the current service area. When the AIMLE client moves back to the original service area, re-adjustment is performed to keep the common layers in the pipeline matched. For example, when an AIMLE client with low computing resources moves to a different service area, part of the pipeline in the current service area can be relaxed for the other AIMLE clients so that more of the pipeline can be on the client side. The edges in the service area of the low-resource AIMLE clients can be part of the pipeline in line with the computing resources of the AIMLE clients in the service area to which it moved. The source and target AIMLE servers can continue to communicate as in step 1 of 8.24.2 TS 23.482.

#### 7.8.1.4	Procedures to support Multi-client AI/ML split operation

##### 7.8.1.4.1	Split operation pipeline join

Figure 7.8.1.4.1-1 illustrates the procedure for one or more AIMLE clients to join to a split operation pipeline with the AIMLE server.

Pre-conditions:

- The AIMLE client has performed split operation pipeline discovery with the AIMLE Server and a multi-client split operation pipeline has been found;

Figure 7.8.1.4.2-1: Split operation pipeline join

- The AIMLE client sends a request to join an existing split operation pipeline. To enable dynamic adjustment of the pipeline based on the capabilities of newly joining clients, each AIMLE client includes its computing resource information in the request.

- Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

- The AIMLE server responds with either a positive acknowledgment or a negative acknowledgment with a cause of failure (e.g., failure to authorize).

##### 7.8.1.4.2	Split operation notification

Figure 7.8.1.4.2-2 illustrates the procedure for an AIMLE server to notify all AIMLE clients subscribed as per clause 8.14.2.5.3 of 3GPP TS23.482.

Pre-conditions:

- The AIMLE clients have joined a pipeline;

- The AIMLE clients have subscribed to pipeline event as per step 1.a. of clause 8.14.2.5.2 of 3GPP TS 23.482;

Figure 7.8.1.4.2-1: Split operation notification

- When an AIMLE client joins the split operation pipeline, the AIMLE server evaluates the updated pipeline configuration and determines whether adjustments are needed to maintain optimal performance and coordination. This also triggers, as a result, a pipeline update event as per step 1 of clause 8.14.2.5.3 of 3GPP TS 23.482.

- The AIMLE server sends the split operation notification to all subscribed AIMLE clients as per step 2 of clause 8.14.2.5.3 of 3GPP TS 23.482.

* * * Next Change * * * *

### 7.8.2	Architecture Impacts

No architectural impact has been identified.

* * * Next Change * * * *

### 7.8.3	Corresponding APIs

#### 7.8.3.1	Split operation pipeline discovery request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.x

- 3GPP TS 23.482 Clause 8.14.3.2	Split operation pipeline discovery request

3GPP TS 23.482 Table 8.14.3.2-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.2-1: Split operation pipeline discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier) |
| Split operation discovery filters | M | Split operation discovery filters. |
| > multi-client split operation pipeline support | O | Indicates if multi-client AIMLE split operation pipeline support is required or not. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.) |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.) |
| > number of nodes | O | Minimum number of nodes required to support AIML operation splitting |

#### 7.8.3.2	Split operation pipeline discovery response

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- 3GPP TS 23.482 Clause 8.14.3.3	Split operation pipeline discovery response

3GPP TS 23.482 Table 8.14.3.3-1 shows the response sent by sent by the AIMLE server after processing of split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.3-1: Split operation pipeline discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the request was successful. |
| > List of nodes (NOTE 1) | O | The list of discovered nodes. |
| > List of split operation profiles (NOTE 1) | O | The list of split operation profiles as specified in Table 8.14.3.3-2 of 3GPP TS 23.482. |
| Failure response | O (NOTE 2) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE 1: Only one of the IE shall be present for successful response. NOTE 2: One of the IEs shall be present. |  |  |

3GPP TS 23.482 Table 8.14.3.3-2: Split operation profile

| Information element | Status | Description |
| --- | --- | --- |
| Split operation pipeline identifier | M | The identifier of the split operation pipeline. |
| Head endpoint | M | The endpoint information of the head node for providing intermediate data. |
| Tail endpoint | M | The endpoint information of the tail node for obtaining results. |
| Mutli-client split operation pipeline support | O | Indicates if multi-client split operation pipeline is supported or not. |
| Usage information | O | The usage information of the AI/ML split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Stage information | M | The list of stage(s) of the AI/ML split operation, the nested information is provided for each stage. |
| > Stage identifier | M | The identifier of the stage. |
| > number of nodes | O | Number of nodes included in the stage |
| > head node | M | Endpoint of the head node - for providing initial the inference data. |
| > tail node | M | Endpoint of the head node - for obtaining inference results. |
| > order of the nodes | M | The order of the nodes in the AI/ML split operation (including head node, tail node). |
| >> list of nodes | M | List of all discovered node in the order in which they process the data. |
| >> notification target | O | Endpoint information where the inference result of the split operation is sent by the nodes |
| > Model information | M | The ML model information used in the stage (e.g., identifiers, versions, etc.). |

#### 7.8.3.3	Split operation pipeline create request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- TS 23.482 Clause 8.14.3.4	Split operation pipeline create request

TS 23.482 Table 8.14.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline create request.

TS 23.482 Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Computing resource information | O | The computing resource information of the requestor. |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > Multi-client split operation pipeline support | O | Indicates if multi-client split operation pipeline support is required or not.. |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the tail node. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

#### 7.8.3.4	Split operation pipeline join request

Table 7.8.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline join request.

Table 7.8.3.4-1: Split operation pipeline join request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Split operation pipeline identifier | M | Identifier of the split operation pipeline for which the requestor is joining. |
| Computing resource information | O | The computing resource information of the requestor. |

#### 7.8.3.5	Split operation pipeline join response

Table 7.8.3.5-1 shows the response sent by an AIMLE server after processing of split operation pipeline join request.

Table 7.8.3.5-1: Split operation pipeline join response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * End of Change * * * *