3GPP TSG-SA WG6 Meeting #68	S6-253753

Gothenburg, Sweden 25th – 29th August 2025	(revision of 3337, 3123, 3645, 3701)

Source:	Lenovo, CMCC, Ericsson, InterDigital

Title:	Support VAL UE energy data utilization for AIML services

Spec:	3GPP TR 23.700-44 v0.0.0

Agenda item:	9.9

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new key issue on supporting VAL UE energy data collection and utilization for AIML services

2. Reason for Change

3GPP SA6 in Rel-18 specified an application data analytics enablement functionality (ADAES, TS 23.436). ADAES is a SEAL functionality for providing end to end performance analytics (e.g. VAL server performance)

One analytics service of ADAES, as introduced in Rel-19, is the Support for DN energy Analytics. This feature supports a logical functionality at the ADAES to provide analytics on the energy consumption /efficiency of an edge platform (including the EESs / EASs). The DN energy analytics is performed per DNN/ DNAI and may be used to trigger the application server migration to different cloud. The analytics are based on NWDAF analytics and UPF/DN measurements on user plane load as well as edge/app side measurements on the energy consumption.

Given the fact that ADAES supports ML-enabled analytics using AIMLE service (as defined in TS 23.482), and in particular the requirement for an ADAE/AIMLE Client undertaking part of an ML task, such as ML model training in an FL operation, the energy consumption / efficiency analytics would help selecting the most appropriate UE as ML/FL member in the application layer ML task (e.g. training, inference).

The key issue aims to study:

- How to acquire energy data from the UE side and support predicting in ADAE whether a UE is suitable to act as ML participant in an application layer ML task (or ML model lifecycle / workflow operation).

- How to monitor and fetch energy information from a UE of interest, where this UE is expected to participate in an FL process as an FL member (or FL client).

- How to decide whether to use a UE listed as a potential FL member to train an ML model based on its energy status (i.e., current or predicted).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.0.0.

* * * First Change * * * *

# 5	Key issues

## 5.x	Key issue #x: Support AIMLE enhancements for AIML services energy saving

### 5.x.1	Description

SA1 identified use cases and requirements for energy efficiency and energy saving in Release-19 and Release-20 (in TS 22.261[x]). Energy efficiency and energy saving is a critical feature in 5G. Such use cases and requirements have application layer impacts. AI/ML requires more energy consumption at the device side as well as the network side. TS 22.261 has defined energy related information as a service criterion for multiple service including AIML.

3GPP SA6 in Rel-18 specified an application data analytics enablement functionality (ADAES, TS 23.436). ADAES is a SEAL functionality for providing end to end performance analytics (e.g. VAL server performance). One analytics service of ADAES, as introduced in Rel-19, is the Support for DN energy Analytics. This feature supports a logical functionality at the ADAES to provide analytics on the energy consumption /efficiency of an edge platform (including the EESs / EASs). The DN energy analytics is performed per DNN/ DNAI and may be used to trigger the application server migration to different cloud. The analytics are based on NWDAF analytics and UPF/DN measurements on user plane load as well as edge/app side measurements on the energy consumption.

In addition, in Rel-19 AIML Enablement (AIMLE) service is described in TS 23.482. AIMLE server is a SEAL server which includes of a common set of services for comprehensive enablement of AIML functionality. AIMLE server defines the following group of capabilities:

- Support for application-layer ML model related aspects, including model retrieval, model training, model monitoring, model selection, model update and model storage / discovery.

- Assistance in AI/ML task transfer and split AI/ML operations.

- Support HFL/VFL operations, including FL member registration, FL grouping and FL-related events notification, VFL feature alignment, HFL training.

- Support for AIMLE client registration, discovery, participation and selection.

Given the fact that ADAES supports ML-enabled analytics using AIMLE service (as defined in TS 23.482), and in particular the requirement for an ADAE/AIMLE Client undertaking part of an ML task, such as ML model training in an FL operation, the energy consumption / efficiency analytics would help selecting the most appropriate UE as ML/FL member in the application layer ML task (e.g. training, inference).

### 5.x.2	Open Issues

The key issue aims to study:

- How to collect and utilize energy consumption data (and which data) from the UE side to support an application layer ML task (or ML model lifecycle / workflow operation).

- How to enhance functionality and architecture of AIMLE to support energy saving.

- Whether and how to use energy consumption information to fulfill FL member selection, AIMLE client selection, split AI/ML operation, etc.