3GPP TSG-SA WG6 Meeting #68	S6-253761

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253737, S6-253615, S6-253065)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0044 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML model training procedure alignment with client selection and HFL/VFL procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.3 ML model training describes a general procedure that supports both HFL and VFL procedures described in clauses 8.12 and 8.18. There are missing information elements described for HFL and VFL training and for AIMLE client selection procedures that are not found in clause 8.3. The following are changes to align the client selection, HFL, and VFL procedures with the ML model training procedure. - Added ML model training identifier (in Table 8.3.3.2-1 and Table 8.3.3.3-1) for ML model training procedure to track the status of ML training requests for cases where multiple ML model training are requested by the same requestor. - In Table 8.3.3.1-1, added Number of required AIML clients information element to be use with Member selection criteria to align with client selection procedures described in clauses 8.9 and 8.13. Also added AIMLE client set identifier from clause 8.9 as an option for List of member clients. - In Table 8.3.3.1-1, added Dataset identifiers, Number of data samples, and Operational schedule required for HFL/VFL operations. These information elements are used in HFL/VFL training procedures. - In Table 8.3.3.1-1, added VFL specific information elements to align with VFL procedure described in clause 8.18. - In Table 8.3.3.3-1, removed duplicate Errors list and added Percentage completion information elements. Since ML model training is a highly iterative process, Percentage completion provides useful status to the requestor as described by step 7 of VFL procedure. - In Table 8.7.3.2-2, clarify training is for ML model in description for Supported AI/ML operations. - In pre-conditions of clause 8.18.2, replace VFL profile with AIMLE client profile, which already contains information for VFL. VFL profile is not defined in the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align information elements among clauses 8.3, 8.9, 8.12, 8.13, and 8.18. Clause 8.3 describes ML model training that triggers HFL (clause 8.12) or VFL (clause 8.18) training. Clauses 8.9 and 8.13 describe AIMLE selection criteria that is referenced in clauses 8.12 and 8.18. There are information elements used in clauses 8.12 and 8.18 that are not found in clause 8.3. This contribution resolves the discrepancies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how HFL and VFL training will not work due to missing information elements in the ML model training procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.1, 8.3.3.2, 8.3.3.3, 8.7.3.2, 8.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.3.3	Information flows

#### 8.3.3.1	ML model training request

Table 8.3.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.3.3.1-1: ML model training request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE 1) | List of member clients or AIMLE client set identifier to be utilized for training the ML model. |
| Member selection criteria | O (NOTE 1) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| Number of required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection criteria. |
| ML model information | O (NOTE 2) | Identifies the ML model that has to be distributed to the selected member clients for training. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model requirement information | O (NOTE 2) | Identifies the requirement for selecting a model to be trained and this information contains the filtering criteria for selecting the model as specified in Table 8.11.4.1-2. |
| Dataset identifiers | M | Identifier of dataset used to HFL or VFL training. For VFL training, multiple dataset identifiers is specified for different data domains. |
| Number of data samples | O | The number of data samples required for a round of HFL or VFL training. |
| Operational schedule | O | A schedule for when training is to occur. |
| VFL specific parameters | O (NOTE 3) | Parameters specific to VFL training |
| > Dataset common features | O (NOTE 3) | A list of one or more common features is specify for VFL training. Common features can be UE identifiers, AIMLE client identifiers, group identifier, VAL service identifier, area of interest, and VAL service area. |
| > Data domain feature ID lists | O (NOTE 3) | List of features for each data domain(s) of the datasets at the UE for VFL training. |
| > Feature alignment information | O | Information provided to align features from dataset of different domains for VFL training. Alignment information is also provided for data domain features with ground truth data. |
| > Data labels | O | Ground truth data provided for VFL training. |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new member clients selected or de-selected. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE 1: At least one of these IE shall be present. NOTE 2: At least one of these IE shall be present. NOTE 3: These IE shall be present for VFL training. |  |  |

#### 8.3.3.2	ML model training response

Table 8.3.3.2-1 describes the information flow of ML model training response from the AIMLE server to the VAL server.

Table 8.3.3.2-1: ML model training response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| > ML model training identifier | M | An identifier for the ML model training request. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for training. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present |  |  |

#### 8.3.3.3	ML model training notification

Table 8.3.3.3-1 describes the information flow of ML model training notification from the AIMLE server to the VAL server.

Table 8.3.3.3-1: ML model training notification

| Information element | Status | Description |
| --- | --- | --- |
| ML model training identifier | M | An identifier for the ML model training request the notification is for. |
| List of AIMLE clients | O (NOTE) | Indicates the list of AIMLE clients selected or de-selected for the ML model training. |
| Training output | O (NOTE) | Output of training, e.g., ML model parameters for the training. |
| Percentage completion | O | Indicates a completion percentage for the training. |
| Errors list | O (NOTE) | A list of errors, if any, encountered during training process. |
| NOTE: At least one of these IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.7.3.2	AIMLE client registration request

Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request.

Table 8.7.3.2-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of supported profiles | M | Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided. |
| > AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2. |
| > List of supported services | M | List of VAL service IDs with corresponding permissions. |
| > VAL service ID | M | The identifier of the VAL service. |
| > Service permission level | O | Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage). |

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as ML model training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

* * * Next Change * * * *

### 8.18.2	Procedure for supporting VFL

Pre-conditions:

1.	VFL members registered their AIMLE client profile to the AIMLE Server. The VFL members may update their status or information in their AIMLE client profile to the AIMLE Server.

2.	The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains.

3.	The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications.

Figure 8.18.2-1: Procedure for supporting VFL

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2.

2.	[Optional] The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1. If the retrieved model is already trained and meets the machine learning requirements requested by the consumer, the procedure continues to step 7.

3.	If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a number of the required AIML clients is provided, the AIMLE server ensures the requirement is met when selecting AIMLE clients for VFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training.

4.	The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3. The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2.

5.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

-	AIML model information for the VFL members and for the AIMLE Server.

6.	The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains.

6a.	The AIMLE server may report to the VAL server with the training status, that includes intermediate training results, The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain.

NOTE:	How AIMLE Server coordinates the selected VFL members to perform VFL training is out of scope for this release.

7.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

* * * End of Changes * * * *